- Title:

Authors:	w08390

Revision:	1.10

Creation Date:	11/11/2015

Modification Date:

Abstract:	This document describes the MIBs supported by ERHMG2

Title:	ERHMG2 MIB Companion	1

i	Revision History	5

ii	References	5

About this document	6

Introduction	6

Consideration	7

Table Format of MIB Information	7

1	Public MIBs List	8

2	Private MIBs list	9

3	RFC1213-MIB	10

3.1	System Group {mib-2.1}	10

3.2	Interfaces Group{mib-2.2}	11

3.3	Address Translation Group(at)  {mib-2.3}	12

3.4	IP Group  {mib-2.4}	12

3.5	ICMP Group{mib-2.5}	15

3.6	TCP Group{mib-2.6}	16

3.7	UDP Group{mib-2.7}	17

3.8	snmp Group{mib-2.11}	18

3.9	ARP MIB	19

4	SNMPv2-MIB	20

4.1	Scalar objects	20

4.2	Trap objects	20

5	EtherLike-MIB(Not supported)	20

5.1	dot3StatsTable	20

5.2	dot3CollTable	22

6	ENTITY-MIB	22

6.1	entPhysicalTable	24

6.2	entLogicalTable(Not supported)	26

6.3	entLPMappingTable（Not supported）	26

6.4	entAliasMappingTable	26

6.5	entPhysicalContainsTable	27

6.6	entityGeneral Group	27

6.7	entityMIBTraps	27

6.8	entity physical index design	27

7	RMON-MIB	28

7.1	Ethernet Statistics	28

7.2	historyControlTable	29

7.3	etherHistoryTable	29

7.4	alarmTable	30

7.5	eventTable	31

7.6	logTable	31

8	HH3C-ENTITY-EXT-MIB	32

8.1	hh3cEntityExtStateTable	32

8.2	Notifications	33

9	BRIDGE-MIB(Not supported)	34

9.1	Scalar objects of dot1dBase group	34

9.2	dot1dBasePortTable	34

9.3	Scalar objects of dot1dTp group	34

9.4	dot1dTpFdbTable	35

9.5	dot1dTpPortTable	35

10	HH3C-FLASH-MAN-MIB	35

10.1	Scalar objects	35

10.2	hh3cFlashTable	36

10.3	hh3cFlhChipTable (Not supported)	37

10.4	hh3cFlhPartitionTable	37

10.5	hh3cFlhFileTable (Not supported)	38

10.6	hh3cFlhOpTable	39

11	HH3C-SYS-MAN-MIB	41

11.1	Scalar objects	41

11.2	hh3cSysCurTable	41

11.3	hh3cSysImageTable	42

11.4	hh3cSysCFGFileTable	42

11.5	Notifications	43

12	HH3C-COMMON-SYSTEM-MIB	43

12.1	Scalar objects	43

13	HH3C-CONFIG-MAN-MIB	43

13.1	Scalar objects of hh3cCfgLog group(Not supported)	43

13.2	hh3cCfgLogTable(Not supported)	44

13.3	Scalar objects of hh3cCfgOperate group	45

13.4	hh3cCfgOperateTable	46

13.5	hh3cCfgOperateResultTable	47

13.6	Scalar objects of hh3cCfgExecuteOperate group(Not supported)	48

13.7	hh3cCfgExecuteResultTable(Not supported)	49

13.8	Notifications(Not supported)	49

14	HOST-RESOURCES-MIB	49

1.1	hrProcessorTable	49

1.2	scalar objects	50

15	HH3C-DHCPS-MIB	50

15.1	Scalar objects	50

15.2	hh3cDHCPSGlobalPoolTable (not supported)	51

15.3	hh3cDHCPSGlobalPoolConfigTable(not supported)	51

15.4	hh3cDHCPSForbiddenIPTable(not supported)	51

16	HH3C-LswINF-MIB	52

16.1	hh3cifXXTable	52

17	HH3C-LswVLAN-MIB	52

17.1	hh3cdot1qVlanMIBTable	52

18	HH3C-NTP-MIB	55

18.1	hh3cNTPPeerTable	55

19	HH3C-NET-MAN-MIB	57

19.1	Scalar objects	57

20	supported Standard Traps	57

20.1	supported Standard Traps	57

20.2	Supported Private Traps	58

20.2.1	Common Traps	58

# i	Revision History

- Every column should be filled

- The format of Revision“” is xx.yy. And xx is major version, yy is minor version. The initial version begins with 0.01.

- All modifications should be recorded here to indicate the reader which changes are happened.

| Revision | Date | Author | Reason for Change |
| --- | --- | --- | --- |
| 0.01 | 2008-03-10 | z03835 | First Draft for Miniware |
| 1.01 | 2008-07-14 | z03835 | add HH3C-NTP-MIB, HOST-RESOURCES-MIB, HH3C-DHCPS-MIB, HH3C-LswINF-MIB, HH3C-LswVLAN-MIB.etc. revise HH3C-ENTITY-EXT-MIB,etc. |
| 1.02 | 2008-09-25 | z03835 | revise RFC1213,some node not supported, SHD06743 |
| 1.03 | 2009-03-09 | Z03835 | Not support ifLastChange |
| 1.10 | 2010-02-07 | Z03835 | Add RMON-MIB |

# ii	References

# About this document

This document describes all of the MIBs and MIB objects in them which are supported by ERHMG2.

This document should be shipped with the corresponding versions released to Miniware’s customs. Here the customs refer to the test staff, product staff, NMS developers, etc, but not for the final commercial customers.

# Introduction

This document contains “Public MIB List” section and “Private MIB list” section.

The “Public MIB List” provides a MIB list containing the Internet standard MIBs

supported by Miniware. The “Private MIB List” provides H3C private MIB list.

The following chapters describe the implementation of the objects of all public and private MIBs.

At the end of document, supported traps are presented.

# Consideration

- Table Format of MIB Information

In this document, scalar and tabular MIB object information is presented with the following table format. The Trap object excluded from such format of table.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifIndex | read-only | Yes | As per MIB |

The meanings of each column of the table are listed below.

| Item | Comments |
| --- | --- |
| ‘Name’ | Name of the MIB object |
| ‘Access’ | The ACCESS or MAX-ACCESS value as defined in MIBs. |
| ‘PDS’ | Permanent Data Storage. This field describes what will happen for MIB objects after device reboots. There are three statuses: 'Yes': After the value of a MIB object changed, the value of MIB object was saved automatically into device, the MIB object can remain the same value after device reboots. ‘No': whatever action is done, the value of MIB object will not remain after device reboots. Please note the ‘PDS’ of Counter/Counter32/Counter64 MIB object should be always ‘No’. ‘Current’: After the value of a MIB object changed, if the value of MIB object is saved explicitly (such as via ‘save’ CLI command), it can remain after device reboots. Otherwise, if not save explicitly, the value of MIB object will be lost. |
| ‘Description’ | General comments. All specific information are provided here. The following contents are just examples. ‘As per MIB’: Just as described in the MIB. ‘Not supported’: The object is not supported, although it can be operated (read and/or write). The user should not use such MIB object. ‘Product specific’: The information is product specific. Although information may provided in this field for some products, we cannot assure to list all product information. To find product specific information, users can access the device via SNMP. `Need confirm by products`: the default value or the supported value range need confirm by products. |

If the meaning of a MIB is not clearly described in the tables, or other information should be expressed, the more detailed information can be provided at the proper positions, with proper format not restricted with table. That is, further descriptions may be given below the tables, at the beginning of the section. These descriptions can be written as a paragraph, a formula, a chart, etc.

For tabular objects，you must return the default value of the conrresponding sytanx type without supporting them; for scalar objects, you are allowed not to care it when not supporting them

# Public MIBs List

In this section, all public MIBs supported by ERHMG2 are listed.

Any action to modify, delete, remove and add must be recorded in Revision History.

In File name column, the MIB file name must conform with the naming rule, that is the MIB module name prefixed with the RFC index like rfc4133-entity.mib. Note that file name is lowercase.

In MIB Version column, it indicates the MIB version used  in devices.

| Module Name | File Name | comments | MIB Version |
| --- | --- | --- | --- |
| ENTITY-MIB | rfc4133-entity.mib |  | rfc4133 |
| IF-MIB | rfc2233-if.mib |  | rfc2233 |
| IP-FORWARD-MIB | rfc2096-ip-forward.mib | Not supported | rfc2096 |
| IP-MIB | rfc2011-ip-icmp.mib | The same as the IP Group of rfc1213 | rfc2011 |
| RFC1213-MIB | rfc1213.mib |  | rfc1213 |
| SNMP-FRAMEWORK-MIB | rfc2571-snmp-framework.mib | Not supported | rfc2571 |
| SNMP-NOTIFICATION-MIB | rfc2273-snmp-notification.mib | Not supported | rfc2273 |
| SNMP-TARGET-MIB | rfc2573-snmp-target.mib | Not supported | rfc2573 |
| SNMP-USER-BASED-SM-MIB | rfc3414-snmp-usm.mib | Not supported | rfc3414 |
| SNMP-USM-AES-MIB | rfc3826-snmp-usm-aes.mib | Not supported | rfc3826 |
| SNMPv2-MIB | rfc3418-snmpv2.mib | From rfc3418 | rfc3418 |
| SNMP-VIEW-BASED-ACM-MIB | rfc3415-snmp-vacm.mib | Not supported | rfc3415 |
| TCP-MIB | rfc2012-tcp.mib | Not supported | rfc2012 |
| UDP-MIB | rfc2013-udp.mib | The same as the UDP Group of rfc1213 | rfc2013 |
| HOST-RESOURCES-MIB | HOST-RESOURCES-MIB-rfc2790 | from rfc2790 | rfc2790 |
| RMON-MIB | Rfc2819-rmon.mib | From rfc2819 | Rfc2819 |

# Private MIBs list

| Module Name | File Name | comments | MIB Version |
| --- | --- | --- | --- |
| HH3C-CONFIG-MAN-MIB | hh3c-config-man.mib |  | V1.9 |
| HH3C-ENTITY-EXT-MIB | hh3c-entity-ext.mib |  | V1.7 |
| HH3C-FLASH-MAN-MIB | hh3c-flash-man.mib |  | V2.6 |
| HH3C-SYS-MAN-MIB | hh3c-sys-man.mib |  | V2.1 |
| HH3C-COMMON-SYSTEM-MIB | hh3c-common-system.mib |  | V2.0 |
| HH3C-DHCPS-MIB | hh3c-dhcps.mib |  | V1.51 |
| HH3C-LswVlan-MIB | hh3c-splat-vlan.mib |  | V1.9 |
| HH3C-LswINF-MIB | hh3c-splat-inf.mib |  | V3.1 |
| HH3C-NTP-MIB | hh3c-ntp.mib |  | V1.2 |
| HH3C-NET-MAN-MIB | hh3c-net-man.mib |  | V1.0 |

# RFC1213-MIB

All of the objects of this MIB are listed here, and each one is given a definite explaination.

  - System Group {mib-2.1}

This group is fully supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| SysDescr | read-only | No | for example H3C Product VersIon ERHMG2V100A001<0D><0A>H3C ERHMG2<0D><0A>Copyright (c) 2014-2014 Hangzhou H3C Technologies Co., Ltd. All rights Reserved<0D><0A>.” |
| SysObjectID | read-only | Yes | hh3c- ERHMG2 |
| SysUpTime | read-only | No | As per MIB |
| SysContact | Read-write | Yes | Default value is “R&D Hangzhou, Hangzhou H3C Technologies Co., Ltd.” |
| SysName | Read-write | Yes | Default value is “H3C” |
| SysLocation | Read-write | Yes | Default value is “Hangzhou, China” |
| SysServices | read-only | Yes | 78 (2^ (2-1) + 2^ (3-1) + 2^ (4-1) + 2^ (7-1)). indicates that can provide the following services: 2 datalink/subnetwork (e.g., bridges) 3 internet (e.g., supports the IP) 4 end-to-end (e.g., supports the TCP) 7 applications (e.g., supports the SMTP) |

  - Interfaces Group{mib-2.2}

The Interfaces group implemented is as modified by RFC2233.

    - ifNumber

The ifNumber value will display the actual number of entries in the ifTable.

    - ifTable mib

| MIB Objects | Access | PDS | Description |
| --- | --- | --- | --- |
| ifIndex | read-only | Yes | As per MIB |
| ifDescr | read-only | Yes | As per MIB |
| ifType | read-only | Yes | As per MIB |
| ifMtu | read-only | Yes | As per MIB |
| ifSpeed | read-only | Yes | As per MIB |
| ifPhysAddress | read-only | Yes | MAC address |
| ifAdminStatus | read- write | Yes | enable or disable an interface, ”testing” is not supported. |
| ifOperStatus | read-only | Yes | The current operational state(link status) of the interface. |
| ifLastChange | read-only | No | Not supported |
| ifInOctets | read-only | No | As per MIB |
| ifInUcastPkts | read-only | No | As per MIB |
| ifInNUcastPkts | read-only | No | As per MIB |
| ifInDiscards | read-only | No | Not supported |
| ifInErrors | read-only | No | Not supported |
| ifInUnknownProtos | read-only | No | Not supported |
| ifOutOctets | read-only | No | As per MIB |
| ifOutUcastPkts | read-only | No | As per MIB |
| ifOutNUcastPkts | read-only | No | Not supported |
| ifOutDiscards | read-only | No | Not supported |
| ifOutErrors | read-only | No | Not supported |
| ifOutQLen | read-only | No | Not supported |
| ifSpecific | read-only | No | Not supported |

  - Address Translation Group(at)  {mib-2.3}

    - atTable mib(Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| atIfIndex | read-write | Yes | deprecated |
| atPhysAddress | read-write | Yes | deprecated |
| atNetAddress | read-write | Yes | deprecated |

  - IP Group 	{mib-2.4}

The following objects will be supported:

    - Scalar Objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipForwarding | read-write | Yes | Not writable |
| ipDefaultTTL | read-write | Yes | Not writable |
| ipInReceives | read-only | No | As per MIB |
| ipInHdrErrors | read-only | No | Not supported |
| ipInAddrErrors | read-only | No | Not supported |
| ipForwDatagrams | read-only | No | Not supported |
| ipInUnknownProtos | read-only | No | As per MIB |
| ipInDiscards | read-only | No | Not supported |
| ipInDelivers | read-only | No | As per MIB |
| ipOutRequests | read-only | No | As per MIB |
| ipOutDiscards | read-only | No | Not supported |
| ipOutNoRoutes | read-only | No | As per MIB |
| ipReasmTimeout | read-only | No | As per MIB |
| ipReasmReqds | read-only | No | Not supported |
| ipReasmOKs | read-only | No | As per MIB |
| ipReasmFails | read-only | No | As per MIB |
| ipFragOKs | read-only | No | As per MIB |
| ipFragFails | read-only | No | Not supported |
| ipFragCreates | read-only | No | Not supported |
| ipRoutingDiscards | read-only | No | Not supported |

    - ipAddrTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAdEntAddr | read-only | Yes | As per MIB |
| ipAdEntIfIndex | read-only | Yes | As per MIB |
| ipAdEntNetMask | read-only | Yes | As per MIB |
| ipAdEntBcastAddr | read-only | Yes | As per MIB |
| ipAdEntReasmMaxSize | read-only | Yes | As per MIB |

    - ipRouteTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipRouteDest | read-write | No | Not writable |
| ipRouteIfIndex | read-write | No | Not writable |
| ipRouteMetric1 | read-write | No | Not writable |
| ipRouteMetric2 | read-write | No | Not supported |
| ipRouteMetric3 | read-write | No | Not writable |
| ipRouteMetric4 | read-write | No | Not writable |
| ipRouteNextHop | read-write | No | Not writable |
| ipRouteType | read-write | No | Not writable |
| ipRouteProto | read-only | No | As per MIB |
| ipRouteAge | read-write | No | Not writable |
| ipRouteMask | read-write | No | Not writable |
| ipRouteMetric5 | read-write | No | Not writable |
| ipRouteInfo | read-only | No | Not supported |

  - ICMP Group{mib-2.5}

    - Scalar objects

| MIB Objects | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpInMsgs | read-only | No | As per MIB |
| icmpInErrors | read-only | No | As per MIB |
| icmpInDestUnreachs | read-only | No | As per MIB |
| icmpInTimeExcds | read-only | No | As per MIB |
| icmpInParmProbs | read-only | No | As per MIB |
| icmpInSrcQuenchs | read-only | No | As per MIB |
| icmpInRedirects | read-only | No | Not supported |
| icmpInEchos | read-only | No | As per MIB |
| icmpInEchoReps | read-only | No | As per MIB |
| icmpInTimestamps | read-only | No | As per MIB |
| icmpInTimestampReps | read-only | No | As per MIB |
| icmpInAddrMasks | read-only | No | As per MIB |
| icmpInAddrMaskReps | read-only | No | As per MIB |
| icmpOutMsgs | read-only | No | As per MIB |
| icmpOutErrors | read-only | No | Not supported |
| icmpOutDestUnreachs | read-only | No | Not supported |
| icmpOutTimeExcds | read-only | No | Not supported |
| icmpOutParmProbs | read-only | No | Not supported |
| icmpOutSrcQuenchs | read-only | No | Not supported |
| icmpOutRedirects | read-only | No | Not supported |
| icmpOutEchos | read-only | No | Not supported |
| icmpOutEchoReps | read-only | No | Not supported |
| icmpOutTimestamps | read-only | No | Not supported |
| icmpOutTimestampReps | read-only | No | Not supported |
| icmpOutAddrMasks | read-only | No | Not supported |
| icmpOutAddrMaskReps | read-only | No | Not supported |

  - TCP Group{mib-2.6}

    - Scalar objects

| MIB Objects | Access | PDS | Description |
| --- | --- | --- | --- |
| TcpRtoAlgorithm | read-only | No | As per MIB |
| tcpRtoMin | read-only | No | As per MIB |
| tcpRtoMax | read-only | No | As per MIB |
| tcpMaxConn | read-only | No | As per MIB |
| tcpActiveOpens | read-only | No | As per MIB |
| tcpPassiveOpens | read-only | No | As per MIB |
| tcpAttemptFails | read-only | No | As per MIB |
| tcpEstabResets | read-only | No | As per MIB |
| tcpCurrEstab | read-only | No | As per MIB |
| tcpInSegs | read-only | No | As per MIB |
| tcpOutSegs | read-only | No | As per MIB |
| tcpRetransSegs | read-only | No | As per MIB |
| tcpInErrs | read-only | No | As per MIB |
| tcpOutRsts | read-only | No | As per MIB |

    - tcpConnTable

| MIB Objects | Access | PDS | Description |
| --- | --- | --- | --- |
| tcpConnState | read-write | No | Not writable |
| tcpConnLocalAddress | read-only | No | As per MIB |
| tcpConnLocalPort | read-only | No | As per MIB |
| tcpConnRemAddress | read-only | No | As per MIB |
| tcpConnRemPort | read-only | No | As per MIB |

  - UDP Group{mib-2.7}

    - Scalar objects

A statistics collection for UDP group.

| MIB Objects | Access | PDS | Description |
| --- | --- | --- | --- |
| udpInDatagrams | read- only | No | As per MIB |
| udpNoPorts | read-only | No | As per MIB |
| udpInErrors | read-only | No | As per MIB |
| udpOutDatagrams | read-only | No | As per MIB |

    - udpTable

| MIB Objects | Access | PDS | Comments |
| --- | --- | --- | --- |
| udpLocalAddress | read- only | Yes | As per MIB |
| udpLocalPort | read-only | Yes | As per MIB |

  - snmp Group{mib-2.11}

| MIB Objects | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpInPkts | read-only | NO | As per MIB |
| snmpOutPkts | read-only | NO | As per MIB |
| snmpInBadVersions | read-only | NO | As per MIB |
| snmpInBadCommunityNames | read-only | NO | As per MIB |
| snmpInBadCommunityUses | read-only | NO | Not supported |
| snmpInASNParseErrs | read-only | NO | As per MIB |
| snmpInTooBigs | read-only | NO | Not supported |
| snmpInNoSuchNames | read-only | NO | Not supported |
| snmpInBadValues | read-only | NO | Not supported |
| snmpInReadOnlys | read-only | NO | Not supported |
| snmpInGenErrs | read-only | NO | Not supported |
| snmpInTotalReqVars | read-only | NO | Not supported |
| snmpInTotalSetVars | read-only | NO | Not supported |
| snmpInGetRequests | read-only | NO | As per MIB |
| snmpInGetNexts | read-only | NO | As per MIB |
| snmpInSetRequests | read-only | NO | As per MIB |
| snmpInGetResponses | read-only | NO | As per MIB |
| snmpInTraps | read-only | NO | As per MIB |
| snmpOutTooBigs | read-only | NO | As per MIB |
| snmpOutNoSuchNames | read-only | NO | As per MIB |
| snmpOutBadValues | read-only | NO | As per MIB |
| snmpOutGenErrs | read-only | NO | As per MIB |
| snmpOutGetRequests | read-only | NO | As per MIB |
| snmpOutGetNexts | read-only | NO | As per MIB |
| snmpOutSetRequests | read-only | NO | As per MIB |
| snmpOutGetResponses | read-only | N/A | As per MIB |
| snmpOutTraps | read-only | NO | As per MIB |
| snmpEnableAuthenTraps | read-write | YES | Not supported |

  - ARP MIB

    - ipNetToMediaTable

This talbe only support read-only access

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipNetToMediaIfIndex | read-write | no | Not writable |
| ipNetToMediaPhysAddress | read-write | no | Not writable |
| ipNetToMediaNetAddress | read-write | no | Not writable |
| ipNetToMediaType | read-write | no | Not writable |

# SNMPv2-MIB

  - Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| SnmpTrapOID | accessible-for-notify | No | Not supported |
| snmpTrapEnterprise | accessible-for-notify | No | Not supported |
| snmpSetSerialNo | Read-write | No | As per MIB |

  - Trap objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| coldStart |  | NA | As per MIB |
| warmStart |  | NA | As per MIB |
| linkDown |  | NA | As per MIB |
| linkUp |  | NA | As per MIB |
| authenticationFailure |  | NA | As per MIB |

# EtherLike-MIB(Not supported)

  - dot3StatsTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot3StatsIndex | read-only | Yes | As per MIB |
| dot3StatsAlignmentErrors | read-only | Yes | As per MIB |
| dot3StatsFCSErrors | read-only | Yes | As per MIB |
| dot3StatsSingleCollisionFrames | read-only | Yes | As per MIB |
| dot3StatsMultipleCollisionFrames | read-only | Yes | As per MIB |
| dot3StatsSQETestErrors | read-only | Yes | As per MIB |
| dot3StatsDeferredTransmissions | read-only | Yes | As per MIB |
| dot3StatsLateCollisions | read-only | Yes | As per MIB |
| dot3StatsExcessiveCollisions | read-only | Yes | As per MIB |
| dot3StatsInternalMacTransmitErrors | read-only | Yes | As per MIB |
| dot3StatsCarrierSenseErrors | read-only | Yes | As per MIB |
| dot3StatsFrameTooLongs | read-only | Yes | As per MIB |
| dot3StatsInternalMacReceiveErrors | read-only | Yes | As per MIB |
| dot3StatsEtherChipSet | read-only | Yes | Not supported |
| dot3StatsSymbolErrors | read-only | Yes | As per MIB |
| dot3StatsDuplexStatus | read-only | Yes | As per MIB |

  - dot3CollTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot3CollCount | not-accessible | No | Not supported |
| dot3CollFrequencies | read-only | Yes | Not supported |

# ENTITY-MIB

This MIB is supported only when the products support RFC4133.

This chapter describes the information of Entity MIB. The Entity MIB contains five groups of MIB objects: entityPhysical group, entityLogical group, entityMapping group, entityGeneral group and entityNotifications group. Thereinto, the entityPhysical group describes the physical entities managed by a single agent. This group contains a single table to identify physical system components, called the entPhysicalTable. The entPhysicalTable contains one row per physical entity, and must always contain at least one row for an "overall" physical entity, which should have an entPhysicalClass value of 'stack(11)', 'chassis(3)' or 'module(9)'. Each row is indexed by an arbitrary, small integer, and contains a description and type of the physical entity.  It also optionally contains the index number of another entPhysicalEntry indicating a containment relationship between the two.

The entityLogical group describes the logical entities managed by a single agent. This group contains a single table to identify logical entities, called the entLogicalTable. The entLogicalTable contains one row per logical entity.  Each row is indexed by an arbitrary, small integer and contains a name, description, and type of the logical entity. It also contains information to allow access to the MIB information for the logical entity. This includes SNMP versions that use a community name (with some form of implied context representation) and SNMP versions that use the SNMP ARCH [RFC2571] method of context identification. If a agent represents multiple logical entities with this MIB, then this group must be implemented for all logical entities known to the agent. If an agent represents a single logical entity, or multiple logical entities within a single naming scope, then implementation of this group may be omitted by the agent.

The entityMapping group describes the associations between the physical entities logical entities, interfaces, and non-interface ports managed by a single agent. This group contains three tables to identify associations between different system components. The entLPMappingTable contains mappings between entLogicalIndex values (logical entities) and entPhysicalIndex values (the physical components supporting that entity). A logical entity can map to more than one physical component, and more than one logical entity can map to (share) the same physical component.  If an agent represents a single logical entity, or multiple logical entities within a single naming scope, then implementation of this table may be omitted by the agent. The entAliasMappingTable contains mappings between entLogicalIndex, entPhysicalIndex pairs and 'alias' object identifier values.  This allows resources managed with other MIBs (e.g., repeater ports, bridge ports, physical and logical interfaces) to be identified in the physical entity hierarchy. Note that each alias identifier isonly relevant in a particular naming scope.  If an agent represents a single logical entity, or multiple logical entities within a single naming scope, then implementation of this table may be omitted by the agent. The entPhysicalContainsTable contains simple mappings between 'entPhysicalContainedIn' values for each container/'containee' relationship in the managed system. The indexing of this table allows an NMS to quickly discover the 'entPhysicalIndex' values for all children of a given physical entity.

The entityGeneral group describes general system attributes shared by potentially all types of entities managed by a single agent. This group contains general information relating to the other object groups. At this time, the entGeneral group contains a single scalar object (entLastChangeTime), which represents the value of sysUptime when any part of the Entity MIB configuration last changed.

The entityNotifications group contains status indication notifications. This group contains notification definitions relating to the overall status of the Entity MIB instantiation.

The informatioin in this mib should be provided and confirmed by products because there are so much information to be decided by products.

  - entPhysicalTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entPhysicalIndex | not-accessible | No | As per MIB |
| entPhysicalDescr | read-only | No | As per MIB |
| entPhysicalVendorType | read-only | No | As per MIB |
| entPhysicalContainedIn | read-only | No | As per MIB |
| entPhysicalClass | read-only | No | As per MIB |
| entPhysicalParentRelPos | read-only | No | As per MIB |
| entPhysicalName | read-only | No | As per MIB |
| entPhysicalHardwareRev | read-only | No | As per MIB |
| entPhysicalFirmwareRev | read-only | No | As per MIB |
| entPhysicalSoftwareRev | read-only | No | As per MIB |
| entPhysicalSerialNum | read-write | No | As per MIB |
| entPhysicalMfgName | read-only | No | As per MIB |
| entPhysicalModelName | read-only | No | As per MIB |
| entPhysicalAlias | read-write | No | As per MIB, support set operation, but can’t save your changes. |
| entPhysicalAssetID | read-write | No | As per MIB |
| entPhysicalIsFRU | read-only | No | As per MIB |
| entPhysicalMfgDate | read-only | No | Not supported |
| entPhysicalUris | read-write | No | As per MIB |

  - entLogicalTable(Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entLogicalIndex | not-accessible | No | As per MIB |
| entLogicalDescr | read-only | No | As per MIB |
| entLogicalType | read-only | No | As per MIB |
| entLogicalCommunity | read-only | No | Deprecated in mib |
| entLogicalTAddress | read-only | No | As per MIB |
| entLogicalTDomain | read-only | No | As per MIB |
| entLogicalContextEngineID | read-only | No | As per MIB |
| entLogicalContextName | read-only | No | As per MIB |

  - entLPMappingTable（Not supported）

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entLPPhysicalIndex | read-only | No | As per MIB |

  - entAliasMappingTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entAliasLogicalIndexOrZero | not-accessible | No | As per MIB |
| entAliasMappingIdentifier | read-only | No | As per MIB |

  - entPhysicalContainsTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entPhysicalChildIndex | read-only | No | As per MIB |

  - entityGeneral Group

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entLastChangeTime | read-only | No | Not supported |

  - entityMIBTraps

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entConfigChange |  |  | Not supported |

  - entity physical index design

1. For the chassis: its index is 1;

2. For the container level 1, its index is 2;

3. For the module level 1, its index is 3;

4. For the No.x of L2 container, its index is 3+ x ;

5. For the first L2 module, its index is 3+MAX_L2_CONTAINER+1;

6. For the first power, its index is 3+ MAX_L2_CONTAINER+MAX_L2_MODULE+1;

7. For the first Fan, its index is 3 + MAX_L2_CONTAINER+ MAX_L2_MODULE + MAX_POWER + 1

8. For the CPU, its index is

3 + MAX_L2_CONTAINER+ MAX_L2_MODULE + MAX_POWER + MAX_FAN+MAX_OTHER+1

8. For the No.y port of Moudle No.x, its index is

3 + MAX_L2_CONTAINER+ MAX_L2_MODULE + MAX_POWER + MAX_FAN+MAX_OTHER+MAX_CPU+ (x-1)*MAX_PORT_PER_MOUDLE +y;

For ER3200, Number of different entities are defined as below:

MAX_CHASSIS = 1

MAX_L2_CONTAINER = 8

MAX_L2_MODULE = 4

MAX_POWER = 1

MAX_FAN = 1

MAX_OTHER = 1

MAX_CPU = 1

MAX_PORT_PER_MOUDLE = 8

# RMON-MIB

  - Ethernet Statistics

The maximum entries for the table are 50;

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| etherStatsIndex | read-only | No | As per MIB |
| etherStatsDataSource | read-create | No | As per MIB |
| etherStatsDropEvents | read-only | No | Not supported |
| etherStatsOctets | read-only | No | As per MIB |
| etherStatsPkts | read-only | No | As per MIB |
| etherStatsBroadcastPkts | read-only | No | As per MIB |
| etherStatsMulticastPkts | read-only | No | As per MIB |
| etherStatsCRCAlignErrors | read-only | No | Not supported |
| etherStatsUndersizePkts | read-only | No | Not supported |
| etherStatsOversizePkts | read-only | No | Not supported |
| etherStatsFragments | read-only | No | Not supported |
| etherStatsJabbers | read-only | No | Not supported |
| etherStatsCollisions | read-only | No | Not supported |
| etherStatsPkts64Octets | read-only | No | Not supported |
| etherStatsPkts65to127Octets | read-only | No | Not supported |
| etherStatsPkts128to255Octets | read-only | No | Not supported |
| etherStatsPkts256to511Octets | read-only | No | Not supported |
| etherStatsPkts512to1023Octets | read-only | No | Not supported |
| etherStatsPkts1024to1518Octets | read-only | No | Not supported |
| etherStatsOwner | read-create | No | As per MIB |
| etherStatsStatus | read-create | No | As per MIB |

  - historyControlTable

The maximum entries for the table are 50;

The max buckets in historyControlTable are 10;

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| historyControlIndex | read-only | No | As per MIB |
| historyControlDataSource | read-create | No | As per MIB |
| historyControlBucketsRequested | read-create | No | historyControlBucketsRequested only supports 10 buckets at maximum currently, If a value more than 10 is set, the system will return success, but the historyControlBucketGranted will be assigned to 10 as for implementation. |
| historyControlBucketsGranted | read-only | No | As per MIB |
| historyControlInterval | read-create | No | Range from 5 to 3600 |
| historyControlOwner | read-create | No | As per MIB |
| historyControlStatus | read-create | No | As per MIB |

  - etherHistoryTable

As the definition above, the maximum entries of the table are 500(50*10);

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| etherHistoryIndex | read-only | No | As per MIB |
| etherHistorySampleIndex | read-only | No | As per MIB |
| etherHistoryIntervalStart | read-only | No | As per MIB |
| etherHistoryDropEvents | read-only | No | Not supported |
| etherHistoryOctets | read-only | No | As per MIB |
| etherHistoryPkts | read-only | No | As per MIB |
| etherHistoryBroadcastPkts | read-only | No | As per MIB |
| etherHistoryMulticastPkts | read-only | No | As per MIB |
| etherHistoryCRCAlignErrors | read-only | No | Not supported |
| etherHistoryUndersizePkts | read-only | No | Not supported |
| etherHistoryOversizePkts | read-only | No | Not supported |
| etherHistoryFragments | read-only | No | Not supported |
| etherHistoryJabbers | read-only | No | Not supported |
| etherHistoryCollisions | read-only | No | Not supported |
| etherHistoryUtilization | read-only | No | As per MIB |

  - alarmTable

The maximum entries of the table are 60;

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| alarmIndex | read-only | No | As per MIB |
| alarmInterval | read-create | No | The range is from 5 to 65535 |
| alarmVariable | read-create | No | As per MIB |
| alarmSampleType | read-create | No | As per MIB |
| alarmValue | read-only | No | When the value of this object is out of the range of alarmValue, the value of alarmValue will be a negative |
| alarmStartupAlarm | read-create | No | As per MIB |
| alarmRisingThreshold | read-create | No | As per MIB |
| alarmFallingThreshold | read-create | No | As per MIB |
| alarmRisingEventIndex | read-create | No | As per MIB |
| alarmFallingEventIndex | read-create | No | As per MIB |
| alarmOwner | read-create | No | As per MIB |
| alarmStatus | read-create | No | As per MIB |

  - eventTable

The maximum entries for the table are 60;

For every event, the maximum logs are 10;

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| eventIndex | read-only | No | As per MIB |
| eventDescription | read-create | No | The node can not be set string with space. |
| eventType | read-create | No | As per MIB |
| eventCommunity | read-create | No | Not supported. When eventType is none(1) or log(2), the value of eventCommunity will not be saved; when eventType is snmptrap(3) or logandtrap(4), the value of eventCommunity will be saved but can take no effect. |
| eventLastTimeSent | read-only | No | As per MIB |
| eventOwner | read-create | No | The length of value of this object is range from 1 to 127 |
| eventStatus | read-create | No | eventStatus can be set to valid when the row is created |

  - logTable

The maximum entries for the table are 600(60*10)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| logEventIndex | read-only | No | As per MIB |
| logIndex | read-only | No | As per MIB |
| logTime | read-only | No | As per MIB |
| logDescription | read-only | No | As per MIB |

# HH3C-ENTITY-EXT-MIB

  - hh3cEntityExtStateTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cEntityExtPhysicalIndex | accessible-for-notify | No | As per MIB |
| hh3cEntityExtAdminStatus | read-write | No | As per MIB |
| hh3cEntityExtOperStatus | read-only | No | As per MIB |
| hh3cEntityExtStandbyStatus | read-only | No | As per MIB |
| hh3cEntityExtAlarmLight | read-only | No | Not supported. |
| hh3cEntityExtCpuUsage | read-only | No | As per MIB |
| hh3cEntityExtCpuUsageThreshold | read-write | No | As per MIB |
| hh3cEntityExtMemUsage | read-only | No | As per MIB |
| hh3cEntityExtMemUsageThreshold | read-write | No | As per MIB |
| hh3cEntityExtMemSize | read-write | No | As per MIB |
| hh3cEntityExtUpTime | read-only | No | As per MIB |
| hh3cEntityExtTemperature | read-only | No | Not supported |
| hh3cEntityExtTemperatureThreshold | read-write | No | Not supported |
| hh3cEntityExtVoltage | read-only | No | Not supported |
| hh3cEntityExtVoltageLowThreshold | read-write | No | Not supported |
| hh3cEntityExtVoltageHighThreshold | read-write | No | Not supported |
| hh3cEntityExtCriticalTemperatureThreshold | read-write | No | Not supported |
| hh3cEntityExtMacAddress | read-only | No | As per MIB |
| hh3cEntityExtErrorStatus | read-only | No | As per MIB |

  - Notifications

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cEntityExtTemperatureThresholdNotification | NA | NA | If the device supports temperature monitor and entity extend MIB, this object will be supported. |
| hh3cEntityExtVoltageLowThresholdNotification | NA | NA | Not supported |
| hh3cEntityExtVoltageHighThresholdNotification | NA | NA | Not supported |
| hh3cEntityExtCpuUsageThresholdNotfication | NA | NA | As per MIB |
| hh3cEntityExtMemUsageThresholdNotification | NA | NA | As per MIB |
| hh3cEntityExtOperEnabled | NA | NA | Not supported |
| hh3cEntityExtOperDisabled | NA | NA | Not supported |
| hh3cEntityExtCriticalTemperatureThresholdNotification | NA | NA | If the device supports temperature monitor and entity extend MIB, this object will be supported. |
| hh3cEntityExtSFPAlarmOn | NA | NA | Not supported |
| hh3cEntityExtSFPAlarmOff | NA | NA | Not supported |

# BRIDGE-MIB(Not supported)

  - Scalar objects of dot1dBase group

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1dBaseBridgeAddress | read-only | No | As per MIB |
| dot1dBaseNumPorts | read-only | No | As per MIB |
| dot1dBaseType | read-only | No | As per MIB |

  - dot1dBasePortTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1dBasePort | read-only | No | As per MIB |
| dot1dBasePortIfIndex | read-only | No | As per MIB |
| dot1dBasePortCircuit | read-only | No | As per MIB |
| dot1dBasePortDelayExceededDiscards | read-only | No | As per MIB |
| dot1dBasePortMtuExceededDiscards | read-only | No | As per MIB |

  - Scalar objects of dot1dTp group

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1dTpAgingTime | read-write | No | As per MIB, default 300s |

  - dot1dTpFdbTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1dTpFdbAddress | read-only | No | As per MIB |
| dot1dTpFdbPort | read-only | No | As per MIB |
| dot1dTpFdbStatus | read-only | No | As per MIB |

  - dot1dTpPortTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1dTpPort | read-only | No | As per MIB |
| dot1dTpPortMaxInfo | read-only | No | As per MIB |
| dot1dTpPortInFrames | read-only | No | As per MIB |
| dot1dTpPortOutFrames | read-only | No | As per MIB |
| dot1dTpPortInDiscards | read-only | No | As per MIB. |

# HH3C-FLASH-MAN-MIB

This Mib supplies the information for the management of flash devices.

  - Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cFlhSupportNum | read-only | No | The value is always 1. |

  - hh3cFlashTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cFlhIndex | read-only | No | As per MIB |
| hh3cFlhSize | read-only | No | As per MIB |
| hh3cFlhPos | read-only | No | Not supported |
| hh3cFlhName | read-only | No | Not supported |
| hh3cFlhChipNum | read-only | No | Not supported |
| hh3cFlhDescr | read-only | No | Not supported |
| hh3cFlhInitTime | read-only | No | Not supported |
| hh3cFlhRemovable | read-only | No | Not supported |
| hh3cFlhPartitionBool | read-only | No | Not supported |
| hh3cFlhMinPartitionSize | read-only | No | Not supported |
| hh3cFlhMaxPartitions | read-only | No | Not supported |
| hh3cFlhPartitionNum | read-only | No | Not supported |

  - hh3cFlhChipTable (Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cFlhChipSerialNo | not-accessible | No | As per MIB |
| hh3cFlhChipID | read-only | No | As per MIB |
| hh3cFlhChipDescr | read-only | No | As per MIB |
| hh3cFlhChipWriteTimesLimit | read-only | No | As per MIB |
| hh3cFlhChipWriteTimes | read-only | No | As per MIB |
| hh3cFlhChipEraseTimesLimit | read-only | No | As per MIB |
| hh3cFlhChipEraseTimes | read-only | No | Not supported |

  - hh3cFlhPartitionTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cFlhPartIndex | not-accessible | No | As per MIB |
| hh3cFlhPartFirstChip | read-only | No | Not supported |
| hh3cFlhPartLastChip | read-only | No | Not supported |
| hh3cFlhPartSpace | read-only | No | As per MIB |
| hh3cFlhPartSpaceFree | read-only | No | As per MIB |
| hh3cFlhPartFileNum | read-only | No | Not supported |
| hh3cFlhPartChecksumMethod | read-only | No | Not supported |
| hh3cFlhPartStatus | read-only | No | As per MIB |
| hh3cFlhPartUpgradeMode | read-only | No | Not supported |
| hh3cFlhPartName | read-only | No | As per MIB |
| hh3cFlhPartRequireErase | read-only | No | Not supported |
| hh3cFlhPartFileNameLen | read-only | No | Not supported |

  - hh3cFlhFileTable (Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cFlhFileIndex | not-accessible | No | As per MIB |
| hh3cFlhFileName | read-only | No | As per MIB |
| hh3cFlhFileSize | read-only | No | As per MIB |
| hh3cFlhFileStatus | read-only | No | As per MIB |
| hh3cFlhFileChecksum | read-only | No | As per MIB |

  - hh3cFlhOpTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cFlhOperIndex | not-accessible | No | As per MIB |
| hh3cFlhOperType | read-create | No | As per MIB |
| hh3cFlhOperProtocol | read-create | No | only support tftp(2) |
| hh3cFlhOperServerAddress | read-create | No | As per MIB |
| hh3cFlhOperServerUser | read-create | No | As per MIB |
| hh3cFlhOperPassword | read-create | No | As per MIB |
| hh3cFlhOperSourceFile | read-create | No | As per MIB |
| hh3cFlhOperDestinationFile | read-create | No | As per MIB |
| hh3cFlhOperStatus | read-only | No | As per MIB |
| hh3cFlhOperEndNotification | read-create | No | As per MIB |
| hh3cFlhOperProgress | read-only | No | As per MIB |
| hh3cFlhOperRowStatus | read-create | No | As per MIB |

When ftp protocol is used to transmit file, the values for h3cFlhOperServerUser and h3cFlhOperPassword must be specified.

The following values of H3cFlashOperationStatus are implemented for tftp protocol:

1: opInProgress(1)

2: opSuccess(2)

10: opDeviceError(10)

12: opDeviceFull(12)

13: opFileOpenError(13)

14: opFileTransferError(14)

16: opNoMemory(16)

19: opUnknownFailure(19)

The following values of H3cFlashOperationStatus are implemented for ftp protocol:

1: opInProgress(1)

2: opSuccess(2)

3: opInvalidOperation(3)

7: opInvalidServerAddress(7)

10: opDeviceError(10)

12: opDeviceFull(12)

13: opFileOpenError(13)

14: opFileTransferError(14)

16: opNoMemory(16)

17: opAuthFail(17)

19: opUnknownFailure(19)

The following values of H3cFlashOperationStatus are implemented for deleting operation:

1: opInProgress(1)

2: opSuccess(2)

20:opDeleteFileOpenError(20),

21:opDeleteInvalidDevice(21),

22:opDeleteInvalidFunction(22),

23:opDeleteOperationError(23),

24:opDeleteInvalidFileName(24),

25:opDeleteDeviceBusy(25),

26:opDeleteParaError(26),

27:opDeleteInvalidPath(27)

# HH3C-SYS-MAN-MIB

  - Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSysReloadAction | read-write | No | As per MIB |

  - hh3cSysCurTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSysCurEntPhysicalIndex | not-accessible | No | As per MIB |
| hh3cSysCurCFGFileIndex | read-only | No | As per MIB |
| hh3cSysCurImageIndex | read-only | No | As per MIB |

  - hh3cSysImageTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSysImageIndex | not-accessible | No | As per MIB |
| hh3cSysImageName | read-only | No | As per MIB |
| hh3cSysImageSize | read-only | No | As per MIB |
| hh3cSysImageLocation | read-only | No | As per MIB |
| hh3cSysImageType | read-write | No | Not supported |

  - hh3cSysCFGFileTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSysCFGFileIndex | not-accessible | No | As per MIB |
| hh3cSysCFGFileName | read-only | No | As per MIB |
| hh3cSysCFGFileSize | read-only | No | As per MIB |
| hh3cSysCFGFileLocation | read-only | No | As per MIB |

  - Notifications

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSysClockChangedNotification | NA | NA | As per MIB |
| hh3cSysReloadNotification | NA | NA | As per MIB |

# HH3C-COMMON-SYSTEM-MIB

  - Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cWriteConfig | read-write | Yes | As per MIB |
| hh3cStartFtpServer | read-write | Yes | Not supported |
| hh3cReboot | read-write | Yes | As per MIB |

# HH3C-CONFIG-MAN-MIB

A configuration management MIB, includes configuration log management and Backup & Restore operation of configuration.

All the configuration change events can be managed by the log management MIB. The configuration may be changed by WEB, SNMP or other ways defined as the MIB.

Backup & Restore MIB group provides the configuration backup and restore

  - Scalar objects of hh3cCfgLog group(Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgRunModifiedLast | read-only | No | System checks the current configuration every 10 minutes. If system detects the current configuration has been changed, the value will be updated. Entering system view in command line or executing SNMP SET operation, the value will also be updated. |
| hh3cCfgRunSavedLast | read-only | No | As per MIB |
| hh3cCfgStartModifiedLast | read-only | No | As per MIB |
| hh3cCfgLogLimitedEntries | read-only | No | As per MIB |
| hh3cCfgLogDeletedEntries | read-only | No | As per MIB |
| hh3cCfgLogWantBackup | read-write | No | Available for distributed systems. Always false value for a centralized system. |

  - hh3cCfgLogTable(Not supported)

All the events of configuration change triggered by SNMP agent will be recorded in the table.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgLogIndex | not-accessible | No | As per MIB |
| hh3cCfgLogTime | read-only | No | As per MIB |
| hh3cCfgLogSrcCmd | read-only | No | As per MIB |
| hh3cCfgLogSrcData | read-only | No | As per MIB |
| hh3cCfgLogDesData | read-only | No | As per MIB |
| hh3cCfgLogTerminalType | read-only | No | As per MIB |
| hh3cCfgLogTerminalUser | read-only | No | In version 1 and version2, When users modify configuration through SNMP agent, the value of this instance is the name of the write community, while in version 3 the value is the user's name. A snmp operation produce two piece of log, the value of this instance in the second log is a zero length string. |
| hh3cCfgLogTerminalNum | read-only | No | As per MIB |
| hh3cCfgLogTerminalLocation | read-only | No | As per MIB |
| hh3cCfgLogCmdSrcAddress | read-only | No | A snmp operation produce two piece of log, the value of this instance in the second log is 0.0.0.0 when h3cCfgLogSrcData and h3cCfgLogDesData haven’t the value of “netFtp”. |
| hh3cCfgLogVirHost | read-only | No | A zero length string. |
| hh3cCfgLogUserName | read-only | No | When the value of h3cCfgLogSrcCmd is “cmdLine”, the value of this instance is a zero length string. |
| hh3cCfgLogServerAddress | read-only | No | A snmp operation produce two piece of log, the value of this instance in the first log is always 0.0.0.0, and this instance in the second log is 0.0.0.0 when h3cCfgLogSrcData and h3cCfgLogDesData haven’t the value of “netFtp”. |
| hh3cCfgLogFile | read-only | No | As per MIB |

  - Scalar objects of hh3cCfgOperate group

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgOperateGlobalEntryLimit | read-only | No | As per MIB |
| hh3cCfgOperateEntryAgeOutTime | read-write | Current | Not supported |
| hh3cCfgOperateResultGlobalEntryLimit | read-write | Current | Not supported |

  - hh3cCfgOperateTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgOperateIndex | not-accessible | No | This index can be randomly selected from 1 to 2147483647 |
| hh3cCfgOperateType | read-create | No | only support value net2startup(5) startup2net(6) |
| hh3cCfgOperateProtocol | read-create | No | only supported value tftp(2) |
| hh3cCfgOperateFileName | read-create | No | The maximum length of the full file name (including path and file name) is 116. In addition, the length of file name is no more than 52, and the length of string which describes path is no more than 64. |
| hh3cCfgOperateServerAddress | read-create | No | As per MIB |
| hh3cCfgOperateUserName | read-create | No | Not supported |
| hh3cCfgOperateUserPassword | read-create | No | Not supported |
| hh3cCfgOperateEndNotificationSwitch | read-create | No | Not supported |
| hh3cCfgOperateRowStatus | read-create | No | The status of this table entry.When the status is active all the object's value in the entry is not allowed to modify. The config-operation requests which are in progress are not allowed to destroy. |

  - hh3cCfgOperateResultTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgOperateResultIndex | not-accessible | No | As per MIB |
| hh3cCfgOperateResultOptIndex | read-only | No | As per MIB |
| hh3cCfgOperateResultOpType | read-only | No | As per MIB |
| hh3cCfgOperateState | read-only | No | The signification of opFileOpenError(13) is: invalid file name;file not found in partition; access violation |
| hh3cCfgOperateTime | read-only | No | As per MIB |
| hh3cCfgOperateEndTime | read-only | No | As per MIB |

The following values of h3cCfgOperateState are implemented for "netFtp" operation with ftp protocol:

-     1: opInProgress(1)

-     2: opSuccess(2)

-     3: opInvalidOperation(3)

-     4: opInvalidProtocol

-     5: opInvalidSourceName

-     7: opInvalidServerAddress(7)

-     8: opDeviceBusy

-    10: opDeviceError(10)

-    12: opDeviceFull(12)

-    13: opFileOpenError(13)

-    14: opFileTransferError(14)

-    16: opNoMemory(16)

-    17: opAuthFail(17)

-    19: opUnknownFailure(19)

The following values of h3cCfgOperateState are implemented for "netFtp" operation with tftp protocol:

-     1: opInProgress(1)

-     2: opSuccess(2)

-     3: opInvalidOperation(3)

-     4: opInvalidProtocol

-     7: opInvalidServerAddress(7)

-     8: opDeviceBusy

-    10: opDeviceError(10)

-    12: opDeviceFull(12)

-    13: opFileOpenError(13)

-    14: opFileTransferError(14)

-    16: opNoMemory(16)

-    19: opUnknownFailure(19)

The following values of h3cCfgOperateState are implemented for "running2Startup" and "startup2Running" operation:

-     1: opInProgress(1)

-     2: opSuccess(2)

-    10: opDeviceError(10)

  - Scalar objects of hh3cCfgExecuteOperate group(Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgExecuteOperateResultEntryLimit | read-write | Current | Not supported |

  - hh3cCfgExecuteResultTable(Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgExecuteResultIndex | not-accessible | Current | Need confirm by products |
| hh3cCfgExecuteResultOptIndex | read-only | Current | Need confirm by products |
| hh3cCfgExecuteResultOpType | read-only | Current | Need confirm by products |
| hh3cCfgExecuteState | read-only | Current | Need confirm by products |
| hh3cCfgExecuteTime | read-only | Current | Need confirm by products |
| hh3cCfgExecuteEndTime | read-only | Current | Need confirm by products |

  - Notifications(Not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgManEventlog | NA | NA | As per MIB |
| hh3cCfgOperateCompletion | NA | NA | As per MIB |

# HOST-RESOURCES-MIB

## hrProcessorTable

The (conceptual) table of processors contained by the 	host.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hrProcessorFrwID | read-only | No | Not supported |
| hrProcessorLoad | read-only | No | As per MIB |

## scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hrMemorySize | read-only | No | As per MIB |

# HH3C-DHCPS-MIB

## Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cDHCPSServiceStatus | read-write | Current | As per MIB |
| hh3cDHCPSDetectingServerStatus | read-write | Current | Not supported |
| hh3cDHCPSPingNum | read-write | Current | Not supported. |
| hh3cDHCPSPingTimeout | read-write | Current | Not supported |
| hh3cDHCPSWriteDataStatus | read-write | Current | Not support. The value is always disabled(0) |
| hh3cDHCPSWriteDataDirection | read-only | No | Not supported |
| hh3cDHCPSWriteDataDelay | read-write | Current | Not support. The value is always 300. |
| hh3cDHCPSWriteDataRecover | read-write | Current | Not supported. The value is always disabled(0). |
| hh3cDHCPSIPInUseResetIP | read-write | Current | Not supported |
| hh3cDHCPSConflictIPResetIP | read-write | Current | Not supported |
| hh3cDHCPSIPResetFlag | read-write | Current | Not supported |
| hh3cDHCPSGlobalPoolNumber | read-only | Current | Not supported |
| hh3cDHCPSGlobalPoolAutoBindingNum | read-only | No | Not supported |
| hh3cDHCPSGlobalPoolManualBindingNum | read-only | Current | Not supported |
| hh3cDHCPSGlobalPoolExpiredBindingNum | read-only | No | Not supported |
| hh3cDHCPSInterfacePoolNumber | read-only | Current | Not supported |
| hh3cDHCPSInterfacePoolAutoBindingNum | read-only | No | Not supported |
| hh3cDHCPSInterfacePoolManualBindingNum | read-only | Current | Not supported |
| hh3cDHCPSInterfacePoolExpiredBindingNum | read-only | No | Not supported |
| hh3cDHCPSBadPktNum | read-only | No | Not supported |
| hh3cDHCPSBootRequestPktNum | read-only | No | Not supported |
| hh3cDHCPSDiscoverPktNum | read-only | No | Not supported |
| hh3cDHCPSRequestPktNum | read-only | No | Not supported |
| hh3cDHCPSDeclinePktNum | read-only | No | Not supported |
| hh3cDHCPSReleasePktNum | read-only | No | Not supported |
| hh3cDHCPSInformPktNum | read-only | No | Not supported |
| hh3cDHCPSBootReplyPktNum | read-only | No | Not supported |
| hh3cDHCPSOfferPktNum | read-only | No | Not supported |
| hh3cDHCPSAckPktNum | read-only | No | Not supported |
| hh3cDHCPSNakPktNum | read-only | No | Not supported |
| hh3cDHCPSStatisticsReset | read-write | Current | Not supported |

## hh3cDHCPSGlobalPoolTable (not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cDHCPSGlobalPoolName | read-only | Current | As per MIB |
| hh3cDHCPSGlobalPoolRowStatus | read-create | Current | As per MIB |

## hh3cDHCPSGlobalPoolConfigTable(not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cDHCPSGlobalPoolType | read-write | Current | As per MIB |
| hh3cDHCPSGlobalPoolNetwork | read-write | Current | As per MIB. |
| hh3cDHCPSGlobalPoolNetworkMask | read-write | Current | As per MIB |
| hh3cDHCPSGlobalPoolHostIPAddr | read-write | Current | As per MIB |
| hh3cDHCPSGlobalPoolHostMask | read-write | Current | As per MIB |
| hh3cDHCPSGlobalPoolHostHAddr | read-write | Current | As per MIB |
| hh3cDHCPSGlobalPoolConfigUndoFlag | read-write | Current | As per MIB |

## hh3cDHCPSForbiddenIPTable(not supported)

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cDHCPSForbiddenIPStart | read-only | Current | As per MIB |
| hh3cDHCPSForbiddenIPEnd | read-only | Current | As per MIB |
| hh3cDHCPSForbiddenIPRowStatus | read-create | Current | As per MIB |

# HH3C-LswINF-MIB

## hh3cifXXTable

This table is only supported by interfaces working at bridge mode.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cifUnBoundPort | read-only | No | Not supported |
| hh3cifISPhyPort | read-only | No | Not supported |
| hh3cifAggregatePort | read-only | No | Not supported |
| hh3cifMirrorPort | read-write | Yes | Not supported. The value is always false for read operation. |
| hh3cifVLANType | read-write | Yes | read-only, always return 1 |
| hh3cifMcastControl | read-write | Yes | Not supported |
| hh3cifFlowControl | read-write | Yes | Not supported. |
| hh3cifSrcMacControl | read-only | No | Not supported |
| hh3cifClearStat | read-write | No | Not supported |
| hh3cifXXBasePortIndex | read-only | No | As per MIB |
| hh3cifXXDevPortIndex | read-only | No | As per MIB |

# HH3C-LswVLAN-MIB

## hh3cdot1qVlanMIBTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cdot1qVlanIndex | read-only | No | As per MIB |
| hh3cdot1qVlanName | read-write | Current | Not supported |
| hh3cdot1qVlanPorts | read-write | Current | As per MIB |
| hh3cdot1qVlanType | read-write | Current | Only support Common vlan(2) |
| hh3cdot1qVlanMacFilter | read-only | No | Not supported, it’s value is always flase(2), which means “not supported”. |
| hh3cdot1qVlanMcastUnknownProtos | read-only | No | Not supported, it’s value is always flase(2), which means “not supported”. |
| hh3cExistInterface | read-only | No | As per MIB |
| hh3cVlanInterfaceIndex | read-only | No | As per MIB |
| hh3cdot1qVlanMacLearn | read-only | No | Not supported. Its value is always 2(flase), which means “not supported”. |
| hh3cdot1qVlanStatus | read-only | No | Not supported |
| hh3cdot1qVlanCreationTime | read-only | No | Not supported |
| hh3cdot1qVlanPriority | read-write | No | This value is always 0. |
| hh3cdot1qVlanRowStatus | read-create | No | Only support active(1), createAndgo(4) and destroy(6) |
| hh3cdot1qVlanBroadcastSuppression | read-write | No | Not Supported. Always return value 100. |

1.hh3cdot1qVlanMIBTable is used for VLAN information management and monitor, It can be used for VLAN row creation, deletion, and VLAN property configuration and query.

In this table the leaf node can be classified as config node and query node, config node can both be used for config and query purposes, while query node can be used for query only.

The config node in this table include:

hh3cdot1qVlanName

hh3cdot1qVlanPorts

hh3cdot1qVlanType

hh3cdot1qVlanPriority

hh3cdot1qVlanRowStatus

hh3cdot1qVlanBroadcastSuppression

hh3cdot1qVlanBcastSuppressionPPS

hh3cdot1qVlanMulticast (Not support)

Rest of the leaf nodes are query nodes.

2.hh3cdot1qVlanMIBTable configuration instruction:

hh3cdot1qVlanMIBTable use VLAN Index as the table index, which is normally called VLANID,

if a row is to be created, it is recommended to traverse this table first and use the unused VLAN index to create the row.  In this table rowstatus only support active(1), createAndGo(4), destory(6).

For this MIB ,there are two kinds of operation:

2.1 hh3cdot1qVlanRowStatus(createAndGo(4)) and nodes are set together

When the rowstatus node and the following nodes are set simultaneously

hh3cdot1qVlanName

hh3cdot1qVlanPorts

hh3cdot1qVlanType

hh3cdot1qVlanPriority

hh3cdot1qVlanBroadcastSuppression

hh3cdot1qVlanBcastSuppressionPPS

hh3cdot1qVlanMulticast (Not support)

For hh3cdot1qVlanName, VLAN name string check is performed, the row is created only if the string check succeeds.

For rest of the leaf nodes listed above, the row will be created before setting the properties. If errors happen when setting the properties, the operation will fail while the row has already been created.

Here is some examples for the error mentioned above:

1) hh3cdot1qPorts: through this MIB node only access type ports can be added or deleted. Operation will fail while the row has already been created, if some of the ports to be added or deleted are not of access type.

2) hh3cdot1qVlanType: with value sub-vlan(3) can not be set using this MIB, when rowstatus(createAndGo(4)) and hh3cdot1qVlanType(sub-vlan(3)) are set together, Operation will fail while the row has already been created.

It is recommended to set hh3cdot1qVlanRowStatus(createAndGo(4)) and other node properties separately to avoid such kind of error.

2.2 Row already exist and set node properities:

For the following nodes:

hh3cdot1qVlanName      The name length can not exceed 32 charcters.

hh3cdot1qVlanPorts      To set the node successfully you should first get the value of the node, and then only change the corresponding bit of the access port.

hh3cdot1qVlanType       sub-vlan(3)  secondary-vlan(5) can not be set using this MIB

hh3cdot1qVlanPriority

hh3cdot1qVlanBroadcastSuppression

hh3cdot1qVlanBcastSuppressionPPS

hh3cdot1qVlanMulticast (Not support)

3. configuration example

1) Configuration succeess

hh3cdot1qVlanRowStatus(createAndGo(4)) are set together with hh3cdot1qVlanName using value "vlantest", use the index unused in this table, the row is created and hh3cdot1qVlanName is set successfully.

2) Configuration fail

Precondition: there is trunk type port existing on the device and port not belongs to this vlan. hh3cdot1qVlanRowStatus(createAndGo(4)) and hh3cdot1qVlanPorts with trunk port corresponding bit in the octet string setting to 1 are set together, Operation will fail while the row has already been created.

# HH3C-NTP-MIB

## hh3cNTPPeerTable

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cNTPPeerConfig | read-only | No | As per MIB |
| hh3cNTPPeerAuthenable | read-only | No | Not supported |
| hh3cNTPPeerAuthentic | read-only | No | Not supported |
| hh3cNTPPeerRemAdr | not-accessible | No | As per MIB |
| hh3cNTPPeerRemPort | read-only | No | Not supported |
| hh3cNTPPeerLocPort | read-only | No | Not supported |
| hh3cNTPPeerLeap | read-only | No | Not supported |
| hh3cNTPPeerHMode | not-accessible | No | As per MIB |
| hh3cNTPPeerStratum | read-only | No | Not supported |
| hh3cNTPPeerPPoll | read-only | No | Not supported |
| hh3cNTPPeerHPoll | read-only | No | Not supported |
| hh3cNTPPeerPrecision | read-only | No | Not supported |
| hh3cNTPPeerRootDelay | read-only | No | Not supported |
| hh3cNTPPeerRootDispersion | read-only | No | Not supported |
| hh3cNTPPeerRefId | read-only | No | Not supported |
| hh3cNTPPeerRefTime | read-only | No | Not supported |
| hh3cNTPPeerOrg | read-only | No | Not supported |
| hh3cNTPPeerRec | read-only | No | Not supported |
| hh3cNTPPeerXmt | read-only | No | Not supported |
| hh3cNTPPeerReach | read-only | No | Not supported |
| hh3cNTPPeerValid | read-only | No | Not supported |
| hh3cNTPPeerTimer | read-only | No | Not supported |
| hh3cNTPPeerDelay | read-only | No | Not supported |
| hh3cNTPPeerOffset | read-only | No | Not supported |
| hh3cNTPPeerJitter | read-only | No | Not supported |
| hh3cNTPPeerDispersion | read-only | No | Not supported |
| hh3cNTPPeerKeyId | read-only | No | Not supported |
| hh3cNTPPeerFiltDelay | read-only | No | Not supported |
| hh3cNTPPeerFiltOffset | read-only | No | Not supported |
| hh3cNTPPeerFiltError | read-only | No | Not supported |
| hh3cNTPPeerPMode | read-only | No | Not supported |
| hh3cNTPPeerReceived | read-only | No | Not supported |
| hh3cNTPPeerSent | read-only | No | Not supported |
| hh3cNTPPeerFlash | read-only | No | Not supported |
| hh3cNTPPeerRowStatus | read-create | No | Now only support active, createAndGo and destroy |

# HH3C-NET-MAN-MIB

## Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cNMIpAddressType | accessible-for-notify | No | As per MIB |
| hh3cNMIpAddress | accessible-for-notify | No | As per MIB |
| hh3cNMCustomBuildInfo | read-only | No | As per MIB |
| hh3cNMSerialNum | accessible-for-notify | No | As per MIB |

# supported Standard Traps

  - supported Standard Traps

This section contains a list of the traps defined in the standard MIB(defined by IETF or IEEE).

| Trap | MIB | Comments |
| --- | --- | --- |
| ColdStart | SNMPv2-MIB | As per MIB |
| WamStart | SNMPv2-MIB | As per MIB |
| LinkDown | SNMPv2-MIB | As per MIB |
| LinkUp | SNMPv2-MIB | As per MIB |
| AuthenticationFailure | SNMPv2-MIB | As per MIB |
| egpNeighborLoss | SNMPv2-MIB | Not supported |
| risingAlarm | RMON-MIB | Not supported |
| fallingAlarm | RMON-MIB | Not supported |
| entConfigChange | ENTITY-MIB | As per MIB |

  - Supported Private Traps

This section contains a list of the traps defined in the private MIB(defined by enterprise).

    - Common Traps

Traps both supported by router and switch

| Trap | MIB | Comments |
| --- | --- | --- |
| hh3cCfgManEventlog | HH3C-CONFIG-MAN-MIB | As per MIB |
| hh3cCfgOperateCompletion | HH3C-CONFIG-MAN-MIB | Not supported |
| hh3cSysClockChangedNotification | HH3C-SYS-MAN-MIB | Not supported |
| hh3cSysReloadNotification | HH3C-SYS-MAN-MIB | Not supported |
| hh3cFlhOperNotification | HH3C-FLASH-MAN-MIB | Not supported |
| hh3cEntityExtTemperatureThresholdNotification | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtVoltageLowThresholdNotification | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtVoltageHighThresholdNotification | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtCpuUsageThresholdNotfication | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtMemUsageThresholdNotification | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtOperEnabled | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtOperDisabled | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtCriticalTemperatureThresholdNotification | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cRebootSendTrap | HH3C-COMMON-SYSTEM-MIB | As per MIB |
| hh3cIpAddrChangeNotify | HH3C-NET-MAN-MIB | As per MIB |