# Contents

DISMAN-TRACEROUTE-MIB	2

Scalar objects	2

traceRouteCtlTable	2

traceRouteResultsTable	4

traceRouteProbeHistoryTable	5

traceRouteHopsTable	5

DISMAN-TRACEROUTE-MIB

This MIB should be supported by the device that implements the disman traceroute function. This MIB cannot mix using with CLI.

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| traceRouteMaxConcurrentRequests (1.3.6.1.2.1.81.1.1) | read-only | Current | It is the same with pingMaxConcurrentRequests. |

traceRouteCtlTable

OID of this table is: 1.3.6.1.2.1.81.1.2

  - When creating a test entry, the default value of traceRouteCtlType is traceRouteUsingUdpProbes. The current traceRouteCtlType only supports traceRouteUsingUdpProbes.

  - If any of the configurations in traceRouteCtlTable changes, the corresponding results, histories and statistics will be cleared, except the following objects:

traceRouteCtlTrapGeneration

traceRouteCtlDescr

traceRouteCtlMaxRows (If the value of this object has changed to a smaller value, history records beyond the value will be deleted.)

The results, histories and statistics must match the current configurations. If not, users will get confused.

  - traceRouteCtlTargetAddressType and traceRouteCtlTargetAddress must be modified together.

  - traceRouteCtlSourceAddressType and traceRouteCtlSourceAddress must be modified together.

  - The value of traceRouteCtlOwnerIndex and traceRouteCtlTestName must be different from pingCtlOwnerIndex and pingCtlTestName when creating a new entry.

  - Any of the configurations in traceRouteCtlTable cannot be changed when the traceRouteCtlAdminStatus is enabled.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| traceRouteCtlOwnerIndex (1.3.6.1.2.1.81.1.2.1.1) | not-accessible | Current | The uppercase letters in this string will be converted to lowercase letters in a setting operation. The value of this object cannot contain hyphens (-) or question marks (?). |
| traceRouteCtlTestName (1.3.6.1.2.1.81.1.2.1.2) | not-accessible | Current | The uppercase letters in this string will be converted to lowercase letters in a setting operation. The value of this object cannot contain hyphens (-) or question marks (?). |
| traceRouteCtlTargetAddressType (1.3.6.1.2.1.81.1.2.1.3) | read-create | Current | Only supports ipv4(1) and unknown(0). |
| traceRouteCtlTargetAddress (1.3.6.1.2.1.81.1.2.1.4) | read-create | Current | If the value of traceRouteCtlTargetAddressType is ipv4(1), the value of this object must be an IPv4 address. If the value of traceRouteCtlTargetAddressType is unknown(0), the value of this object must be a zero-length string. |
| traceRouteCtlByPassRouteTable(1.3.6.1.2.1.81.1.2.1.5) | read-create | Current | As per MIB |
| traceRouteCtlDataSize (1.3.6.1.2.1.81.1.2.1.6) | read-create | Current | The range is from 20 to 8100. The default value is 100. |
| traceRouteCtlTimeOut (1.3.6.1.2.1.81.1.2.1.7) | read-create | Current | As per MIB. |
| traceRouteCtlProbePerHop (1.3.6.1.2.1.81.1.2.1.8) | read-create | Current | As per MIB. |
| traceRouteCtlPort (1.3.6.1.2.1.81.1.2.1.9) | read-create | Current | As per MIB. |
| traceRouteCtlMaxTtl (1.3.6.1.2.1.81.1.2.1.10) | read-create | Current | As per MIB. |
| traceRouteCtlDSField (1.3.6.1.2.1.81.1.2.1.11) | read-create | Current | As per MIB |
| traceRouteCtlSourceAddressType (1.3.6.1.2.1.81.1.2.1.12) | read-create | Current | Only supports ipv4(1) and unknown(0). Default value is unknown(0). |
| traceRouteCtlSourceAddress (1.3.6.1.2.1.81.1.2.1.13) | read-create | Current | If the value of traceRouteCtlSourceAddressType is ipv4(1), the value of this object must be an IPv4 address. If the value of traceRouteCtlSourceAddressType is unknown(0), the value of this object must be a zero-length string. |
| traceRouteCtlIfIndex (1.3.6.1.2.1.81.1.2.1.14) | read-create | Current | As per MIB. |
| traceRouteCtlMiscOptions (1.3.6.1.2.1.81.1.2.1.15) | read-create | Current | The length of this object is from 0 to 200. The corresponding function is not supported. |
| traceRouteCtlMaxFailures (1.3.6.1.2.1.81.1.2.1.16) | read-create | Current | As per MIB. |
| traceRouteCtlDontFragment (1.3.6.1.2.1.81.1.2.1.17) | read-create | Current | As per MIB. |
| traceRouteCtlInitialTtl (1.3.6.1.2.1.81.1.2.1.18) | read-create | Current | As per MIB. |
| traceRouteCtlFrequency (1.3.6.1.2.1.81.1.2.1.19) | read-create | Current | Range from 0 to 604800 If the value is not times of seconds (set by CLI), the result of reading operation is inaccurate. Such as: If the value set by CLI is 1050ms, the result of reading operation is 2s. The default value is 0s. |
| traceRouteCtlStorageType (1.3.6.1.2.1.81.1.2.1.20) | read-create | Current | The corresponding function is not supported. |
| traceRouteCtlAdminStatus (1.3.6.1.2.1.81.1.2.1.21) | read-create | Current | As per MIB |
| traceRouteCtlDescr (1.3.6.1.2.1.81.1.2.1.22) | read-create | Current | The length of this object is from 0 to 200. The value cannot start with space. |
| traceRouteCtlMaxRows (1.3.6.1.2.1.81.1.2.1.23) | read-create | Current | Ranges from 0 to 50. |
| traceRouteCtlTrapGeneration (1.3.6.1.2.1.81.1.2.1.24) | read-create | Current | As per MIB. |
| traceRouteCtlCreateHopsEntries(1.3.6.1.2.1.81.1.2.1.25) | read-create | Current | As per MIB. |
| traceRouteCtlType (1.3.6.1.2.1.81.1.2.1.26) | read-create | Current | Can not be modified after creation. |
| traceRouteCtlRowStatus (1.3.6.1.2.1.81.1.2.1.27) | read-create | Current | Only supports active(1), createAndgo(4) and destroy(6). |

traceRouteResultsTable

OID of this table is: 1.3.6.1.2.1.81.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| traceRouteResultsOperStatus (1.3.6.1.2.1.81.1.3.1.1) | read-only | No | As per MIB |
| traceRouteResultsCurHopCount (1.3.6.1.2.1.81.1.3.1.2) | read-only | No | As per MIB |
| traceRouteResultsCurProbeCount (1.3.6.1.2.1.81.1.3.1.3) | read-only | No | As per MIB |
| traceRouteResultsIpTgtAddrType (1.3.6.1.2.1.81.1.3.1.4) | read-only | No | As per MIB |
| traceRouteResultsIpTgtAddr (1.3.6.1.2.1.81.1.3.1.5) | read-only | No | As per MIB |
| traceRouteResultsTestAttempts (1.3.6.1.2.1.81.1.3.1.6) | read-only | No | As per MIB |
| traceRouteResultsTestSuccesses (1.3.6.1.2.1.81.1.3.1.7) | read-only | No | As per MIB |
| traceRouteResultsLastGoodPath (1.3.6.1.2.1.81.1.3.1.8) | read-only | No | As per MIB |

traceRouteProbeHistoryTable

OID of this table is: 1.3.6.1.2.1.81.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| traceRouteProbeHistoryIndex (1.3.6.1.2.1.81.1.4.1.1) | not-accessible | No | As per MIB |
| traceRouteProbeHistoryHopIndex (1.3.6.1.2.1.81.1.4.1.2) | not-accessible | No | As per MIB |
| traceRouteProbeHistoryProbeIndex (1.3.6.1.2.1.81.1.4.1.3) | not-accessible | No | As per MIB |
| traceRouteProbeHistoryHAddrType(1.3.6.1.2.1.81.1.4.1.4) | read-only | No | As per MIB |
| traceRouteProbeHistoryHAddr (1.3.6.1.2.1.81.1.4.1.5) | read-only | No | As per MIB |
| traceRouteProbeHistoryResponse (1.3.6.1.2.1.81.1.4.1.6) | read-only | No | As per MIB |
| traceRouteProbeHistoryStatus (1.3.6.1.2.1.81.1.4.1.7) | read-only | No | As per MIB |
| traceRouteProbeHistoryLastRC (1.3.6.1.2.1.81.1.4.1.8) | read-only | No | As per MIB |
| traceRouteProbeHistoryTime (1.3.6.1.2.1.81.1.4.1.9) | read-only | No | As per MIB |

traceRouteHopsTable

OID of this table is: 1.3.6.1.2.1.81.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| traceRouteHopsHopIndex (1.3.6.1.2.1.81.1.5.1.1) | not-accessible | No | As per MIB |
| traceRouteHopsIpTgtAddressType (1.3.6.1.2.1.81.1.5.1.2) | read-only | No | As per MIB |
| traceRouteHopsIpTgtAddress (1.3.6.1.2.1.81.1.5.1.3) | read-only | No | As per MIB |
| traceRouteHopsMinRtt (1.3.6.1.2.1.81.1.5.1.4) | read-only | No | As per MIB |
| traceRouteHopsMaxRtt (1.3.6.1.2.1.81.1.5.1.5) | read-only | No | As per MIB |
| traceRouteHopsAverageRtt (1.3.6.1.2.1.81.1.5.1.6) | read-only | No | As per MIB |
| traceRouteHopsRttSumOfSquares (1.3.6.1.2.1.81.1.5.1.7) | read-only | No | As per MIB |
| traceRouteHopsSentProbes (1.3.6.1.2.1.81.1.5.1.8) | read-only | No | As per MIB |
| traceRouteHopsProbeResponses (1.3.6.1.2.1.81.1.5.1.9) | read-only | No | As per MIB |
| traceRouteHopsLastGoodProbe (1.3.6.1.2.1.81.1.5.1.10) | read-only | No | As per MIB |