# Contents

HH3C-CONFIG-MAN-MIB	2

hh3cCfgLog group	2

Scalar objects of hh3cCfgLog group	2

hh3cCfgLogTable	3

hh3cCfgOperate group	4

Scalar objects of hh3cCfgOperate group	4

hh3cCfgOperateTable	4

hh3cCfgOperateResultTable	6

Scalar objects of hh3cCfgExecuteOperate group	6

hh3cCfgExecuteResultTable	7

HH3C-CONFIG-MAN-MIB

A configuration management MIB, includes configuration log management and Backup & Restore operation of configuration.

All the configuration change events can be managed by the log management MIB. The configuration may be changed by CLI, SNMP or other ways defined as the MIB.

Backup & Restore MIB group provides the configuration backup and restore among flash, memory and network.

Note: Writing flash can be done by backup(save) as described in figure below.

Flash

Memory

Network

restore

backup(ftp/tftp)

backup(save)

restore(ftp/tftp)

backup(ftp/tftp)

restore

hh3cCfgLog group

Scalar objects of hh3cCfgLog group

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgRunModifiedLast (1.3.6.1.4.1.25506.2.4.1.1.1) | read-only | No | If system detects the current configuration has been changed, the value will be updated. Executing SNMP SET operation, the value will also be updated. |
| hh3cCfgRunSavedLast (1.3.6.1.4.1.25506.2.4.1.1.2) | read-only | No | As per MIB |
| hh3cCfgStartModifiedLast (1.3.6.1.4.1.25506.2.4.1.1.3) | read-only | No | As per MIB |
| hh3cCfgLogLimitedEntries (1.3.6.1.4.1.25506.2.4.1.1.4) | read-only | No | As per MIB |
| hh3cCfgLogDeletedEntries (1.3.6.1.4.1.25506.2.4.1.1.5) | read-only | No | As per MIB |
| hh3cCfgLogWantBackup (1.3.6.1.4.1.25506.2.4.1.1.6) | read-write | No | Available for distributed systems. Always false value for a centralized system. |
| hh3cCfgFirstTrapTime (1.3.6.1.4.1.25506.2.4.1.1.8) | accessible-for-notify | No | Not supported |

hh3cCfgLogTable

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.1.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgLogIndex (1.3.6.1.4.1.25506.2.4.1.1.7.1.1) | not-accessible | No | As per MIB |
| hh3cCfgLogTime (1.3.6.1.4.1.25506.2.4.1.1.7.1.2) | read-only | No | As per MIB |
| hh3cCfgLogSrcCmd (1.3.6.1.4.1.25506.2.4.1.1.7.1.3) | read-only | No | As per MIB |
| hh3cCfgLogSrcData (1.3.6.1.4.1.25506.2.4.1.1.7.1.4) | read-only | No | As per MIB |
| hh3cCfgLogDesData (1.3.6.1.4.1.25506.2.4.1.1.7.1.5) | read-only | No | As per MIB |
| hh3cCfgLogTerminalType (1.3.6.1.4.1.25506.2.4.1.1.7.1.6) | read-only | No | As per MIB |
| hh3cCfgLogTerminalUser (1.3.6.1.4.1.25506.2.4.1.1.7.1.7) | read-only | No | When users modify configuration through SNMP agent, the value of this instance is the name of the SNMP v3 user. A snmp operation produce two piece of log, the value of this instance in the second log is a zero length string. |
| hh3cCfgLogTerminalNum (1.3.6.1.4.1.25506.2.4.1.1.7.1.8) | read-only | No | As per MIB |
| hh3cCfgLogTerminalLocation (1.3.6.1.4.1.25506.2.4.1.1.7.1.9) | read-only | No | As per MIB |
| hh3cCfgLogCmdSrcAddress (1.3.6.1.4.1.25506.2.4.1.1.7.1.10) | read-only | No | As per MIB |
| hh3cCfgLogVirHost (1.3.6.1.4.1.25506.2.4.1.1.7.1.11) | read-only | No | A zero length string. |
| hh3cCfgLogUserName (1.3.6.1.4.1.25506.2.4.1.1.7.1.12) | read-only | No | When the value of hh3cCfgLogSrcCmd is “cmdLine”, the value of this instance is a zero length string. |
| hh3cCfgLogServerAddress (1.3.6.1.4.1.25506.2.4.1.1.7.1.13) | read-only | No | A snmp operation produce two piece of log, the value of this instance in the first log is always 0.0.0.0, and this instance in the second log is 0.0.0.0 when hh3cCfgLogSrcData and hh3cCfgLogDesData haven’t the value of “netFtp”. |
| hh3cCfgLogFile (1.3.6.1.4.1.25506.2.4.1.1.7.1.14) | read-only | No | As per MIB |
| hh3cCfgLogCmdSrcAddrType (1.3.6.1.4.1.25506.2.4.1.1.7.1.15) | read-only | No | Not supported |
| hh3cCfgLogCmdSrcAddrRev (1.3.6.1.4.1.25506.2.4.1.1.7.1.16) | read-only | No | Not supported |
| hh3cCfgLogCmdSrcAddrVPNName(1.3.6.1.4.1.25506.2.4.1.1.7.1.17) | read-only | No | Not supported |
| hh3cCfgLogServerAddrType (1.3.6.1.4.1.25506.2.4.1.1.7.1.18) | read-only | No | Not supported |
| hh3cCfgLogServerAddrRev (1.3.6.1.4.1.25506.2.4.1.1.7.1.19) | read-only | No | Not supported |
| hh3cCfgLogServerAddrVPNName(1.3.6.1.4.1.25506.2.4.1.1.7.1.20) | read-only | No | Not supported |

hh3cCfgOperate group

Scalar objects of hh3cCfgOperate group

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgOperateGlobalEntryLimit (1.3.6.1.4.1.25506.2.4.1.2.1) | read-only | No | As per MIB |
| hh3cCfgOperateEntryAgeOutTime (1.3.6.1.4.1.25506.2.4.1.2.2) | read-write | No | As per MIB |
| hh3cCfgOperateResultGlobalEntryLimit (1.3.6.1.4.1.25506.2.4.1.2.3) | read-write | No | As per MIB |
| hh3cCfgReset 1.3.6.1.4.1.25506.2.4.1.2.7 | read-write | No | As per MIB |

hh3cCfgOperateTable

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.2.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgOperateIndex (1.3.6.1.4.1.25506.2.4.1.2.4.1.1) | not-accessible | No | This index can be randomly selected from 1 to 2147483647 |
| hh3cCfgOperateType (1.3.6.1.4.1.25506.2.4.1.2.4.1.2) | read-create | No | As per MIB |
| hh3cCfgOperateProtocol (1.3.6.1.4.1.25506.2.4.1.2.4.1.3) | read-create | No | As per MIB |
| hh3cCfgOperateFileName (1.3.6.1.4.1.25506.2.4.1.2.4.1.4) | read-create | No | The maximum length of the full file name (including path and file name) is 128. |
| hh3cCfgOperateServerAddress (1.3.6.1.4.1.25506.2.4.1.2.4.1.5) | read-create | No | As per MIB |
| hh3cCfgOperateUserName (1.3.6.1.4.1.25506.2.4.1.2.4.1.6) | read-create | No | When the operation type is running2Net, net2Running, net2Startup or startup2net, the user name for the ftp server from/to which to download/upload should be specified. The object must be created if hh3cCfgOperateProtocol has the value of ftp. The length of name is no more than 40 characters, and must larger than 0 characters. |
| hh3cCfgOperateUserPassword (1.3.6.1.4.1.25506.2.4.1.2.4.1.7) | read-create | No | When the operation type is running2Net, net2Running, net2Startup or startup2net, the user password for the ftp server from/to which to download/upload should be specified. The object must be created if hh3cCfgOperateProtocol has the value of ftp. The length of password is no more than 40 characters, and must larger than 0 characters. A zero-length string will be returned when read. |
| hh3cCfgOperateEndNotificationSwitch (1.3.6.1.4.1.25506.2.4.1.2.4.1.8) | read-create | No | As per MIB |
| hh3cCfgOperateRowStatus (1.3.6.1.4.1.25506.2.4.1.2.4.1.9) | read-create | No | The status of this table entry. When the status is active all the object's value in the entry is not allowed to modify, except itself. The configuration operation requests which are in progress are not allowed to destroy. |
| hh3cCfgOperateServerPort (1.3.6.1.4.1.25506.2.4.1.2.4.1.10) | read-create | No | As per MIB |
| hh3cCfgOperateSrvAddrType (1.3.6.1.4.1.25506.2.4.1.2.4.1.11) | read-create | No | Only support ipv4,ipv6 and dns. |
| hh3cCfgOperateSrvAddrRev (1.3.6.1.4.1.25506.2.4.1.2.4.1.12) | read-create | No | When the address type is ipv4,ipv6 or dns, the address should be specified. when it is ipv4 address,the length of address must be 4 characters; when it is ipv6 address,the length of address must be 16 characters; when it is dns,the length of address is no more than 253 characters and must larger than 0 characters. |
| hh3cCfgOperateSrvVPNName(1.3.6.1.4.1.25506.2.4.1.2.4.13) | read-create | No | The length of vpn is no more than 31 characters, and must larger than 0 characters. |

hh3cCfgOperateResultTable

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.2.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgOperateResultIndex (1.3.6.1.4.1.25506.2.4.1.2.5.1.1) | not-accessible | No | As per MIB |
| hh3cCfgOperateResultOptIndex (1.3.6.1.4.1.25506.2.4.1.2.5.1.2) | read-only | No | As per MIB |
| hh3cCfgOperateResultOpType (1.3.6.1.4.1.25506.2.4.1.2.5.1.3) | read-only | No | As per MIB |
| hh3cCfgOperateState (1.3.6.1.4.1.25506.2.4.1.2.5.1.4) | read-only | No | The operation result of the hh3cCfgOperateTable. The signification of opFileOpenError(13) is: invalid file name; file not found in partition; access violation. |
| hh3cCfgOperateTime (1.3.6.1.4.1.25506.2.4.1.2.5.1.5) | read-only | No | As per MIB |
| hh3cCfgOperateEndTime (1.3.6.1.4.1.25506.2.4.1.2.5.1.6) | read-only | No | As per MIB |
| hh3cCfgOperFailReason (1.3.6.1.4.1.25506.2.4.1.2.5.1.7) | read-only | No | As per MIB |

Scalar objects of hh3cCfgExecuteOperate group

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.2.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgExecuteOperateResultEntryLimit (1.3.6.1.4.1.25506.2.4.1.2.6.1) | read-write | No | As per MIB |

hh3cCfgExecuteResultTable

OID of this table is: 1.3.6.1.4.1.25506.2.4.1.2.6.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cCfgExecuteResultIndex (1.3.6.1.4.1.25506.2.4.1.2.6.2.1.1) | not-accessible | No | As per MIB |
| hh3cCfgExecuteResultOptIndex (1.3.6.1.4.1.25506.2.4.1.2.6.2.1.2) | read-only | No | As per MIB |
| hh3cCfgExecuteResultOpType (1.3.6.1.4.1.25506.2.4.1.2.6.2.1.3) | read-only | No | As per MIB |
| hh3cCfgExecuteState (1.3.6.1.4.1.25506.2.4.1.2.6.2.1.4) | read-only | No | As per MIB |
| hh3cCfgExecuteTime (1.3.6.1.4.1.25506.2.4.1.2.6.2.1.5) | read-only | No | As per MIB |
| hh3cCfgExecuteEndTime (1.3.6.1.4.1.25506.2.4.1.2.6.2.1.6) | read-only | No | As per MIB |