# Contents

HH3C-MDC-MIB	2

Scalar objects	2

hh3cMDCControlTable	2

hh3cMDCDISKResourceTable	3

hh3CMDCMemoryResourceTable	3

hh3cMDCCPUResourceTable	4

hh3cMDCLocationTable	4

hh3cMDCGroupIfTable	4

hh3cMDCAllocateTable	4

HH3C-MDC-MIB

The HH3C-MDC-MIB contains objects to manage MDC (Multitenant Devices Context) operations.

Such as:

1. Configure MDCs for a device and display MDCs and their status.

2. Assign CPU, storage space, and memory space resources to the MDC, and display their status.

3. Authorize the MDC to use the interface cards, and Assign physical interfaces to an MDC.

Default MDC: A device supporting MDCs is an MDC itself, and it is called the default MDC.        From the default MDC, you can manage the entire physical device, create and delete Non-default MDCs, and assign interfaces, CPU resources, storage space, and memory space to Non-default MDCs.

Non-default MDC: No MDCs can be created on a Non-default MDC. Administrators of Non-default MDCs can only manage and maintain their respective MDCs. A Non-default MDC can use only the resources assigned to it.

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCMaxMDCNum (1.3.6.1.4.1.25506.2.136.1.1) | read-only | No | The maximum number of MDCs depends on the device model. |
| hh3cMDCCurrentMDCNum (1.3.6.1.4.1.25506.2.136.1.2) | read-only | No | As per MIB |

hh3cMDCControlTable

Note:When creating a new instance of this table, the following objects should be set simultaneously:

hh3cMDCName, hh3cMDCRowStatus.

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCIndex (1.3.6.1.4.1.25506.2.136.2.1.1.1.1) | accessible-for-notify | No | MDC Index cannot be modified after creation The default MDC always uses the ID 1, its ID cannot be changed. |
| hh3cMDCName (1.3.6.1.4.1.25506.2.136.2.1.1.1.2) | read-create | Current | Can not be modified after creation. The default MDC always uses the name “Admin”, its name cannot be changed. |
| hh3cMDCAction (1.3.6.1.4.1.25506.2.136.2.1.1.1.3) | read-write | Current | To avoid configuration loss, save the running configuration of an MDC before stopping the MDC. |
| hh3cMDCStatus (1.3.6.1.4.1.25506.2.136.2.1.1.1.4) | read-only | No | As per MIB |
| hh3cMDCRowStatus (1.3.6.1.4.1.25506.2.136.2.1.1.1.5) | read-create | No | Only support active(1),createAndGo(4), and destroy(6). The default MDC is system predefined. You cannot delete it. Deleting an MDC removes all configurations and hard disk files of the MDC, and the configurations and files cannot be restored. |

hh3cMDCDISKResourceTable

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCDISKResourceInstance (1.3.6.1.4.1.25506.2.136.2.2.1.1.1) | not-accessible | No | As per MIB |
| hh3cMDCDISKResourceInstanceName (1.3.6.1.4.1.25506.2.136.2.2.1.1.2) | read-only | No | As per MIB |
| hh3cMDCDISKResourceMinLimit (1.3.6.1.4.1.25506.2.136.2.2.1.1.3) | read-write | Current | Reserve resources The value must be 0. |
| hh3cMDCDISKResourceMaxLimit (1.3.6.1.4.1.25506.2.136.2.2.1.1.4) | read-write | Current | Range from 1 to 100 |
| hh3cMDCDISKResourceReserve (1.3.6.1.4.1.25506.2.136.2.2.1.1.5) | read-only | No | Reserve resources Not supported |
| hh3cMDCDISKResourceQuota (1.3.6.1.4.1.25506.2.136.2.2.1.1.6) | read-only | No | As per MIB |
| hh3cMDCDISKResourceUsage (1.3.6.1.4.1.25506.2.136.2.2.1.1.7) | read-only | No | As per MIB |
| hh3cMDCDISKResourceAvailable (1.3.6.1.4.1.25506.2.136.2.2.1.1.8) | read-only | No | As per MIB |

hh3CMDCMemoryResourceTable

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.2.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3CMDCMemoryResourceMinLimit (1.3.6.1.4.1.25506.2.136.2.2.2.1.1) | read-write | Current | Reserve resources The value must be 0. |
| hh3CMDCMemoryResourceMaxLimit (1.3.6.1.4.1.25506.2.136.2.2.2.1.2) | read-write | Current | Range from 1 to 100 |
| hh3CMDCMemoryResourceReserve (1.3.6.1.4.1.25506.2.136.2.2.2.1.3) | read-only | No | Reserve resources Not supported |
| hh3CMDCMemoryResourceQuota (1.3.6.1.4.1.25506.2.136.2.2.2.1.4) | read-only | No | As per MIB |
| hh3CMDCMemoryResourceUsage (1.3.6.1.4.1.25506.2.136.2.2.2.1.5) | read-only | No | As per MIB |
| hh3CMDCMemoryResourceAvailable (1.3.6.1.4.1.25506.2.136.2.2.2.1.6) | read-only | No | As per MIB |

hh3cMDCCPUResourceTable

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.2.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCCPUResourceLimit (1.3.6.1.4.1.25506.2.136.2.2.3.1.1) | read-write | Current | The default value is 10 Range from 1 to 10. |
| hh3cMDCCPUResourceUsage (1.3.6.1.4.1.25506.2.136.2.2.3.1.2) | read-only | No | As per MIB |

hh3cMDCLocationTable

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCLocationStatus (1.3.6.1.4.1.25506.2.136.2.3.1.1.1) | read-write | Current | As per MIB |

hh3cMDCGroupIfTable

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.4.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCGroupIdentity (1.3.6.1.4.1.25506.2.136.2.4.1.1.2) | read-only | No | As per MIB |

hh3cMDCAllocateTable

OID of this table is: 1.3.6.1.4.1.25506.2.136.2.4.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cMDCAllocateGroupIndex (1.3.6.1.4.1.25506.2.136.2.4.2.1.1) | not-accessible | No | As per MIB |
| hh3cMDCAllocateGroupDescription (1.3.6.1.4.1.25506.2.136.2.4.2.1.2) | read-only | No | As per MIB |
| hh3cMDCAllocateMDCId (1.3.6.1.4.1.25506.2.136.2.4.2.1.3) | read-write | Current | As per MIB |