# Contents

HH3C-SSH-MIB	2

hh3cSSHServerGlobalConfig	2

hh3cSSHUserConfigTable	2

hh3cSSHSessionInfoTable	3

Scalar objects for notify	4

HH3C-SSH-MIB

Secure Shell (SSH) is a network security protocol. Using encryption and authentication, SSH can implement secure remote access and file transfer over an insecure network. Adopting the typical client/server model, SSH can establish a channel to protect data transfer based on TCP. SSH includes two versions: SSH1.x and SSH2.0 (hereinafter referred to as SSH1 and SSH2), which are not compatible. SSH2 is better than SSH1 in performance and security.

The HH3C SSH MIB is used to define that managed for SSH Service。

hh3cSSHServerGlobalConfig

OID of this table is: 1.3.6.1.4.1.25506.2.22.1.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSSHServerVersion (1.3.6.1.4.1.25506.2.22.1.1.1.1) | read-only | No | The default value is 1.99, meaning that it is compatible with SSH versions 1.x. |
| hh3cSSHServerCompatibleSSH1x (1.3.6.1.4.1.25506.2.22.1.1.1.2) | read-write | Current | The default value is enableCompatibleSSH1x(1). |
| hh3cSSHServerRekeyInterval (1.3.6.1.4.1.25506.2.22.1.1.1.3) | read-write | Current | Range from 0 to 24. The default value is 0, meaning that the key will not be refreshed. |
| hh3cSSHServerAuthRetries (1.3.6.1.4.1.25506.2.22.1.1.1.4) | read-write | Current | Range from 1 to 5. The default value is 3. |
| hh3cSSHServerAuthTimeout (1.3.6.1.4.1.25506.2.22.1.1.1.5) | read-write | Current | Range from 1 to 120. The default value is 60. |
| hh3cSFTPServerIdleTimeout (1.3.6.1.4.1.25506.2.22.1.1.1.6) | read-write | Current | Range from 1 to 35791. The default value is 10. |
| hh3cSSHServerEnable (1.3.6.1.4.1.25506.2.22.1.1.1.7) | read-write | Current | The default value is disableSSHServer(2). |
| hh3cSFTPServerEnable (1.3.6.1.4.1.25506.2.22.1.1.1.8) | read-write | Current | The default value is disableSFTPService(2). |
| hh3cSTelnetServerEnable (1.3.6.1.4.1.25506.2.22.1.1.1.1.9) | read-write | Current | As per MIB |
| hh3cSCPServerEnable (1.3.6.1.4.1.25506.2.22.1.1.1.1.10) | read-write | Current | As per MIB |

hh3cSSHUserConfigTable

OID of this table is: 1.3.6.1.4.1.25506.2.22.1.1.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSSHUserName (1.3.6.1.4.1.25506.2.22.1.1.2.1.1.1) | not-accessible | Current | The length of value is from 1 to 80. |
| hh3cSSHUserServiceType (1.3.6.1.4.1.25506.2.22.1.1.2.1.1.2) | read-create | Current | The default value is invalid(1). |
| hh3cSSHUserAuthType (1.3.6.1.4.1.25506.2.22.1.1.2.1.1.3) | read-create | Current | The default value is invalid(1). When this object is set to password, means that this user uses password authentication in AAA(Authentication, Authorization, Accounting) module. When this object is set to publicKey, means that this user uses public key authentication in PKEY(Public Key) module. When this object is set to any, means that this user uses either password authentication or publickey authentication. When this object is set to publicKeyPassword, means that this user uses both publickey authentication and password authentication. |
| hh3cSSHUserPublicKeyName (1.3.6.1.4.1.25506.2.22.1.1.2.1.1.4) | read-create | Current | The length of value is from 1 to 64. The default value is null string. This object only can get value from an existent public key, which is managed in PKEY (Public Key) module. |
| hh3cSSHUserWorkDirectory (1.3.6.1.4.1.25506.2.22.1.1.2.1.1.5) | read-create | Current | Not supported. |
| hh3cSSHUserRowStatus (1.3.6.1.4.1.25506.2.22.1.1.2.1.1.6) | read-create | No | Four actions are used: active(1), notInService(2), createAndGo(4), destroy(6). When `hh3cSSHUserRowStatus' is set to active(1), no object in the conceptual row can be modified. In particular, a newly created user row which uses public key authentication cannot be made active(1) until the corresponding instance of `hh3cSSHUserAuthType' is 'publicKey' or 'publicKeyPassword', and the 'hh3cSSHUserPublicKeyName' is configured appropriately. |

hh3cSSHSessionInfoTable

OID of this table is: 1.3.6.1.4.1.25506.2.22.1.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSSHSessionID (1.3.6.1.4.1.25506.2.22.1.1.3.1.1) | not-accessible | No | As per MIB |
| hh3cSSHSessionUserName (1.3.6.1.4.1.25506.2.22.1.1.3.1.2) | read-only | No | The length of value is from 1 to 80. |
| hh3cSSHSessionUserIpAddrType (1.3.6.1.4.1.25506.2.22.1.1.3.1.3) | read-only | No | As per MIB |
| hh3cSSHSessionUserIpAddr (1.3.6.1.4.1.25506.2.22.1.1.3.1.4) | read-only | No | As per MIB |
| hh3cSSHSessionClientVersion (1.3.6.1.4.1.25506.2.22.1.1.3.1.5) | read-only | No | As per MIB |
| hh3cSSHSessionServiceType (1.3.6.1.4.1.25506.2.22.1.1.3.1.6) | read-only | No | As per MIB |
| hh3cSSHSessionEncry (1.3.6.1.4.1.25506.2.22.1.1.3.1.7) | read-only | No | As per MIB |
| hh3cSSHSessionState (1.3.6.1.4.1.25506.2.22.1.1.3.1.8) | read-only | No | As per MIB |

Scalar objects for notify

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cSSHAttemptUserName (1.3.6.1.4.1.25506.2.22.1.2.1) | accessible-for-notify | No | The length of value is from 1 to 80. |
| hh3cSSHAttemptIpAddrType (1.3.6.1.4.1.25506.2.22.1.2.2) | accessible-for-notify | No | As per MIB |
| hh3cSSHAttemptIpAddr (1.3.6.1.4.1.25506.2.22.1.2.3) | accessible-for-notify | No | As per MIB |
| hh3cSSHUserAuthFailureReason (1.3.6.1.4.1.25506.2.22.1.2.4) | accessible-for-notify | No | As per MIB |