# Contents

HH3C-VSI-MIB	2

hh3cVsiScalarGroup	2

hh3cVsiTable	2

hh3cVsiXconnectTable	3

hh3cVsiPwBindTable	3

HH3C-VSI-MIB

hh3cVsiScalarGroup

OID of this table is: 1.3.6.1.4.1.25506.2.105.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cVsiNextAvailableVsiIndex (1.3.6.1.4.1.25506.2.105.1.1.1) | read-only | Current | As per MIB |
| hh3cVsiL2vpnStatus (1.3.6.1.4.1.25506.2.105.1.1.2) | read-write | Current | As per MIB |

hh3cVsiTable

OID of this table is: 1.3.6.1.4.1.25506.2.105.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cVsiIndex (1.3.6.1.4.1.25506.2.105.1.2.1.1) | not-accessible | No | As per MIB |
| hh3cVsiName (1.3.6.1.4.1.25506.2.105.1.2.1.2) | read-create | Current | Can not be modified after creation |
| hh3cVsiMode (1.3.6.1.4.1.25506.2.105.1.2.1.3) | read-create | Current | Not supported. The value is always 0. The variable value of set operation is ignored. |
| hh3cMinmIsid (1.3.6.1.4.1.25506.2.105.1.2.1.4) | read-create | Current | Can not be modified after creation |
| hh3cVsiId (1.3.6.1.4.1.25506.2.105.1.2.1.5) | read-create | Current | Can not be modified after creation |
| hh3cVsiTransMode (1.3.6.1.4.1.25506.2.105.1.2.1.6) | read-create | Current | Not supported. The value is always vlan(1). The variable value of set operation is ignored. |
| hh3cVsiEnableHubSpoke (1.3.6.1.4.1.25506.2.105.1.2.1.7) | read-create | Current | Can not be modified after creation |
| hh3cVsiAdminState (1.3.6.1.4.1.25506.2.105.1.2.1.8) | read-create | Current | The default value is adminUp(1) |
| hh3cVsiRowStatus (1.3.6.1.4.1.25506.2.105.1.2.1.9) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6). Active(1) is only used to change the value of hh3cVsiAdminState. |
| hh3cVsiSpbIsid (1.3.6.1.4.1.25506.2.105.1.2.1.10) | read-create | Current | Can not be modified after creation |

hh3cVsiXconnectTable

OID of this table is: 1.3.6.1.4.1.25506.2.105.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cVsiXconnectIfIndex (1.3.6.1.4.1.25506.2.105.1.3.1.1) | not-accessible | Currnet | As per MIB |
| hh3cVsiXconnectEvcSrvInstId (1.3.6.1.4.1.25506.2.105.1.3.1.2) | not-accessible | Current | As per MIB |
| hh3cVsiXconnectVsiName (1.3.6.1.4.1.25506.2.105.1.3.1.3) | read-create | Current | Can not be modified after creation |
| hh3cVsiXconnectAccessMode (1.3.6.1.4.1.25506.2.105.1.3.1.4) | read-create | Current | Can not be modified after creation |
| hh3cVsiXconnectHubSpoke (1.3.6.1.4.1.25506.2.105.1.3.1.5) | read-create | Current | Can not be modified after creation |
| hh3cVsiXconnectRowStatus (1.3.6.1.4.1.25506.2.105.1.3.1.6) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |

hh3cVsiPwBindTable

OID of this table is: 1.3.6.1.4.1.25506.2.105.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cVsiPwIndex (1.3.6.1.4.1.25506.2.105.1.4.1.1) | not-accessible | No | As per MIB |
| hh3cVsiPwBindAttributes (1.3.6.1.4.1.25506.2.105.1.4.1.2) | read-create | Current | As per MIB |
| hh3cVsiPwBindRowStatus (1.3.6.1.4.1.25506.2.105.1.4.1.3) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |