# Contents

IF-MIB	2

Scalar objects	2

ifTable	2

ifXTable	3

ifStackTable	5

ifTestTable	5

ifRcvAddressTable	5

IF-MIB

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifNumber(1.3.6.1.2.1.2.1) | read-only | No | As per MIB |
| ifTableLastChange (1.3.6.1.2.1.31.1.5) | read-only | No | Not supported |
| ifStackLastChange (1.3.6.1.2.1.31.1.6) | read-only | No | Not supported |

ifTable

OID of this table is: 1.3.6.1.2.1.2.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifIndex (1.3.6.1.2.1.2.2.1.1) | read-only | Current | As per MIB |
| ifDescr (1.3.6.1.2.1.2.2.1.2) | read-only | No | A textual string containing information about the interface. Such as null0, Vlan-interface2, Gigabitethernet1/0/1 |
| ifType (1.3.6.1.2.1.2.2.1.3) | read-only | Current | As per MIB |
| ifMtu (1.3.6.1.2.1.2.2.1.4) | read-only | Current | The size of the largest datagram which can be sent/received by interface in octets. If this port is stack port, the value is invalid. |
| ifSpeed (1.3.6.1.2.1.2.2.1.5) | read-only | Current | An estimate of the interface's current bandwidth in bits per second. |
| ifPhysAddress (1.3.6.1.2.1.2.2.1.6) | read-only | Current | MAC address For switches, the following type of interfaces support mac address: M-Ethernet(Management Ethernet) port Vlan interface Ethernet port GigabitEthernet port XGigabitEthernet port Forty-GigabitEthernet port Hundred-GigabitEthernet port Other type of interfaces don’t support mac address |
| ifAdminStatus (1.3.6.1.2.1.2.2.1.7) | read- write | Current | enable or disable an interface, ”testing” is not supported |
| ifOperStatus (1.3.6.1.2.1.2.2.1.8) | read-only | Current | The current operational state (link status) of the interface. |
| ifLastChange (1.3.6.1.2.1.2.2.1.9) | read-only | No | The value of sysUpTime at the time the interface entered its current operational state. If the current state was entered prior to the last re-initialization of the local network management subsystem, then this object contains a zero value. |
| ifInOctets (1.3.6.1.2.1.2.2.1.10) | read-only | No | As per MIB |
| ifInUcastPkts (1.3.6.1.2.1.2.2.1.11) | read-only | No | As per MIB |
| ifInNUcastPkts (1.3.6.1.2.1.2.2.1.12) | read-only | No | As per MIB |
| ifInDiscards (1.3.6.1.2.1.2.2.1.13) | read-only | No | Now not supported by swtich products. |
| ifInErrors (1.3.6.1.2.1.2.2.1.14) | read-only | No | As per MIB |
| ifInUnknownProtos (1.3.6.1.2.1.2.2.1.15) | read-only | No | As per MIB |
| ifOutOctets (1.3.6.1.2.1.2.2.1.16) | read-only | No | As per MIB |
| ifOutUcastPkts (1.3.6.1.2.1.2.2.1.17) | read-only | No | As per MIB |
| ifOutNUcastPkts (1.3.6.1.2.1.2.2.1.18) | read-only | No | As per MIB |
| ifOutDiscards (1.3.6.1.2.1.2.2.1.19) | read-only | No | As per MIB |
| ifOutErrors (1.3.6.1.2.1.2.2.1.20) | read-only | No | As per MIB |
| ifOutQLen (1.3.6.1.2.1.2.2.1.21) | read-only | No | As per MIB |
| ifSpecific (1.3.6.1.2.1.2.2.1.22) | read-only | No | Not supported |

ifXTable

OID of this table is: 1.3.6.1.2.1.31.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifName (1.3.6.1.2.1.31.1.1.1.1) | read-only | No | interface name, same as ifDescr in ifTable |
| ifInMulticastPkts (1.3.6.1.2.1.31.1.1.1.2) | read-only | No | As per MIB |
| ifInBroadcastPkts (1.3.6.1.2.1.31.1.1.1.3) | read-only | No | As per MIB |
| ifOutMulticastPkts (1.3.6.1.2.1.31.1.1.1.4) | read-only | No | As per MIB |
| ifOutBroadcastPkts (1.3.6.1.2.1.31.1.1.1.5) | read-only | No | As per MIB |
| ifHCInOctets (1.3.6.1.2.1.31.1.1.1.6) | read-only | No | As per MIB |
| ifHCInUcastPkts (1.3.6.1.2.1.31.1.1.1.7) | read-only | No | As per MIB |
| ifHCInMulticastPkts (1.3.6.1.2.1.31.1.1.1.8) | read-only | No | As per MIB |
| ifHCInBroadcastPkts (1.3.6.1.2.1.31.1.1.1.9) | read-only | No | As per MIB |
| ifHCOutOctets (1.3.6.1.2.1.31.1.1.1.10) | read-only | No | As per MIB |
| ifHCOutUcastPkts (1.3.6.1.2.1.31.1.1.1.11) | read-only | No | As per MIB |
| ifHCOutMulticastPkts (1.3.6.1.2.1.31.1.1.1.12) | read-only | No | As per MIB |
| ifHCOutBroadcastPkts (1.3.6.1.2.1.31.1.1.1.13) | read-only | No | As per MIB |
| ifLinkUpDownTrapEnable (1.3.6.1.2.1.31.1.1.1.14) | read-write | Current | As per MIB |
| ifHighSpeed (1.3.6.1.2.1.31.1.1.1.15) | read-only | No | As per MIB |
| ifPromiscuousMode (1.3.6.1.2.1.31.1.1.1.16) | read-write | No | It is only available for Ethernet (GigabitEthernet, Ten-GigabitEthernet, Forty-GigabitEthernet, Hundred-GigabitEthernet) interface working at route mode. For others, only read is supported and the value is always false(2), which means not supported. |
| ifConnectorPresent (1.3.6.1.2.1.31.1.1.1.17) | read-only | No | As per MIB |
| ifAlias (1.3.6.1.2.1.31.1.1.1.18) | read-write | Current | Spaces at the beginning or at the end of the ifAlias are not supported and will be removed. A string fully composed with spaces is not supported either. A null string inputted as ifAlias will be replaced with the whole name of the interface which is the default value of the ifAlias. The max length of ifAlias mib supports is 64, while the commandline supports 255. So if the length of ifAlias is larger than 64, only the former 64 letters are legal for MIB. Default for each interface is “xxxx Interface”. xxxx is interface name. |
| ifCounterDiscontinuityTime (1.3.6.1.2.1.31.1.1.1.19) | read-only | No | As per MIB |

ifStackTable

OID of this table is: 1.3.6.1.2.1.31.1.2

This table is not supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifStackHigherLayer (1.3.6.1.2.1.31.1.2.1.1) | not-accessible | No | As per MIB |
| ifStackLowerLayer (1.3.6.1.2.1.31.1.2.1.2) | not-accessible | No | As per MIB |
| ifStackStatus (1.3.6.1.2.1.31.1.2.1.3) | read-create | No | As per MIB |

ifTestTable

OID of this table is: 1.3.6.1.2.1.31.1.3

This table is not supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifTestId (1.3.6.1.2.1.31.1.3.1.1) | read-write | No | As per MIB |
| ifTestStatus (1.3.6.1.2.1.31.1.3.1.2) | read-write | No | As per MIB |
| ifTestType (1.3.6.1.2.1.31.1.3.1.3) | read-write | No | Only supported noTest, testFullDuplexLoopBack |
| ifTestResult (1.3.6.1.2.1.31.1.3.1.4) | read-only | No | As per MIB |
| ifTestCode (1.3.6.1.2.1.31.1.3.1.5) | read-only | No | The value is always testCodeUnknown(0.0) |
| ifTestOwner (1.3.6.1.2.1.31.1.3.1.6) | read-write | No | As per MIB |

Note for ifTestTable:

At the same time, only one test can be in progress, the second test is rejected.

For distributed device, the result is holding on master board. If the device reboots, the test result is lost.

The objects ifTestStatus, ifTestId, and ifTestOwner must be set together when ownership of the ifTestEntry is obtained successfully.

ifRcvAddressTable

OID of this table is: 1.3.6.1.2.1.31.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifRcvAddressAddress (1.3.6.1.2.1.31.1.4.1.1) | not-accessible | No | Only supported read operation. |
| ifRcvAddressStatus (1.3.6.1.2.1.31.1.4.1.2) | read-create | No | Only supported read operation. |
| ifRcvAddressType (1.3.6.1.2.1.31.1.4.1.3) | read-create | No | Only supported read operation. |