# Contents

IGMP-STD-MIB	2

igmpInterfaceTable	2

igmpCacheTable	3

IGMP-STD-MIB

The IGMP-STD-MIB contains two tables:

(1)  the IGMP Interface Table which contains one row for each interface on which IGMP is enabled.

(2)  the IGMP Cache Table which contains one row for each IP multicast group for which there are members on a particular interface.

Both tables are intended to be implemented by hosts and routers, but some columnar objects in each table apply only to routers.

igmpInterfaceTable

OID of this table is: 1.3.6.1.2.1.85.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| igmpInterfaceIfIndex (1.3.6.1.2.1.85.1.1.1.1) | not-accessible | Current | As per MIB |
| igmpInterfaceQueryInterval (1.3.6.1.2.1.85.1.1.1.2) | read-create | Current | Range from 1 to 31744 |
| igmpInterfaceStatus (1.3.6.1.2.1.85.1.1.1.3) | read-create | Current | Only support active(1), createAndGo(4), destroy(6) |
| igmpInterfaceVersion (1.3.6.1.2.1.85.1.1.1.4) | read-create | Current | Range from 1 to 2 |
| igmpInterfaceQuerier (1.3.6.1.2.1.85.1.1.1.5) | read-only | No | As per MIB |
| igmpInterfaceQueryMaxResponseTime (1.3.6.1.2.1.85.1.1.1.6) | read-create | Current | Range from 10 to 250 |
| igmpInterfaceQuerierUpTime (1.3.6.1.2.1.85.1.1.1.7) | read-only | No | As per MIB |
| igmpInterfaceQuerierExpiryTime (1.3.6.1.2.1.85.1.1.1.8) | read-only | No | As per MIB |
| igmpInterfaceVersion1QuerierTimer (1.3.6.1.2.1.85.1.1.1.9) | read-only | No | Not supported |
| igmpInterfaceWrongVersionQueries (1.3.6.1.2.1.85.1.1.1.10) | read-only | No | Not supported |
| igmpInterfaceJoins (1.3.6.1.2.1.85.1.1.1.11) | read-only | No | As per MIB |
| igmpInterfaceProxyIfIndex (1.3.6.1.2.1.85.1.1.1.12) | read-create | Current | Only support read operation |
| igmpInterfaceGroups (1.3.6.1.2.1.85.1.1.1.13) | read-only | No | As per MIB |
| igmpInterfaceRobustness (1.3.6.1.2.1.85.1.1.1.14) | read-create | Current | As per MIB |
| igmpInterfaceLastMembQueryIntvl (1.3.6.1.2.1.85.1.1.1.15) | read-create | Current | Range from 10 to 250 |

igmpCacheTable

OID of this table is: 1.3.6.1.2.1.85.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| igmpCacheAddress (1.3.6.1.2.1.85.1.2.1.1) | not-accessible | No | As per MIB |
| igmpCacheIfIndex (1.3.6.1.2.1.85.1.2.1.2) | not-accessible | No | As per MIB |
| igmpCacheSelf (1.3.6.1.2.1.85.1.2.1.3) | read-create | No | Only support read operation, and the default value is false(2) |
| igmpCacheLastReporter (1.3.6.1.2.1.85.1.2.1.4) | read-only | No | As per MIB |
| igmpCacheUpTime (1.3.6.1.2.1.85.1.2.1.5) | read-only | No | As per MIB |
| igmpCacheExpiryTime (1.3.6.1.2.1.85.1.2.1.6) | read-only | No | As per MIB |
| igmpCacheStatus (1.3.6.1.2.1.85.1.2.1.7) | read-create | No | Only support read operation |
| igmpCacheVersion1HostTimer (1.3.6.1.2.1.85.1.2.1.8) | read-only | No | As per MIB |