# Contents

IPV6-MIB	2

Scalar objects	2

ipv6IfTable	2

ipv6IfStatsTable	3

ipv6AddrPrefixTable	4

ipv6AddrTable	4

ipv6RouteTable	5

ipv6NetToMediaTable	5

IPV6-MIB

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6Forwarding (1.3.6.1.2.1.55.1.1) | read-write | Current | Only support read operation |
| ipv6DefaultHopLimit (1.3.6.1.2.1.55.1.2) | read-write | Current | Only support read operation |
| ipv6Interfaces (1.3.6.1.2.1.55.1.3) | read-only | Current | As per MIB |
| ipv6IfTableLastChange (1.3.6.1.2.1.55.1.4) | read-only | No | Not supported, the value is always 0. |
| ipv6RouteNumber (1.3.6.1.2.1.55.1.9) | read-only | No | As per MIB |
| ipv6DiscardedRoutes (1.3.6.1.2.1.55.1.10) | read-only | No | As per MIB |

ipv6IfTable

OID of this table is: 1.3.6.1.2.1.55.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6IfIndex (1.3.6.1.2.1.55.1.5.1.1) | not-accessible | No | As per MIB |
| ipv6IfDescr (1.3.6.1.2.1.55.1.5.1.2) | read-write | No | The string is no more than 255 characters. This variable is the same as ifAlias in ifXTable. |
| ipv6IfLowerLayer (1.3.6.1.2.1.55.1.5.1.3) | read-only | No | Not supported |
| ipv6IfEffectiveMtu (1.3.6.1.2.1.55.1.5.1.4) | read-only | Current | As per MIB |
| ipv6IfReasmMaxSize (1.3.6.1.2.1.55.1.5.1.5) | read-only | No | Not supported, the value is always 65535. |
| ipv6IfIdentifier (1.3.6.1.2.1.55.1.5.1.6) | read-write | No | Only support read operation |
| ipv6IfIdentifierLength (1.3.6.1.2.1.55.1.5.1.7) | read-write | No | Only support read operation |
| ipv6IfPhysicalAddress (1.3.6.1.2.1.55.1.5.1.8) | read-only | Current | As per MIB |
| ipv6IfAdminStatus (1.3.6.1.2.1.55.1.5.1.9) | read-write | Current | Only support read operation |
| ipv6IfOperStatus (1.3.6.1.2.1.55.1.5.1.10) | read-only | No | As per MIB |
| ipv6IfLastChange (1.3.6.1.2.1.55.1.5.1.11) | read-only | No | As per MIB |

ipv6IfStatsTable

OID of this table is: 1.3.6.1.2.1.55.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6IfStatsInReceives (1.3.6.1.2.1.55.1.6.1.1) | read-only | No | As per MIB |
| ipv6IfStatsInHdrErrors (1.3.6.1.2.1.55.1.6.1.2) | read-only | No | As per MIB |
| ipv6IfStatsInTooBigErrors (1.3.6.1.2.1.55.1.6.1.3) | read-only | No | As per MIB |
| ipv6IfStatsInNoRoutes (1.3.6.1.2.1.55.1.6.1.4) | read-only | No | As per MIB |
| ipv6IfStatsInAddrErrors (1.3.6.1.2.1.55.1.6.1.5) | read-only | No | As per MIB |
| ipv6IfStatsInUnknownProtos (1.3.6.1.2.1.55.1.6.1.6) | read-only | No | As per MIB |
| ipv6IfStatsInTruncatedPkts (1.3.6.1.2.1.55.1.6.1.7) | read-only | No | As per MIB |
| ipv6IfStatsInDiscards (1.3.6.1.2.1.55.1.6.1.8) | read-only | No | As per MIB |
| ipv6IfStatsInDelivers (1.3.6.1.2.1.55.1.6.1.9) | read-only | No | As per MIB |
| ipv6IfStatsOutForwDatagrams (1.3.6.1.2.1.55.1.6.1.10) | read-only | No | As per MIB |
| ipv6IfStatsOutRequests (1.3.6.1.2.1.55.1.6.1.11) | read-only | No | As per MIB |
| ipv6IfStatsOutDiscards (1.3.6.1.2.1.55.1.6.1.12) | read-only | No | As per MIB |
| ipv6IfStatsOutFragOKs (1.3.6.1.2.1.55.1.6.1.13) | read-only | No | As per MIB |
| ipv6IfStatsOutFragFails (1.3.6.1.2.1.55.1.6.1.14) | read-only | No | As per MIB |
| ipv6IfStatsOutFragCreates (1.3.6.1.2.1.55.1.6.1.15) | read-only | No | As per MIB |
| ipv6IfStatsReasmReqds (1.3.6.1.2.1.55.1.6.1.16) | read-only | No | As per MIB |
| ipv6IfStatsReasmOKs (1.3.6.1.2.1.55.1.6.1.17) | read-only | No | As per MIB |
| ipv6IfStatsReasmFails (1.3.6.1.2.1.55.1.6.1.18) | read-only | No | As per MIB |
| ipv6IfStatsInMcastPkts (1.3.6.1.2.1.55.1.6.1.19) | read-only | No | As per MIB |
| ipv6IfStatsOutMcastPkts (1.3.6.1.2.1.55.1.6.1.20) | read-only | No | As per MIB |

ipv6AddrPrefixTable

OID of this table is: 1.3.6.1.2.1.55.1.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6AddrPrefix (1.3.6.1.2.1.55.1.7.1.1) | not-accessible | Current | As per MIB |
| ipv6AddrPrefixLength (1.3.6.1.2.1.55.1.7.1.2) | not-accessible | Current | As per MIB |
| ipv6AddrPrefixOnLinkFlag (1.3.6.1.2.1.55.1.7.1.3) | read-only | No | As per MIB |
| ipv6AddrPrefixAutonomousFlag (1.3.6.1.2.1.55.1.7.1.4) | read-only | No | As per MIB |
| ipv6AddrPrefixAdvPreferredLifetime (1.3.6.1.2.1.55.1.7.1.5) | read-only | No | As per MIB |
| ipv6AddrPrefixAdvValidLifetime (1.3.6.1.2.1.55.1.7.1.6) | read-only | No | As per MIB |

ipv6AddrTable

OID of this table is: 1.3.6.1.2.1.55.1.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6AddrAddress (1.3.6.1.2.1.55.1.8.1.1) | not-accessible | Current | As per MIB |
| ipv6AddrPfxLength (1.3.6.1.2.1.55.1.8.1.2) | read-only | Current | As per MIB |
| ipv6AddrType (1.3.6.1.2.1.55.1.8.1.3) | read-only | Current | Unknow is not supported |
| ipv6AddrAnycastFlag (1.3.6.1.2.1.55.1.8.1.4) | read-only | Current | As per MIB |
| ipv6AddrStatus (1.3.6.1.2.1.55.1.8.1.5) | read-only | Current | As per MIB |

ipv6RouteTable

OID of this table is: 1.3.6.1.2.1.55.1.11

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6RouteDest (1.3.6.1.2.1.55.1.11.1.1) | not-accessible | No | As per MIB |
| ipv6RoutePfxLength (1.3.6.1.2.1.55.1.11.1.2) | not-accessible | No | As per MIB |
| ipv6RouteIndex (1.3.6.1.2.1.55.1.11.1.3) | not-accessible | No | As per MIB |
| ipv6RouteIfIndex (1.3.6.1.2.1.55.1.11.1.4) | read-only | No | As per MIB |
| ipv6RouteNextHop (1.3.6.1.2.1.55.1.11.1.5) | read-only | No | As per MIB |
| ipv6RouteType (1.3.6.1.2.1.55.1.11.1.6) | read-only | No | As per MIB |
| ipv6RouteProtocol (1.3.6.1.2.1.55.1.11.1.7) | read-only | No | As per MIB |
| ipv6RoutePolicy (1.3.6.1.2.1.55.1.11.1.8) | read-only | No | As per MIB |
| ipv6RouteAge (1.3.6.1.2.1.55.1.11.1.9) | read-only | No | As per MIB |
| ipv6RouteNextHopRDI (1.3.6.1.2.1.55.1.11.1.10) | read-only | No | As per MIB |
| ipv6RouteMetric (1.3.6.1.2.1.55.1.11.1.11) | read-only | No | As per MIB |
| ipv6RouteWeight (1.3.6.1.2.1.55.1.11.1.12) | read-only | No | As per MIB |
| ipv6RouteInfo (1.3.6.1.2.1.55.1.11.1.13) | read-only | No | As per MIB |
| ipv6RouteValid (1.3.6.1.2.1.55.1.11.1.14) | read-write | No | Only support read operation |

ipv6NetToMediaTable

OID of this table is: 1.3.6.1.2.1.55.1.12

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6NetToMediaNetAddress (1.3.6.1.2.1.55.1.12.1.1) | not-accessible | No | As per MIB |
| ipv6NetToMediaPhysAddress (1.3.6.1.2.1.55.1.12.1.2) | read-only | No | As per MIB |
| ipv6NetToMediaType (1.3.6.1.2.1.55.1.12.1.3) | read-only | No | As per MIB |
| ipv6IfNetToMediaState (1.3.6.1.2.1.55.1.12.1.4) | read-only | No | As per MIB |
| ipv6IfNetToMediaLastUpdated (1.3.6.1.2.1.55.1.12.1.5) | read-only | No | As per MIB |
| ipv6NetToMediaValid (1.3.6.1.2.1.55.1.12.1.6) | read-write | No | Only support read operation |