# Contents

ISIS-MIB	3

isisSysObject	3

isisManAreaAddrTable	4

isisAreaAddrTable	4

isisSummAddrTable	4

isisRedistributeAddrTable	4

isisRouterTable	5

isisSysLevelTable	5

isisCirc	6

isisCircTable	6

isisCircLevelTable	7

isisSystemCounterTable	8

isisCircuitCounterTable	8

isisPacketCounterTable	9

isisISAdjTable	9

isisISAdjAreaAddrTable	10

isisISAdjIPAddrTable	10

isisISAdjProtSuppTable	10

isisRATable	11

isisIPRATable	11

isisLSPSummaryTable	12

isisLSPTLVTable	12

isisNotificationEntry	13

ISIS-MIB (for SPBM)	14

isisSysObject	14

isisAreaAddrTable	14

isisRouterTable	15

isisSysLevelTable	15

isisCirc	16

isisCircTable	16

isisCircLevelTable	17

isisSystemCounterTable	18

isisCircuitCounterTable	18

isisPacketCounterTable	19

isisISAdjTable	19

isisRATable	20

isisLSPSummaryTable	21

isisLSPTLVTable	21

isisNotificationEntry	21

ISIS-MIB (for TRILL)	22

Scalar objects of isisSysObject	22

isisRouterTable	23

isisSysLevelTable	23

Scalar objects of isisCirc	24

isisCircTable	24

isisCircLevelTable	25

isisSystemCounterTable	26

isisCircuitCounterTable	26

isisPacketCounterTable	27

isisISAdjTable	27

isisLSPSummaryTable	28

isisLSPTLVTable	29

IsisNotification	29

ISIS-MIB

isisSysObject

OID of this table is: 1.3.6.1.2.1.138.1.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysVersion (1.3.6.1.2.1.138.1.1.1.1) | read-only | No | The value is always one(1) |
| isisSysLevelType (1.3.6.1.2.1.138.1.1.1.2) | read-write | No | Only support read operation |
| isisSysID (1.3.6.1.2.1.138.1.1.1.3) | read-write | No | Only support read operation |
| isisSysMaxPathSplits (1.3.6.1.2.1.138.1.1.1.4) | read-write | Current | The range and the default value are product specific This object can be modified whatever the value of isisSysAdminState is on(1) or not |
| isisSysMaxLSPGenInt (1.3.6.1.2.1.138.1.1.1.5) | read-write | Current | The value should be less than isisSysMaxAge The value must be greater than any value configured for isisSysLevelMinLSPGenInt is not implemented The value should be at least 300 seconds less than isisSysMaxAge is not implemented |
| isisSysPollESHelloRate (1.3.6.1.2.1.138.1.1.1.6) | read-write | No | Only support read operation The value is always 50 |
| isisSysWaitTime (1.3.6.1.2.1.138.1.1.1.7) | read-write | No | Only support read operation The value is always 60 |
| isisSysAdminState (1.3.6.1.2.1.138.1.1.1.8) | read-write | No | Only support read operation |
| isisSysL2toL1Leaking (1.3.6.1.2.1.138.1.1.1.9) | read-write | Current | As per MIB |
| isisSysMaxAge (1.3.6.1.2.1.138.1.1.1.10) | read-write | Current | The value should be greater than isisSysMaxLSPGenInt, but it should be at least 300 seconds greater than isisSysMaxLSPGenInt is not implemented |
| isisSysReceiveLSPBufferSize(1.3.6.1.2.1.138.1.1.1.11) | read-write | Current | The default value is 1497 |
| isisSysProtSupported (1.3.6.1.2.1.138.1.1.1.12) | read-only | No | Only support ipv4 (1), ipv6 (2) When supporting IPv4 and IPv6, return the value of ipv6(2) |
| isisSysNotificationEnable (1.3.6.1.2.1.138.1.1.1.13) | read-write | Current | As per MIB |

isisManAreaAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisManAreaAddr (1.3.6.1.2.1.138.1.1.2.1.1) | not-accessible | No | AS per MIB |
| isisManAreaAddrExistState （1.3.6.1.2.1.138.1.1.2.1.2） | read-create | No | Only support read operation Only support active(1) |

isisAreaAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisAreaAddr （1.3.6.1.2.1.138.1.1.3.1.1） | read-only | No | Only can return value when the value of isisSysLevelType is level1and2(3) |

isisSummAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSummAddressType (1.3.6.1.2.1.138.1.1.4.1.1) | not-accessible | No | As per MIB |
| isisSummAddress (1.3.6.1.2.1.138.1.1.4.1.2) | not-accessible | No | As per MIB |
| isisSummAddrPrefixLen (1.3.6.1.2.1.138.1.1.4.1.3) | not-accessible | No | As per MIB |
| isisSummAddrExistState (1.3.6.1.2.1.138.1.1.4.1.4) | read-create | No | Only support read operation Only support active(1) |
| isisSummAddrMetric (1.3.6.1.2.1.138.1.1.4.1.5) | read-create | No | Only support read operation The value is always 20 |
| isisSummAddrFullMetric (1.3.6.1.2.1.138.1.1.4.1.6) | read-create | No | Only support read operation The value is always 20 |

isisRedistributeAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisRedistributeAddrType (1.3.6.1.2.1.138.1.1.5.1.1) | not-accessible | No | As per MIB |
| isisRedistributeAddrAddress (1.3.6.1.2.1.138.1.1.5.1.2) | not-accessible | No | As per MIB |
| isisRedistributeAddrPrefixLen (1.3.6.1.2.1.138.1.1.5.1.3) | not-accessible | No | As per MIB |
| isisRedistributeAddrExistState (1.3.6.1.2.1.138.1.1.5.1.4) | read-create | No | Only support read operation Only support active(1) |

isisRouterTable

OID of this table is: 1.3.6.1.2.1.138.1.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisRouterSysID (1.3.6.1.2.1.138.1.1.6.1.1) | not-accessible | No | As per MIB |
| isisRouterLevel (1.3.6.1.2.1.138.1.1.6.1.2) | not-accessible | No | As per MIB |
| isisRouterHostName (1.3.6.1.2.1.138.1.1.6.1.3) | read-only | No | As per MIB |
| isisRouterID (1.3.6.1.2.1.138.1.1.6.1.4) | read-only | No | As per MIB |

isisSysLevelTable

OID of this table is: 1.3.6.1.2.1.138.1.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysLevelIndex (1.3.6.1.2.1.138.1.2.1.1.1) | not-accessible | No | As per MIB |
| isisSysLevelOrigLSPBuffSize (1.3.6.1.2.1.138.1.2.1.1.2) | read-write | Current | The default value is 1497. The value should be less than isisSysReceiveLSPBufferSize. This object can be modified whatever the value of isisSysAdminState is on(1) or not |
| isisSysLevelMinLSPGenInt (1.3.6.1.2.1.138.1.2.1.1.3) | read-write | Current | Range from 1 to 120 The default value is 1. As configure intelligent timer,return 1 when the value is less than 1 and otherwise return integer part |
| isisSysLevelState (1.3.6.1.2.1.138.1.2.1.1.4) | read-only | No | Only support off(1), on(2) and overloaded(4) |
| isisSysLevelSetOverload (1.3.6.1.2.1.138.1.2.1.1.5) | read-write | Current | As per MIB |
| isisSysLevelSetOverloadUntil (1.3.6.1.2.1.138.1.2.1.1.6) | read-write | Current | If this object is non-zero, the overload bit is set at this level when the isisSysAdminState variable goes to state 'on' for this Intermediate System is not implemented |
| isisSysLevelMetricStyle (1.3.6.1.2.1.138.1.2.1.1.7) | read-write | Current | As per MIB |
| isisSysLevelSPFConsiders (1.3.6.1.2.1.138.1.2.1.1.8) | read-write | No | Only support read operation |
| isisSysLevelTEEnabled (1.3.6.1.2.1.138.1.2.1.1.9) | read-write | No | As per MIB |

isisCirc

OID of this table is: 1.3.6.1.2.1.138.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisNextCircIndex (1.3.6.1.2.1.138.1.3.1) | read-only | No | As per MIB |

isisCircTable

OID of this table is: 1.3.6.1.2.1.138.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircIndex (1.3.6.1.2.1.138.1.3.2.1.1) | not-accessible | No | As per MIB |
| isisCircIfIndex (1.3.6.1.2.1.138.1.3.2.1.2) | read-create | No | Only support read operation |
| isisCircAdminState (1.3.6.1.2.1.138.1.3.2.1.3) | read-create | No | Only support read operation |
| isisCircExistState (1.3.6.1.2.1.138.1.3.2.1.4) | read-create | No | Only support read operation Only support active(1) |
| isisCircType (1.3.6.1.2.1.138.1.3.2.1.5) | read-create | No | Only support read operation Only support broadcast(1) and ptToPt(2) The value is broadcast(1) on broadcast circuit, and is ptToPt(2) on other circuit types |
| isisCircExtDomain (1.3.6.1.2.1.138.1.3.2.1.6) | read-create | No | Only support read operation The value is always false（2） |
| isisCircLevelType (1.3.6.1.2.1.138.1.3.2.1.7) | read-create | Current | Only support read-write operation |
| isisCircPassiveCircuit (1.3.6.1.2.1.138.1.3.2.1.8) | read-create | Current | Only support read-write operation |
| isisCircMeshGroupEnabled (1.3.6.1.2.1.138.1.3.2.1.9) | read-create | No | Only support read operation |
| isisCircMeshGroup (1.3.6.1.2.1.138.1.3.2.1.10) | read-create | No | Only support read operation |
| isisCircSmallHellos (1.3.6.1.2.1.138.1.3.2.1.11) | read-create | Current | Only support read-write operation |
| isisCircLastUpTime (1.3.6.1.2.1.138.1.3.2.1.12) | read-only | No | As per MIB |
| isisCirc3WayEnabled (1.3.6.1.2.1.138.1.3.2.1.13) | read-create | No | Only support read operation. |
| isisCircExtendedCircID (1.3.6.1.2.1.138.1.3.2.1.14) | read-create | No | Only support read operation. |

isisCircLevelTable

OID of this table is: 1.3.6.1.2.1.138.1.4.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircLevelIndex (1.3.6.1.2.1.138.1.4.1.1.1) | not-accessible | No | As per MIB |
| isisCircLevelMetric (1.3.6.1.2.1.138.1.4.1.1.2) | read-write | Current | Range from 1 to 63 |
| isisCircLevelWideMetric (1.3.6.1.2.1.138.1.4.1.1.3) | read-write | Current | Range from 1 to 16777215 |
| isisCircLevelISPriority (1.3.6.1.2.1.138.1.4.1.1.4) | read-write | No | As per MIB |
| isisCircLevelIDOctet (1.3.6.1.2.1.138.1.4.1.1.5) | read-only | No | As per MIB |
| isisCircLevelID (1.3.6.1.2.1.138.1.4.1.1.6) | read-only | No | The value is isisSysID with one-byte isisCircLevelIDOctet for point to point circuit, and zero length octet string for broadcast circuit |
| isisCircLevelDesIS (1.3.6.1.2.1.138.1.4.1.1.7) | read-only | No | As per MIB |
| isisCircLevelHelloMultiplier (1.3.6.1.2.1.138.1.4.1.1.8) | read-write | Current | Range from 3 to 100 The default value is 3 |
| isisCircLevelHelloTimer (1.3.6.1.2.1.138.1.4.1.1.9) | read-write | Current | Range from 3000 to 255000 The default value is 10000 The residue of the set value divided by 1000 is ignored |
| isisCircLevelDRHelloTimer (1.3.6.1.2.1.138.1.4.1.1.10) | read-write | No | Only support read operation Range from 1000 to 85000 The default value is 3000 The residue of the set value moded by 1000 is ignored |
| isisCircLevelLSPThrottle (1.3.6.1.2.1.138.1.4.1.1.11) | read-write | Current | Range from 1 to 1000 The default value is 33 |
| isisCircLevelMinLSPRetransInt (1.3.6.1.2.1.138.1.4.1.1.12) | read-write | Current | As per MIB |
| isisCircLevelCSNPInterval (1.3.6.1.2.1.138.1.4.1.1.13) | read-write | Current | As per MIB |
| isisCircLevelPartSNPInterval (1.3.6.1.2.1.138.1.4.1.1.14) | read-write | No | Only support read operation The value is always 2 |

isisSystemCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysStatLevel (1.3.6.1.2.1.138.1.5.1.1.1) | not-accessible | No | As per MIB |
| isisSysStatCorrLSPs (1.3.6.1.2.1.138.1.5.1.1.2) | read-only | No | The value is always 0 |
| isisSysStatAuthTypeFails (1.3.6.1.2.1.138.1.5.1.1.3) | read-only | No | As per MIB |
| isisSysStatAuthFails (1.3.6.1.2.1.138.1.5.1.1.4) | read-only | No | As per MIB |
| isisSysStatLSPDbaseOloads (1.3.6.1.2.1.138.1.5.1.1.5) | read-only | No | As per MIB |
| isisSysStatManAddrDropFromAreas (1.3.6.1.2.1.138.1.5.1.1.6) | read-only | No | As per MIB |
| isisSysStatAttmptToExMaxSeqNums (1.3.6.1.2.1.138.1.5.1.1.7) | read-only | No | As per MIB |
| isisSysStatSeqNumSkips (1.3.6.1.2.1.138.1.5.1.1.8) | read-only | No | As per MIB |
| isisSysStatOwnLSPPurges (1.3.6.1.2.1.138.1.5.1.1.9) | read-only | No | As per MIB |
| isisSysStatIDFieldLenMismatches (1.3.6.1.2.1.138.1.5.1.1.10) | read-only | No | As per MIB |
| isisSysStatPartChanges (1.3.6.1.2.1.138.1.5.1.1.11) | read-only | No | The value is always 0 |
| isisSysStatSPFRuns (1.3.6.1.2.1.138.1.5.1.1.12) | read-only | No | As per MIB |
| isisSysStatLSPErrors (1.3.6.1.2.1.138.1.5.1.1.13) | read-only | No | As per MIB |

isisCircuitCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircuitType (1.3.6.1.2.1.138.1.5.2.1.1) | not-accessible | No | As per MIB |
| isisCircAdjChanges (1.3.6.1.2.1.138.1.5.2.1.2) | read-only | No | As per MIB |
| isisCircNumAdj (1.3.6.1.2.1.138.1.5.2.1.3) | read-only | No | As per MIB |
| isisCircInitFails (1.3.6.1.2.1.138.1.5.2.1.4) | read-only | No | The value is always 0 |
| isisCircRejAdjs (1.3.6.1.2.1.138.1.5.2.1.5) | read-only | No | As per MIB |
| isisCircIDFieldLenMismatches (1.3.6.1.2.1.138.1.5.2.1.6) | read-only | No | As per MIB |
| isisCircMaxAreaAddrMismatches (1.3.6.1.2.1.138.1.5.2.1.7) | read-only | No | As per MIB |
| isisCircAuthTypeFails (1.3.6.1.2.1.138.1.5.2.1.8) | read-only | No | As per MIB |
| isisCircAuthFails (1.3.6.1.2.1.138.1.5.2.1.9) | read-only | No | As per MIB |
| isisCircLANDesISChanges (1.3.6.1.2.1.138.1.5.2.1.10) | read-only | No | As per MIB |

isisPacketCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisPacketCountLevel (1.3.6.1.2.1.138.1.5.3.1.1) | not-accessible | No | As per MIB |
| isisPacketCountDirection (1.3.6.1.2.1.138.1.5.3.1.2) | not-accessible | No | As per MIB |
| isisPacketCountIIHello (1.3.6.1.2.1.138.1.5.3.1.3) | read-only | No | As per MIB |
| isisPacketCountISHello (1.3.6.1.2.1.138.1.5.3.1.4) | read-only | No | The value is always 0 |
| isisPacketCountESHello (1.3.6.1.2.1.138.1.5.3.1.5) | read-only | No | The value is always 0 |
| isisPacketCountLSP (1.3.6.1.2.1.138.1.5.3.1.6) | read-only | No | As per MIB |
| isisPacketCountCSNP (1.3.6.1.2.1.138.1.5.3.1.7) | read-only | No | As per MIB |
| isisPacketCountPSNP (1.3.6.1.2.1.138.1.5.3.1.8) | read-only | No | As per MIB |
| isisPacketCountUnknown (1.3.6.1.2.1.138.1.5.3.1.9) | read-only | No | As per MIB |

isisISAdjTable

OID of this table is: 1.3.6.1.2.1.138.1.6.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisISAdjIndex (1.3.6.1.2.1.138.1.6.1.1.1) | not-accessible | No | As per MIB |
| isisISAdjState (1.3.6.1.2.1.138.1.6.1.1.2) | read-only | No | Only support down(1), initializing(2) and up(3) |
| isisISAdj3WayState (1.3.6.1.2.1.138.1.6.1.1.3) | read-only | No | As per MIB |
| isisISAdjNeighSNPAAddress (1.3.6.1.2.1.138.1.6.1.1.4) | read-only | No | As per MIB |
| isisISAdjNeighSysType (1.3.6.1.2.1.138.1.6.1.1.5) | read-only | No | Only support l1IntermediateSystem(1), l2IntermediateSystem(2) and l1L2IntermediateSystem(3) |
| isisISAdjNeighSysID (1.3.6.1.2.1.138.1.6.1.1.6) | read-only | No | As per MIB |
| isisISAdjNbrExtendedCircID (1.3.6.1.2.1.138.1.6.1.1.7) | read-only | No | As per MIB |
| isisISAdjUsage (1.3.6.1.2.1.138.1.6.1.1.8) | read-only | No | As per MIB |
| isisISAdjHoldTimer (1.3.6.1.2.1.138.1.6.1.1.9) | read-only | No | As per MIB |
| isisISAdjNeighPriority (1.3.6.1.2.1.138.1.6.1.1.10) | read-only | No | As per MIB |
| isisISAdjLastUpTime (1.3.6.1.2.1.138.1.6.1.1.11) | read-only | No | As per MIB |

isisISAdjAreaAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.6.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisISAdjAreaAddrIndex (1.3.6.1.2.1.138.1.6.2.1.1) | not-accessible | No | As per MIB |
| isisISAdjAreaAddress (1.3.6.1.2.1.138.1.6.2.1.2) | read-only | No | As per MIB |

isisISAdjIPAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.6.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisISAdjIPAddrIndex (1.3.6.1.2.1.138.1.6.3.1.1) | not-accessible | No | As per MIB |
| isisISAdjIPAddrType (1.3.6.1.2.1.138.1.6.3.1.2) | read-only | No | Only support ipv4(1) and ipv6(2) Support ipv6(2) only when the local IS has ipv6 capability |
| isisISAdjIPAddrAddress (1.3.6.1.2.1.138.1.6.3.1.3) | read-only | No | As per MIB |

isisISAdjProtSuppTable

OID of this table is: 1.3.6.1.2.1.138.1.6.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisISAdjProtSuppProtocol (1.3.6.1.2.1.138.1.6.4.1.1) | read-only | No | As per MIB |

isisRATable

OID of this table is: 1.3.6.1.2.1.138.1.7.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisRAIndex (1.3.6.1.2.1.138.1.7.1.1.1) | not-accessible | No | As per MIB |
| isisRAExistState (1.3.6.1.2.1.138.1.7.1.1.2) | read-create | No | The value is always active(1) |
| isisRAAdminState (1.3.6.1.2.1.138.1.7.1.1.3) | read-create | No | The default value is on(1) |
| isisRAAddrPrefix (1.3.6.1.2.1.138.1.7.1.1.4) | read-create | No | As per MIB |
| isisRAMapType (1.3.6.1.2.1.138.1.7.1.1.5) | read-create | No | The default value is none(1) |
| isisRAMetric (1.3.6.1.2.1.138.1.7.1.1.6) | read-create | No | As per MIB |
| isisRAMetricType (1.3.6.1.2.1.138.1.7.1.1.7) | read-create | No | As per MIB |
| isisRASNPAAddress (1.3.6.1.2.1.138.1.7.1.1.8) | read-create | No | As per MIB |
| isisRASNPAMask (1.3.6.1.2.1.138.1.7.1.1.9) | read-create | No | As per MIB |
| isisRASNPAPrefix (1.3.6.1.2.1.138.1.7.1.1.10) | read-create | No | As per MIB |
| isisRAType (1.3.6.1.2.1.138.1.7.1.1.11) | read-create | No | As per MIB |

isisIPRATable

OID of this table is: 1.3.6.1.2.1.138.1.8.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisIPRADestType (1.3.6.1.2.1.138.1.8.1.1.1) | not-accessible | No | As per MIB |
| isisIPRADest (1.3.6.1.2.1.138.1.8.1.1.2) | not-accessible | No | As per MIB |
| isisIPRADestPrefixLen (1.3.6.1.2.1.138.1.8.1.1.3) | not-accessible | No | As per MIB |
| isisIPRANextHopIndex (1.3.6.1.2.1.138.1.8.1.1.4) | not-accessible | No | As per MIB |
| isisIPRANextHopType (1.3.6.1.2.1.138.1.8.1.1.5) | read-create | No | Only support read operation Only support ipv4(1) and ipv6(2) |
| isisIPRANextHop (1.3.6.1.2.1.138.1.8.1.1.6) | read-create | No | Only support read operation |
| isisIPRAType (1.3.6.1.2.1.138.1.8.1.1.7) | read-create | No | Only support read operation The value is always automatic(2) |
| isisIPRAExistState (1.3.6.1.2.1.138.1.8.1.1.8) | read-create | No | Only support read operation The value is always active(1) |
| isisIPRAAdminState (1.3.6.1.2.1.138.1.8.1.1.9) | read-create | No | Only support read operation The value is always on(1) |
| isisIPRAMetric (1.3.6.1.2.1.138.1.8.1.1.10) | read-create | No | Only support read operation The default value is 0 |
| isisIPRAMetricType (1.3.6.1.2.1.138.1.8.1.1.11) | read-create | No | Only support read operation |
| isisIPRAFullMetric (1.3.6.1.2.1.138.1.8.1.1.12) | read-create | No | Only support read operation The default value is 0 |
| isisIPRASNPAAddress (1.3.6.1.2.1.138.1.8.1.1.13) | read-create | No | Only support read operation The value is always zero length octet string |
| isisIPRASourceType (1.3.6.1.2.1.138.1.8.1.1.14) | read-only | No | Only support static(1), direct(2), ospfv2(3), ospfv3(4), isis(5), rip(6) and bgp(9) |

isisLSPSummaryTable

OID of this table is: 1.3.6.1.2.1.138.1.9.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisLSPLevel (1.3.6.1.2.1.138.1.9.1.1.1) | not-accessible | No | As per MIB |
| isisLSPID (1.3.6.1.2.1.138.1.9.1.1.2) | not-accessible | No | As per MIB |
| isisLSPSeq (1.3.6.1.2.1.138.1.9.1.1.3) | read-only | No | As per MIB |
| isisLSPZeroLife (1.3.6.1.2.1.138.1.9.1.1.4) | read-only | No | As per MIB |
| isisLSPChecksum (1.3.6.1.2.1.138.1.9.1.1.5) | read-only | No | As per MIB |
| isisLSPLifetimeRemain (1.3.6.1.2.1.138.1.9.1.1.6) | read-only | No | As per MIB |
| isisLSPPDULength (1.3.6.1.2.1.138.1.9.1.1.7) | read-only | No | As per MIB |
| isisLSPAttributes (1.3.6.1.2.1.138.1.9.1.1.8) | read-only | No | As per MIB |

isisLSPTLVTable

OID of this table is: 1.3.6.1.2.1.138.1.9.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisLSPTLVIndex (1.3.6.1.2.1.138.1.9.2.1.1) | not-accessible | No | As per MIB |
| isisLSPTLVSeq (1.3.6.1.2.1.138.1.9.2.1.2) | read-only | No | As per MIB |
| isisLSPTLVChecksum (1.3.6.1.2.1.138.1.9.2.1.3) | read-only | No | As per MIB |
| isisLSPTLVType (1.3.6.1.2.1.138.1.9.2.1.4) | read-only | No | As per MIB |
| isisLSPTLVLen (1.3.6.1.2.1.138.1.9.2.1.5) | read-only | No | As per MIB |
| isisLSPTLVValue (1.3.6.1.2.1.138.1.9.2.1.6) | read-only | No | As per MIB |

isisNotificationEntry

OID of this table is: 1.3.6.1.2.1.138.1.10.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisNotificationSysLevelIndex (1.3.6.1.2.1.138.1.10.1.1) | accessible-for-notify | No | As per MIB |
| isisNotificationCircIfIndex (1.3.6.1.2.1.138.1.10.1.2) | accessible-for-notify | No | As per MIB |
| isisPduLspId (1.3.6.1.2.1.138.1.10.1.3) | accessible-for-notify | No | As per MIB |
| isisPduFragment (1.3.6.1.2.1.138.1.10.1.4) | accessible-for-notify | No | As per MIB |
| isisPduFieldLen (1.3.6.1.2.1.138.1.10.1.5) | accessible-for-notify | No | As per MIB |
| isisPduMaxAreaAddress (1.3.6.1.2.1.138.1.10.1.6) | accessible-for-notify | No | As per MIB |
| isisPduProtocolVersion (1.3.6.1.2.1.138.1.10.1.7) | accessible-for-notify | No | As per MIB |
| isisPduLspSize (1.3.6.1.2.1.138.1.10.1.8) | accessible-for-notify | No | As per MIB |
| isisPduOriginatingBufferSize (1.3.6.1.2.1.138.1.10.1.9) | accessible-for-notify | No | As per MIB |
| isisPduBufferSize (1.3.6.1.2.1.138.1.10.1.10) | accessible-for-notify | No | As per MIB |
| isisPduProtocolsSupported (1.3.6.1.2.1.138.1.10.1.11) | accessible-for-notify | No | As per MIB |
| isisAdjState (1.3.6.1.2.1.138.1.10.1.12) | accessible-for-notify | No | As per MIB |
| isisErrorOffset (1.3.6.1.2.1.138.1.10.1.13) | accessible-for-notify | No | As per MIB |
| isisErrorTLVType (1.3.6.1.2.1.138.1.10.1.14) | accessible-for-notify | No | As per MIB |
| isisNotificationAreaAddress (1.3.6.1.2.1.138.1.10.1.15) | accessible-for-notify | No | As per MIB |

ISIS-MIB (for SPBM)

The SPBM ISIS-MIB is a re-use of ISIS-MIB to manage the information of Shortest Path Bridging MAC(SPBM) ISIS, which is same with rfc4444 ISIS-MIB.

isisSysObject

OID of this table is: 1.3.6.1.2.1.138.1.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysVersion (1.3.6.1.2.1.138.1.1.1.1) | read-only | No | The value is always one(1) |
| isisSysLevelType (1.3.6.1.2.1.138.1.1.1.2) | read-write | No | Only support read operation. The value is always level1 |
| isisSysID (1.3.6.1.2.1.138.1.1.1.3) | read-write | No | Only support read operation |
| isisSysMaxPathSplits (1.3.6.1.2.1.138.1.1.1.4) | read-write | Current | Only support read operation . The value is always 16 |
| isisSysMaxLSPGenInt (1.3.6.1.2.1.138.1.1.1.5) | read-write | Current | The value should be less than isisSysMaxAge. A value of 65235 will be returned when the actual value is greater than 65235 |
| isisSysPollESHelloRate (1.3.6.1.2.1.138.1.1.1.6) | read-write | No | Only support read operation The value is always 50 |
| isisSysWaitTime (1.3.6.1.2.1.138.1.1.1.7) | read-write | No | Only support read operation The value is always 60 |
| isisSysAdminState (1.3.6.1.2.1.138.1.1.1.8) | read-write | No | Only support read operation |
| isisSysL2toL1Leaking (1.3.6.1.2.1.138.1.1.1.9) | read-write | Current | Only support read operation. The value is always false(2) |
| isisSysMaxAge (1.3.6.1.2.1.138.1.1.1.10) | read-write | Current | A value of 350 will be returned when the actual value is less than 350 |
| isisSysReceiveLSPBufferSize(1.3.6.1.2.1.138.1.1.1.11) | read-write | Current | Only support read operation. The value is always 16000 |
| isisSysProtSupported (1.3.6.1.2.1.138.1.1.1.12) | read-only | No | Not supported. The value is always iso8473(0) |
| isisSysNotificationEnable (1.3.6.1.2.1.138.1.1.1.13) | read-write | Current | As per MIB |

isisAreaAddrTable

OID of this table is: 1.3.6.1.2.1.138.1.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisAreaAddr （1.3.6.1.2.1.138.1.1.3.1.1） | read-only | No | The value is always 0 |

isisRouterTable

OID of this table is: 1.3.6.1.2.1.138.1.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisRouterSysID (1.3.6.1.2.1.138.1.1.6.1.1) | not-accessible | No | As per MIB |
| isisRouterLevel (1.3.6.1.2.1.138.1.1.6.1.2) | not-accessible | No | As per MIB |
| isisRouterHostName (1.3.6.1.2.1.138.1.1.6.1.3) | read-only | No | As per MIB |
| isisRouterID (1.3.6.1.2.1.138.1.1.6.1.4) | read-only | No | The value is always 0 |

isisSysLevelTable

OID of this table is: 1.3.6.1.2.1.138.1.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysLevelIndex (1.3.6.1.2.1.138.1.2.1.1.1) | not-accessible | No | As per MIB |
| isisSysLevelOrigLSPBuffSize (1.3.6.1.2.1.138.1.2.1.1.2) | read-write | Current | Only support read operation. The value is always 1497 |
| isisSysLevelMinLSPGenInt (1.3.6.1.2.1.138.1.2.1.1.3) | read-write | Current | Value range: 1 to 120. Default: 1. This object stores the minimum interval in seconds for the exponential LSP generation timer, but this parameter is set in milliseconds at the CLI. If the value set for this parameter is less than 1 second, this object returns 1. If the value is greater than 1 second, this object returns the floor of the value. For example, if the setting is 2.6, this object returns 2. |
| isisSysLevelState (1.3.6.1.2.1.138.1.2.1.1.4) | read-only | No | Only support on(2)，waiting(3) and overloaded(4) |
| isisSysLevelSetOverload (1.3.6.1.2.1.138.1.2.1.1.5) | read-write | Current | As per MIB |
| isisSysLevelSetOverloadUntil (1.3.6.1.2.1.138.1.2.1.1.6) | read-write | Current | This object stores the amount of time that the overload bit can be set. When the timer expires, the overload flag is cleared unless this Intermediate System has run out of memory, or the overload bit is set manually by using the isisSysLevelSetOverload object. |
| isisSysLevelMetricStyle (1.3.6.1.2.1.138.1.2.1.1.7) | read-write | Current | Only support read operation. The value is always wide(2) |
| isisSysLevelSPFConsiders (1.3.6.1.2.1.138.1.2.1.1.8) | read-write | No | Only support read operation. The value is always wide(2) |
| isisSysLevelTEEnabled (1.3.6.1.2.1.138.1.2.1.1.9) | read-write | No | Not supported. The value is always false(2) |

isisCirc

OID of this table is: 1.3.6.1.2.1.138.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisNextCircIndex (1.3.6.1.2.1.138.1.3.1) | read-only | No | Range from 0 to 255 |

isisCircTable

OID of this table is: 1.3.6.1.2.1.138.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircIndex (1.3.6.1.2.1.138.1.3.2.1.1) | not-accessible | No | As per MIB |
| isisCircIfIndex (1.3.6.1.2.1.138.1.3.2.1.2) | read-create | No | Only support read operation |
| isisCircAdminState (1.3.6.1.2.1.138.1.3.2.1.3) | read-create | No | Only support read operation. The value is always on(1). |
| isisCircExistState (1.3.6.1.2.1.138.1.3.2.1.4) | read-create | No | Only support read operation Only support active(1) |
| isisCircType (1.3.6.1.2.1.138.1.3.2.1.5) | read-create | No | Only support read operation Only support ptToPt(2) |
| isisCircExtDomain (1.3.6.1.2.1.138.1.3.2.1.6) | read-create | No | Only support read operation The value is always false（2） |
| isisCircLevelType (1.3.6.1.2.1.138.1.3.2.1.7) | read-create | Current | Only support read operation. The value is always Level1. |
| isisCircPassiveCircuit (1.3.6.1.2.1.138.1.3.2.1.8) | read-create | Current | Only support read operation. The value is always false. |
| isisCircMeshGroupEnabled (1.3.6.1.2.1.138.1.3.2.1.9) | read-create | No | Not supported. The value is always inactive(1). |
| isisCircMeshGroup (1.3.6.1.2.1.138.1.3.2.1.10) | read-create | No | Not supported. The value is always 0. |
| isisCircSmallHellos (1.3.6.1.2.1.138.1.3.2.1.11) | read-create | Current | Not supported. The value is always false. |
| isisCircLastUpTime (1.3.6.1.2.1.138.1.3.2.1.12) | read-only | No | As per MIB |
| isisCirc3WayEnabled (1.3.6.1.2.1.138.1.3.2.1.13) | read-create | No | Only support read operation The value is always true(1) |
| isisCircExtendedCircID (1.3.6.1.2.1.138.1.3.2.1.14) | read-create | No | Only support read operation The value is always 0 |

isisCircLevelTable

OID of this table is: 1.3.6.1.2.1.138.1.4.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircLevelIndex (1.3.6.1.2.1.138.1.4.1.1.1) | not-accessible | No | As per MIB |
| isisCircLevelMetric (1.3.6.1.2.1.138.1.4.1.1.2) | read-write | Current | Not supported. The value is always 0. |
| isisCircLevelWideMetric (1.3.6.1.2.1.138.1.4.1.1.3) | read-write | Current | Not supported. The value is always 0. |
| isisCircLevelISPriority (1.3.6.1.2.1.138.1.4.1.1.4) | read-write | No | Not supported. The value is always 64. |
| isisCircLevelIDOctet (1.3.6.1.2.1.138.1.4.1.1.5) | read-only | No | As per MIB |
| isisCircLevelID (1.3.6.1.2.1.138.1.4.1.1.6) | read-only | No | The value consists of the isisSysID and the one-byte isisCircLevelIDOctet |
| isisCircLevelDesIS (1.3.6.1.2.1.138.1.4.1.1.7) | read-only | No | Not supported. The value is zero-length OCTET STRING. |
| isisCircLevelHelloMultiplier (1.3.6.1.2.1.138.1.4.1.1.8) | read-write | Current | Range from 3 to 100 The default value is 3 |
| isisCircLevelHelloTimer (1.3.6.1.2.1.138.1.4.1.1.9) | read-write | Current | Range from 3000 to 255000 The default value is 10000 The remainder of the set value divided by 1000 is ignored |
| isisCircLevelDRHelloTimer (1.3.6.1.2.1.138.1.4.1.1.10) | read-write | No | Only support read operation. |
| isisCircLevelLSPThrottle (1.3.6.1.2.1.138.1.4.1.1.11) | read-write | Current | Range from 1 to 1000 The default value is 33 |
| isisCircLevelMinLSPRetransInt (1.3.6.1.2.1.138.1.4.1.1.12) | read-write | Current | Only support read operation. The value is always 5. |
| isisCircLevelCSNPInterval (1.3.6.1.2.1.138.1.4.1.1.13) | read-write | Current | Only support read operation The value is always 10 |
| isisCircLevelPartSNPInterval (1.3.6.1.2.1.138.1.4.1.1.14) | read-write | No | Only support read operation The value is always 2 |

isisSystemCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysStatLevel (1.3.6.1.2.1.138.1.5.1.1.1) | not-accessible | No | As per MIB |
| isisSysStatCorrLSPs (1.3.6.1.2.1.138.1.5.1.1.2) | read-only | No | Not supported. The value is always 0 |
| isisSysStatAuthTypeFails (1.3.6.1.2.1.138.1.5.1.1.3) | read-only | No | As per MIB |
| isisSysStatAuthFails (1.3.6.1.2.1.138.1.5.1.1.4) | read-only | No | As per MIB |
| isisSysStatLSPDbaseOloads (1.3.6.1.2.1.138.1.5.1.1.5) | read-only | No | As per MIB |
| isisSysStatManAddrDropFromAreas (1.3.6.1.2.1.138.1.5.1.1.6) | read-only | No | Not supported. The value is always 0 |
| isisSysStatAttmptToExMaxSeqNums (1.3.6.1.2.1.138.1.5.1.1.7) | read-only | No | As per MIB |
| isisSysStatSeqNumSkips (1.3.6.1.2.1.138.1.5.1.1.8) | read-only | No | As per MIB |
| isisSysStatOwnLSPPurges (1.3.6.1.2.1.138.1.5.1.1.9) | read-only | No | As per MIB |
| isisSysStatIDFieldLenMismatches (1.3.6.1.2.1.138.1.5.1.1.10) | read-only | No | As per MIB |
| isisSysStatPartChanges (1.3.6.1.2.1.138.1.5.1.1.11) | read-only | No | Not supported. The value is always 0 |
| isisSysStatSPFRuns (1.3.6.1.2.1.138.1.5.1.1.12) | read-only | No | As per MIB |
| isisSysStatLSPErrors (1.3.6.1.2.1.138.1.5.1.1.13) | read-only | No | As per MIB |

isisCircuitCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircuitType (1.3.6.1.2.1.138.1.5.2.1.1) | not-accessible | No | As per MIB |
| isisCircAdjChanges (1.3.6.1.2.1.138.1.5.2.1.2) | read-only | No | As per MIB |
| isisCircNumAdj (1.3.6.1.2.1.138.1.5.2.1.3) | read-only | No | As per MIB |
| isisCircInitFails (1.3.6.1.2.1.138.1.5.2.1.4) | read-only | No | Not supported. The value is always 0 |
| isisCircRejAdjs (1.3.6.1.2.1.138.1.5.2.1.5) | read-only | No | As per MIB |
| isisCircIDFieldLenMismatches (1.3.6.1.2.1.138.1.5.2.1.6) | read-only | No | As per MIB |
| isisCircMaxAreaAddrMismatches (1.3.6.1.2.1.138.1.5.2.1.7) | read-only | No | As per MIB |
| isisCircAuthTypeFails (1.3.6.1.2.1.138.1.5.2.1.8) | read-only | No | As per MIB |
| isisCircAuthFails (1.3.6.1.2.1.138.1.5.2.1.9) | read-only | No | As per MIB |
| isisCircLANDesISChanges (1.3.6.1.2.1.138.1.5.2.1.10) | read-only | No | Not supported. The value is always 0 |

isisPacketCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisPacketCountLevel (1.3.6.1.2.1.138.1.5.3.1.1) | not-accessible | No | As per MIB |
| isisPacketCountDirection (1.3.6.1.2.1.138.1.5.3.1.2) | not-accessible | No | As per MIB |
| isisPacketCountIIHello (1.3.6.1.2.1.138.1.5.3.1.3) | read-only | No | As per MIB |
| isisPacketCountISHello (1.3.6.1.2.1.138.1.5.3.1.4) | read-only | No | Not supported. The value is always 0 |
| isisPacketCountESHello (1.3.6.1.2.1.138.1.5.3.1.5) | read-only | No | Not supported. The value is always 0 |
| isisPacketCountLSP (1.3.6.1.2.1.138.1.5.3.1.6) | read-only | No | As per MIB |
| isisPacketCountCSNP (1.3.6.1.2.1.138.1.5.3.1.7) | read-only | No | As per MIB |
| isisPacketCountPSNP (1.3.6.1.2.1.138.1.5.3.1.8) | read-only | No | As per MIB |
| isisPacketCountUnknown (1.3.6.1.2.1.138.1.5.3.1.9) | read-only | No | Not supported. The value is always 0 |

isisISAdjTable

OID of this table is: 1.3.6.1.2.1.138.1.6.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisISAdjIndex (1.3.6.1.2.1.138.1.6.1.1.1) | not-accessible | No | As per MIB |
| isisISAdjState (1.3.6.1.2.1.138.1.6.1.1.2) | read-only | No | Only support down(1), initializing(2) and up(3) |
| isisISAdj3WayState (1.3.6.1.2.1.138.1.6.1.1.3) | read-only | No | Only support up(0), initializing(1) and down(2) |
| isisISAdjNeighSNPAAddress (1.3.6.1.2.1.138.1.6.1.1.4) | read-only | No | As per MIB |
| isisISAdjNeighSysType (1.3.6.1.2.1.138.1.6.1.1.5) | read-only | No | The value is always l1IntermediateSystem(1) |
| isisISAdjNeighSysID (1.3.6.1.2.1.138.1.6.1.1.6) | read-only | No | As per MIB |
| isisISAdjNbrExtendedCircID (1.3.6.1.2.1.138.1.6.1.1.7) | read-only | No | As per MIB |
| isisISAdjUsage (1.3.6.1.2.1.138.1.6.1.1.8) | read-only | No | The value is always level1 |
| isisISAdjHoldTimer (1.3.6.1.2.1.138.1.6.1.1.9) | read-only | No | As per MIB |
| isisISAdjNeighPriority (1.3.6.1.2.1.138.1.6.1.1.10) | read-only | No | The value is always 64 |
| isisISAdjLastUpTime (1.3.6.1.2.1.138.1.6.1.1.11) | read-only | No | As per MIB |

isisRATable

OID of this table is: 1.3.6.1.2.1.138.1.7.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisRAIndex (1.3.6.1.2.1.138.1.7.1.1.1) | not-accessible | No | As per MIB |
| isisRAExistState (1.3.6.1.2.1.138.1.7.1.1.2) | read-create | No | The value is always active(1) |
| isisRAAdminState (1.3.6.1.2.1.138.1.7.1.1.3) | read-create | No | The default value is on(1) |
| isisRAAddrPrefix (1.3.6.1.2.1.138.1.7.1.1.4) | read-create | No | As per MIB |
| isisRAMapType (1.3.6.1.2.1.138.1.7.1.1.5) | read-create | No | The default value is none(1) |
| isisRAMetric (1.3.6.1.2.1.138.1.7.1.1.6) | read-create | No | As per MIB |
| isisRAMetricType (1.3.6.1.2.1.138.1.7.1.1.7) | read-create | No | As per MIB |
| isisRASNPAAddress (1.3.6.1.2.1.138.1.7.1.1.8) | read-create | No | As per MIB |
| isisRASNPAMask (1.3.6.1.2.1.138.1.7.1.1.9) | read-create | No | As per MIB |
| isisRASNPAPrefix (1.3.6.1.2.1.138.1.7.1.1.10) | read-create | No | As per MIB |
| isisRAType (1.3.6.1.2.1.138.1.7.1.1.11) | read-create | No | As per MIB |

isisLSPSummaryTable

OID of this table is: 1.3.6.1.2.1.138.1.9.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisLSPLevel (1.3.6.1.2.1.138.1.9.1.1.1) | not-accessible | No | As per MIB |
| isisLSPID (1.3.6.1.2.1.138.1.9.1.1.2) | not-accessible | No | As per MIB |
| isisLSPSeq (1.3.6.1.2.1.138.1.9.1.1.3) | read-only | No | As per MIB |
| isisLSPZeroLife (1.3.6.1.2.1.138.1.9.1.1.4) | read-only | No | As per MIB |
| isisLSPChecksum (1.3.6.1.2.1.138.1.9.1.1.5) | read-only | No | As per MIB |
| isisLSPLifetimeRemain (1.3.6.1.2.1.138.1.9.1.1.6) | read-only | No | As per MIB |
| isisLSPPDULength (1.3.6.1.2.1.138.1.9.1.1.7) | read-only | No | As per MIB |
| isisLSPAttributes (1.3.6.1.2.1.138.1.9.1.1.8) | read-only | No | As per MIB |

isisLSPTLVTable

OID of this table is: 1.3.6.1.2.1.138.1.9.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisLSPTLVIndex (1.3.6.1.2.1.138.1.9.2.1.1) | not-accessible | No | As per MIB |
| isisLSPTLVSeq (1.3.6.1.2.1.138.1.9.2.1.2) | read-only | No | As per MIB |
| isisLSPTLVChecksum (1.3.6.1.2.1.138.1.9.2.1.3) | read-only | No | As per MIB |
| isisLSPTLVType (1.3.6.1.2.1.138.1.9.2.1.4) | read-only | No | As per MIB |
| isisLSPTLVLen (1.3.6.1.2.1.138.1.9.2.1.5) | read-only | No | As per MIB |
| isisLSPTLVValue (1.3.6.1.2.1.138.1.9.2.1.6) | read-only | No | As per MIB |

isisNotificationEntry

OID of this table is: 1.3.6.1.2.1.138.1.10.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisNotificationSysLevelIndex (1.3.6.1.2.1.138.1.10.1.1) | accessible-for-notify | No | As per MIB |
| isisNotificationCircIfIndex (1.3.6.1.2.1.138.1.10.1.2) | accessible-for-notify | No | As per MIB |
| isisPduLspId (1.3.6.1.2.1.138.1.10.1.3) | accessible-for-notify | No | As per MIB |
| isisPduFragment (1.3.6.1.2.1.138.1.10.1.4) | accessible-for-notify | No | As per MIB |
| isisPduFieldLen (1.3.6.1.2.1.138.1.10.1.5) | accessible-for-notify | No | As per MIB |
| isisPduMaxAreaAddress (1.3.6.1.2.1.138.1.10.1.6) | accessible-for-notify | No | As per MIB |
| isisPduProtocolVersion (1.3.6.1.2.1.138.1.10.1.7) | accessible-for-notify | No | As per MIB |
| isisPduLspSize (1.3.6.1.2.1.138.1.10.1.8) | accessible-for-notify | No | As per MIB |
| isisPduOriginatingBufferSize (1.3.6.1.2.1.138.1.10.1.9) | accessible-for-notify | No | As per MIB |
| isisPduBufferSize (1.3.6.1.2.1.138.1.10.1.10) | accessible-for-notify | No | As per MIB |
| isisPduProtocolsSupported (1.3.6.1.2.1.138.1.10.1.11) | accessible-for-notify | No | As per MIB |
| isisAdjState (1.3.6.1.2.1.138.1.10.1.12) | accessible-for-notify | No | As per MIB |
| isisErrorOffset (1.3.6.1.2.1.138.1.10.1.13) | accessible-for-notify | No | As per MIB |
| isisErrorTLVType (1.3.6.1.2.1.138.1.10.1.14) | accessible-for-notify | No | As per MIB |
| isisNotificationAreaAddress (1.3.6.1.2.1.138.1.10.1.15) | accessible-for-notify | No | As per MIB |

ISIS-MIB (for TRILL)

Scalar objects of isisSysObject

OID of this table is: 1.3.6.1.2.1.138.1.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysVersion (1.3.6.1.2.1.138.1.1.1.1) | read-only | No | As per MIB |
| isisSysLevelType (1.3.6.1.2.1.138.1.1.1.2) | read-write | Current | Only support read operation |
| isisSysID (1.3.6.1.2.1.138.1.1.1.3) | read-write | Current | System will temporarily unavailable after the value is set |
| isisSysMaxPathSplits (1.3.6.1.2.1.138.1.1.1.4) | read-write | Current | The range and the default value are product specific |
| isisSysMaxLSPGenInt (1.3.6.1.2.1.138.1.1.1.5) | read-write | Current | The value should be less than isisSysMaxAge, It should be at least 1 seconds less than isisSysMaxAge. A value of 65235 will be return when the actual is greater than 65235 |
| isisSysPollESHelloRate (1.3.6.1.2.1.138.1.1.1.6) | read-write | No | Not supported，The value is always 50 |
| isisSysWaitTime (1.3.6.1.2.1.138.1.1.1.7) | read-write | No | Not supported，The value is always 60 |
| IsisSysAdminState (1.3.6.1.2.1.138.1.1.1.8) | read-write | No | Only support read operation |
| isisSysL2toL1Leaking (1.3.6.1.2.1.138.1.1.1.9) | read-write | Current | The value is always false(2) |
| isisSysMaxAge (1.3.6.1.2.1.138.1.1.1.10) | read-write | Current | The value should be greater than isisSysMaxLSPGenInt, It should be at least 1 seconds greater than isisSysMaxLSPGenInt. A value of 350 will be return when the actual value is less than 350 |
| IsisSysReceiveLSPBufferSize (1.3.6.1.2.1.138.1.1.1.11) | read-write | Current | A value of 16000 will be return when the actual is greater than 16000, A value of 1492 will be return when the actual value is less than 1492 |
| isisSysProtSupported (1.3.6.1.2.1.138.1.1.1.12) | read-only | No | The value is always iso8473 (0) |
| isisSysNotificationEnable (1.3.6.1.2.1.138.1.1.1.13) | read-write | Current |  |

isisRouterTable

OID of this table is: 1.3.6.1.2.1.138.1.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisRouterSysID (1.3.6.1.2.1.138.1.1.6.1.1) | not-accessible | No | As per MIB |
| isisRouterLevel (1.3.6.1.2.1.138.1.1.6.1.2) | not-accessible | No | The value is always Level1(0) |
| isisRouterHostName (1.3.6.1.2.1.138.1.1.6.1.3) | read-only | No | Not supported. The value is always zero-length string. |
| isisRouterID (1.3.6.1.2.1.138.1.1.6.1.4) | read-only | No | RBridge’s nickname is be returned. |

isisSysLevelTable

OID of this table is: 1.3.6.1.2.1.138.1.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysLevelIndex (1.3.6.1.2.1.138.1.2.1.1.1) | not-accessible | No | As per MIB |
| isisSysLevelOrigLSPBuffSize (1.3.6.1.2.1.138.1.2.1.1.2) | read-write | Current | The default value is 1458. A value of 16000 will be return when the actual is greater than 16000. |
| isisSysLevelMinLSPGenInt (1.3.6.1.2.1.138.1.2.1.1.3) | read- write | NO | Only support read operation, The value is always 2. |
| isisSysLevelState (1.3.6.1.2.1.138.1.2.1.1.4) | read-only | No | Only support on(2) and overload(4) |
| isisSysLevelSetOverload (1.3.6.1.2.1.138.1.2.1.1.5) | read-write | No | As per MIB |
| isisSysLevelSetOverloadUntil (1.3.6.1.2.1.138.1.2.1.1.6) | read-write | No | Range from 5 to 3600 when setting. When multiple value binding, the latter variable will overlay the front variable. |
| isisSysLevelMetricStyle (1.3.6.1.2.1.138.1.2.1.1.7) | read-write | Current | Only support read operation, The value is always wide(2). |
| isisSysLevelSPFConsiders (1.3.6.1.2.1.138.1.2.1.1.8) | read-write | No | Only support read operation, The value is always wide(2). |
| isisSysLevelTEEnabled (1.3.6.1.2.1.138.1.2.1.1.9) | read-write | No | Not supported, The value is always false(2). |

Scalar objects of isisCirc

OID of this table is: 1.3.6.1.2.1.138.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisNextCircIndex (1.3.6.1.2.1.138.1.3.1) | read-only | No | As per MIB |

isisCircTable

OID of this table is: 1.3.6.1.2.1.138.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircIndex (1.3.6.1.2.1.138.1.3.2.1.1) | not-accessible | No | As per MIB |
| isisCircIfIndex (1.3.6.1.2.1.138.1.3.2.1.2) | read-create | No |  |
| isisCircAdminState (1.3.6.1.2.1.138.1.3.2.1.3) | read-create | No | Only support read operation The value is always on(1) |
| isisCircExistState (1.3.6.1.2.1.138.1.3.2.1.4) | read-create | No | Only support active(1), createAndGo(4) and destroy(6). |
| isisCircType (1.3.6.1.2.1.138.1.3.2.1.5) | read-create | No | Only support read operation. The value is always broadcast(1) |
| isisCircExtDomain (1.3.6.1.2.1.138.1.3.2.1.6) | read-create | No | Not supported, The value is always FALSE |
| isisCircLevelType (1.3.6.1.2.1.138.1.3.2.1.7) | read-create | No | Only support read operation The value is always Level1 |
| isisCircPassiveCircuit (1.3.6.1.2.1.138.1.3.2.1.8) | read-create | No | Only support read operation The value is always false |
| isisCircMeshGroupEnabled (1.3.6.1.2.1.138.1.3.2.1.9) | read-create | Current | Not supported, The value is always inactive(1). |
| isisCircMeshGroup (1.3.6.1.2.1.138.1.3.2.1.10) | read-create | Current | Not supported, The value is always 0. |
| isisCircSmallHellos (1.3.6.1.2.1.138.1.3.2.1.11) | read-create | No | Not supported, The value is always false. |
| isisCircLastUpTime (1.3.6.1.2.1.138.1.3.2.1.12) | read-only | No | As per MIB |
| isisCirc3WayEnabled (1.3.6.1.2.1.138.1.3.2.1.13) | read-create | No | Not supported, The value is always false. |
| isisCircExtendedCircID (1.3.6.1.2.1.138.1.3.2.1.14) | read-create | No | Not supported, The value is always 0. |

isisCircLevelTable

OID of this table is: 1.3.6.1.2.1.138.1.4.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircLevelIndex (1.3.6.1.2.1.138.1.4.1.1.1) | not-accessible | No | As per MIB |
| isisCircLevelMetric (1.3.6.1.2.1.138.1.4.1.1.2) | read-write | Current | Not supported, The value is always 0. |
| isisCircLevelWideMetric (1.3.6.1.2.1.138.1.4.1.1.3) | read-write | Current | Range from 0 to 16777214, It will return a value that the system make when the value is set to 0. |
| isisCircLevelISPriority (1.3.6.1.2.1.138.1.4.1.1.4) | read-write | No | As per MIB |
| isisCircLevelIDOctet (1.3.6.1.2.1.138.1.4.1.1.5) | read-only | No |  |
| isisCircLevelID (1.3.6.1.2.1.138.1.4.1.1.6) | read-only | No | Not supported, The value is zero-length OCTET STRING. |
| isisCircLevelDesIS (1.3.6.1.2.1.138.1.4.1.1.7) | read-only | No | As per MIB |
| isisCircLevelHelloMultiplier (1.3.6.1.2.1.138.1.4.1.1.8) | read-write | Current | Range from 2 to 100, The default value is 3. It will return 100 when the value is great than 100. |
| isisCircLevelHelloTimer (1.3.6.1.2.1.138.1.4.1.1.9) | read-write | Current | Range from 1000 to 255000, The default value is 10000ms, in step of 1000. |
| isisCircLevelDRHelloTimer (1.3.6.1.2.1.138.1.4.1.1.10) | read-write | Current | Only support read operation. |
| isisCircLevelLSPThrottle (1.3.6.1.2.1.138.1.4.1.1.11) | read-write | Current | Range from 10 to 1000, The default value is 10ms, in step of 10. |
| isisCircLevelMinLSPRetransInt(1.3.6.1.2.1.138.1.4.1.1.12) | read-write | Current | Not supported P2P Hello, The value is always 5. |
| isisCircLevelCSNPInterval (1.3.6.1.2.1.138.1.4.1.1.13) | read-write | Current | As per MIB |
| isisCircLevelPartSNPInterval (1.3.6.1.2.1.138.1.4.1.1.14) | read-write | Current | Only support read operation. |

isisSystemCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisSysStatLevel (1.3.6.1.2.1.138.1.5.1.1.1) | not-accessible | No | As per MIB |
| isisSysStatCorrLSPs (1.3.6.1.2.1.138.1.5.1.1.2) | read-only | No | Not supported, The value is always 0. |
| isisSysStatAuthTypeFails (1.3.6.1.2.1.138.1.5.1.1.3) | read-only | No | Not supported, The value is always 0. |
| isisSysStatAuthFails (1.3.6.1.2.1.138.1.5.1.1.4) | read-only | No | Not supported, The value is always 0. |
| isisSysStatLSPDbaseOloads (1.3.6.1.2.1.138.1.5.1.1.5) | read-only | No | As per MIB |
| isisSysStatManAddrDropFromAreas (1.3.6.1.2.1.138.1.5.1.1.6) | read-only | No | Not supported, The value is always 0. |
| isisSysStatAttmptToExMaxSeqNums (1.3.6.1.2.1.138.1.5.1.1.7) | read-only | No | As per MIB |
| isisSysStatSeqNumSkips (1.3.6.1.2.1.138.1.5.1.1.8) | read-only | No | As per MIB |
| isisSysStatOwnLSPPurges (1.3.6.1.2.1.138.1.5.1.1.9) | read-only | No | As per MIB |
| isisSysStatIDFieldLenMismatches (1.3.6.1.2.1.138.1.5.1.1.10) | read-only | No | As per MIB |
| isisSysStatPartChanges (1.3.6.1.2.1.138.1.5.1.1.11) | read-only | No | Not supported, The value is always 0. |
| isisSysStatSPFRuns (1.3.6.1.2.1.138.1.5.1.1.12) | read-only | No | As per MIB |
| isisSysStatLSPErrors (1.3.6.1.2.1.138.1.5.1.1.13) | read-only | No | As per MIB |

isisCircuitCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisCircuitType (1.3.6.1.2.1.138.1.5.2.1.1) | not-accessible | No | As per MIB |
| isisCircAdjChanges (1.3.6.1.2.1.138.1.5.2.1.2) | read-only | No | As per MIB |
| isisCircNumAdj (1.3.6.1.2.1.138.1.5.2.1.3) | read-only | No | As per MIB |
| isisCircInitFails (1.3.6.1.2.1.138.1.5.2.1.4) | read-only | No | Not supported, The value is always 0. |
| isisCircRejAdjs (1.3.6.1.2.1.138.1.5.2.1.5) | read-only | No | As per MIB |
| IsisCircIDFieldLenMismatches (1.3.6.1.2.1.138.1.5.2.1.6) | read-only | No | As per MIB |
| isisCircMaxAreaAddrMismatches(1.3.6.1.2.1.138.1.5.2.1.7) | read-only | No | Not supported, The value is always 0. |
| isisCircAuthTypeFails (1.3.6.1.2.1.138.1.5.2.1.8) | read-only | No | Not supported, The value is always 0. |
| isisCircAuthFails (1.3.6.1.2.1.138.1.5.2.1.9) | read-only | No | Not supported, The value is always 0. |
| isisCircLANDesISChanges (1.3.6.1.2.1.138.1.5.2.1.10) | read-only | No | As per MIB |

isisPacketCounterTable

OID of this table is: 1.3.6.1.2.1.138.1.5.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisPacketCountLevel (1.3.6.1.2.1.138.1.5.3.1.1) | not-accessible | No | As per MIB |
| isisPacketCountDirection (1.3.6.1.2.1.138.1.5.3.1.2) | not-accessible | No | As per MIB |
| isisPacketCountIIHello (1.3.6.1.2.1.138.1.5.3.1.3) | read-only | No | As per MIB |
| isisPacketCountISHello (1.3.6.1.2.1.138.1.5.3.1.4) | read-only | No | Not supported, The value is always 0. |
| isisPacketCountESHello (1.3.6.1.2.1.138.1.5.3.1.5) | read-only | No | Not supported, The value is always 0. |
| isisPacketCountLSP (1.3.6.1.2.1.138.1.5.3.1.6) | read-only | No | As per MIB |
| isisPacketCountCSNP (1.3.6.1.2.1.138.1.5.3.1.7) | read-only | No | As per MIB |
| isisPacketCountPSNP (1.3.6.1.2.1.138.1.5.3.1.8) | read-only | No | As per MIB |
| isisPacketCountUnknown (1.3.6.1.2.1.138.1.5.3.1.9) | read-only | No | Not supported, The value is always 0. |

isisISAdjTable

OID of this table is: 1.3.6.1.2.1.138.1.6.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisISAdjIndex (1.3.6.1.2.1.138.1.6.1.1.1) | not-accessible | No | As per MIB |
| isisISAdjState (1.3.6.1.2.1.138.1.6.1.1.2) | read-only | No | With TRILL protocol, Each adjacency is in one of the following four states： { Down (1), Detect (2), 2-Way (3), Report (4) }. Current, The “2-Way” is be supported. |
| isisISAdj3WayState (1.3.6.1.2.1.138.1.6.1.1.3) | read-only | No | Not supported, The value is always down(2). |
| isisISAdjNeighSNPAAddress (1.3.6.1.2.1.138.1.6.1.1.4) | read-only | No | As per MIB |
| isisISAdjNeighSysType (1.3.6.1.2.1.138.1.6.1.1.5) | read-only | No | As per MIB |
| isisISAdjNeighSysID (1.3.6.1.2.1.138.1.6.1.1.6) | read-only | No | As per MIB |
| isisISAdjNbrExtendedCircID (1.3.6.1.2.1.138.1.6.1.1.7) | read-only | No | Not supported, The value is always 0. |
| isisISAdjUsage (1.3.6.1.2.1.138.1.6.1.1.8) | read-only | No | As per MIB |
| isisISAdjHoldTimer (1.3.6.1.2.1.138.1.6.1.1.9) | read-only | No | As per MIB |
| isisISAdjNeighPriority (1.3.6.1.2.1.138.1.6.1.1.10) | read-only | No | As per MIB |
| isisISAdjLastUpTime (1.3.6.1.2.1.138.1.6.1.1.11) | read-only | No | As per MIB |

isisLSPSummaryTable

OID of this table is: 1.3.6.1.2.1.138.1.9.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisLSPLevel (1.3.6.1.2.1.138.1.9.1.1.1) | not-accessible | No | As per MIB |
| isisLSPID (1.3.6.1.2.1.138.1.9.1.1.2) | not-accessible | No | As per MIB |
| isisLSPSeq (1.3.6.1.2.1.138.1.9.1.1.3) | read-only | No | As per MIB |
| isisLSPZeroLife (1.3.6.1.2.1.138.1.9.1.1.4) | read-only | No | As per MIB |
| isisLSPChecksum (1.3.6.1.2.1.138.1.9.1.1.5) | read-only | No | As per MIB |
| isisLSPLifetimeRemain (1.3.6.1.2.1.138.1.9.1.1.6) | read-only | No | As per MIB |
| isisLSPPDULength (1.3.6.1.2.1.138.1.9.1.1.7) | read-only | No | As per MIB |
| isisLSPAttributes (1.3.6.1.2.1.138.1.9.1.1.8) | read-only | No | As per MIB |

isisLSPTLVTable

OID of this table is: 1.3.6.1.2.1.138.1.9.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisLSPTLVIndex (1.3.6.1.2.1.138.1.9.2.1.1) | not-accessible | No | As per MIB |
| isisLSPTLVSeq (1.3.6.1.2.1.138.1.9.2.1.2) | read-only | No | As per MIB |
| isisLSPTLVChecksum (1.3.6.1.2.1.138.1.9.2.1.3) | read-only | No | As per MIB |
| isisLSPTLVType (1.3.6.1.2.1.138.1.9.2.1.4) | read-only | No | As per MIB |
| isisLSPTLVLen (1.3.6.1.2.1.138.1.9.2.1.5) | read-only | No | As per MIB |
| isisLSPTLVValue (1.3.6.1.2.1.138.1.9.2.1.6) | read-only | No | As per MIB |

IsisNotification

OID of this table is: 1.3.6.1.2.1.138.1.10

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| isisNotificationSysLevelIndex (1.3.6.1.2.1.138.1.10.1.1) | accessible-for-notify | No | As per MIB |
| isisNotificationCircIfIndex (1.3.6.1.2.1.138.1.10.1.2) | accessible-for-notify | No | As per MIB |
| isisPduLspId (1.3.6.1.2.1.138.1.10.1.3) | accessible-for-notify | No | As per MIB |
| IsisPduFragment (1.3.6.1.2.1.138.1.10.1.4) | accessible-for-notify | No | As per MIB |
| IsisPduFieldLen (1.3.6.1.2.1.138.1.10.1.5) | accessible-for-notify | No | As per MIB |
| isisPduMaxAreaAddress (1.3.6.1.2.1.138.1.10.1.6) | accessible-for-notify | No | As per MIB |
| isisPduProtocolVersion (1.3.6.1.2.1.138.1.10.1.7) | accessible-for-notify | No | As per MIB |
| isisPduLspSize (1.3.6.1.2.1.138.1.10.1.8) | accessible-for-notify | No | As per MIB |
| isisPduOriginatingBufferSize (1.3.6.1.2.1.138.1.10.1.9) | accessible-for-notify | No | As per MIB |
| isisPduBufferSize (1.3.6.1.2.1.138.1.10.1.10) | accessible-for-notify | No | As per MIB |
| isisPduProtocolsSupported (1.3.6.1.2.1.138.1.10.1.11) | accessible-for-notify | No | As per MIB |
| isisAdjState (1.3.6.1.2.1.138.1.10.1.12) | accessible-for-notify | No | As per MIB |
| isisErrorOffset (1.3.6.1.2.1.138.1.10.1.13) | accessible-for-notify | No | As per MIB |
| isisErrorTLVType (1.3.6.1.2.1.138.1.10.1.14) | accessible-for-notify | No | As per MIB |
| isisNotificationAreaAddress (1.3.6.1.2.1.138.1.10.1.15) | accessible-for-notify | No | As per MIB |