# Contents

LLDP-EXT-MED-MIB	2

Scalar Objects	2

lldpXMedPortConfigEntry	2

lldpXMedLocLocationTable	3

lldpXMedRemCapabilitiesTable	3

lldpXMedRemMediaPolicyTable	3

lldpXMedRemInventoryTable	4

lldpXMedRemLocationTable	4

lldpXMedRemXPoETable	5

lldpXMedRemXPoEPSETable	5

lldpXMedRemXPoEPDTable	5

LLDP-EXT-MED-MIB

The LLDP Management Information Base extension module for TIA-TR41.4 Media Endpoint Discovery information.

Scalar Objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedLocDeviceClass (1.0.8802.1.1.2.1.5.4795.1.1.1) | read-only | Current | As per MIB |
| lldpXMedFastStartRepeatCount (1.0.8802.1.1.2.1.5.4795.1.1.3) | read-write | Current | Range from 1 to 8. The default value is 4. |
| lldpXMedLocHardwareRev (1.0.8802.1.1.2.1.5.4795.1.2.2) | read-only | Current | As per MIB |
| lldpXMedLocFirmwareRev (1.0.8802.1.1.2.1.5.4795.1.2.3) | read-only | Current | As per MIB |
| lldpXMedLocSoftwareRev (1.0.8802.1.1.2.1.5.4795.1.2.4) | read-only | Current | As per MIB |
| lldpXMedLocSerialNum (1.0.8802.1.1.2.1.5.4795.1.2.5) | read-only | Current | As per MIB |
| lldpXMedLocMfgName (1.0.8802.1.1.2.1.5.4795.1.2.6) | read-only | Current | As per MIB |
| lldpXMedLocModelName (1.0.8802.1.1.2.1.5.4795.1.2.7) | read-only | Current | As per MIB |
| lldpXMedLocAssetID (1.0.8802.1.1.2.1.5.4795.1.2.8) | read-only | Current | As per MIB |
| lldpxMedLocXPoEDeviceType (1.0.8802.1.1.2.1.5.4795.1.2.10) | read-only | Current | Not supported |
| lldpXMedLocXPoEPSEPowerSource (1.0.8802.1.1.2.1.5.4795.1.2.12) | read-only | Current | Not supported |
| lldpXMedLocXPoEPDPowerReq (1.0.8802.1.1.2.1.5.4795.1.2.13) | read-only | Current | Not supported |
| lldpXMedLocXPoEPDPowerSource (1.0.8802.1.1.2.1.5.4795.1.2.14) | read-only | Current | Not supported |
| lldpXMedLocXPoEPDPowerPriority (1.0.8802.1.1.2.1.5.4795.1.2.15) | read-only | Current | Not supported |

lldpXMedPortConfigEntry

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.1.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedPortCapSupported (1.0.8802.1.1.2.1.5.4795.1.1.2.1.1) | read-only | Current | As per MIB |
| lldpXMedPortConfigTLVsTxEnable (1.0.8802.1.1.2.1.5.4795.1.1.2.1.2) | read-write | Current | The networkPolicy bit will be set to '1' by default when interface works in bridge mode. Otherwise, it will be set to '0' by default. The location and extendedPD bits will be set to '0' by default, and other bits will be set to '1' by default. If the extendedPSE bit is set but respective capability is not supported per port, the agent will not send the corresponding TLV. The location bit only support read operation. |
| lldpXMedPortConfigNotifEnable (1.0.8802.1.1.2.1.5.4795.1.1.2.1.3) | read-write | Current | As per MIB |

lldpXMedLocLocationTable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.2.9

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedLocLocationSubtype (1.0.8802.1.1.2.1.5.4795.1.2.9.1.1) | not-accessible | Current | Only civicAddress(3) and elin(4) are supported. |
| lldpXMedLocLocationInfo (1.0.8802.1.1.2.1.5.4795.1.2.9.1.2) | read-write | Current | The bit 'location(2)' of the lldpXMedPortConfigTLVsTxEnable object will be set to '1' when the location information is configured, and it will be set to '0' when the location information is deleted. |

lldpXMedRemCapabilitiesTable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemCapSupported (1.0.8802.1.1.2.1.5.4795.1.3.1.1.1) | read-only | No | As per MIB |
| lldpXMedRemCapCurrent (1.0.8802.1.1.2.1.5.4795.1.3.1.1.2) | read-only | No | As per MIB |
| lldpXMedRemDeviceClass (1.0.8802.1.1.2.1.5.4795.1.3.1.1.3) | read-only | No | As per MIB |

lldpXMedRemMediaPolicyTable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemMediaPolicyAppType (1.0.8802.1.1.2.1.5.4795.1.3.2.1.1) | not-accessible | No | As per MIB |
| lldpXMedRemMediaPolicyVlanID (1.0.8802.1.1.2.1.5.4795.1.3.2.1.2) | read-only | No | As per MIB |
| lldpXMedRemMediaPolicyPriority (1.0.8802.1.1.2.1.5.4795.1.3.2.1.3) | read-only | No | As per MIB |
| lldpXMedRemMediaPolicyDscp (1.0.8802.1.1.2.1.5.4795.1.3.2.1.4) | read-only | No | As per MIB |
| lldpXMedRemMediaPolicyUnknown (1.0.8802.1.1.2.1.5.4795.1.3.2.1.5) | read-only | No | As per MIB |
| lldpXMedRemMediaPolicyTagged (1.0.8802.1.1.2.1.5.4795.1.3.2.1.6) | read-only | No | As per MIB |

lldpXMedRemInventoryTable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemHardwareRev (1.0.8802.1.1.2.1.5.4795.1.3.3.1.1) | read-only | No | As per MIB |
| lldpXMedRemFirmwareRev (1.0.8802.1.1.2.1.5.4795.1.3.3.1.2) | read-only | No | As per MIB |
| lldpXMedRemSoftwareRev (1.0.8802.1.1.2.1.5.4795.1.3.3.1.3) | read-only | No | As per MIB |
| lldpXMedRemSerialNum (1.0.8802.1.1.2.1.5.4795.1.3.3.1.4) | read-only | No | As per MIB |
| lldpXMedRemMfgName (1.0.8802.1.1.2.1.5.4795.1.3.3.1.5) | read-only | No | As per MIB |
| lldpXMedRemModelName (1.0.8802.1.1.2.1.5.4795.1.3.3.1.6) | read-only | No | As per MIB |
| lldpXMedRemAssetID (1.0.8802.1.1.2.1.5.4795.1.3.3.1.7) | read-only | No | As per MIB |

lldpXMedRemLocationTable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemLocationSubtype(1.0.8802.1.1.2.1.5.4795.1.3.4.1.1) | not-accessible | No | As per MIB |
| lldpXMedRemLocationInfo (1.0.8802.1.1.2.1.5.4795.1.3.4.1.2) | read-only | No | As per MIB |

lldpXMedRemXPoETable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemXPoEDeviceType (1.0.8802.1.1.2.1.5.4795.1.3.5.1.1) | read-only | No | As per MIB |

lldpXMedRemXPoEPSETable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemXPoEPSEPowerAv (1.0.8802.1.1.2.1.5.4795.1.3.6.1.1) | read-only | No | As per MIB |
| lldpXMedRemXPoEPSEPowerSource (1.0.8802.1.1.2.1.5.4795.1.3.6.1.2) | read-only | No | As per MIB |
| lldpXMedRemXPoEPSEPowerPriority (1.0.8802.1.1.2.1.5.4795.1.3.6.1.3) | read-only | No | As per MIB |

lldpXMedRemXPoEPDTable

OID of this table is: 1.0.8802.1.1.2.1.5.4795.1.3.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpXMedRemXPoEPDPowerReq (1.0.8802.1.1.2.1.5.4795.1.3.7.1.1) | read-only | No | As per MIB |
| lldpXMedRemXPoEPDPowerSource (1.0.8802.1.1.2.1.5.4795.1.3.7.1.2) | read-only | No | As per MIB |
| lldpXMedRemXPoEPDPowerPriority (1.0.8802.1.1.2.1.5.4795.1.3.7.1.3) | read-only | No | As per MIB |