# Contents

LLDP-MIB	2

Scalar Objects	2

lldpPortConfigTable	2

lldpConfigManAddrTable	3

lldpStatsTxPortTable	3

lldpStatsRxPortTable	3

lldpLocPortTable	4

lldpLocManAddrTable	4

lldpRemTable	4

lldpRemManAddrTable	5

lldpRemUnknownTLVTable	5

lldpRemOrgDefInfoTable	6

LLDP-MIB

Management Information Base module for LLDP configuration statistics, local system data and remote systems data components.

Scalar Objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpMessageTxInterval (1.0.8802.1.1.2.1.1.1) | read-write | Current | As per MIB |
| lldpMessageTxHoldMultiplier (1.0.8802.1.1.2.1.1.2) | read-write | Current | As per MIB |
| lldpReinitDelay (1.0.8802.1.1.2.1.1.3) | read-write | Current | As per MIB |
| lldpTxDelay (1.0.8802.1.1.2.1.1.4) | read-only | No | Not supported.The value is always 2. |
| lldpNotificationInterval (1.0.8802.1.1.2.1.1.5) | read-write | Current | The default value is 30. |
| lldpStatsRemTablesLastChangeTime (1.0.8802.1.1.2.1.2.1) | read-only | No | As per MIB |
| lldpStatsRemTablesInserts (1.0.8802.1.1.2.1.2.2) | read-only | No | As per MIB |
| lldpStatsRemTablesDeletes (1.0.8802.1.1.2.1.2.3) | read-only | No | As per MIB |
| lldpStatsRemTablesDrops (1.0.8802.1.1.2.1.2.4) | read-only | No | As per MIB |
| lldpStatsRemTablesAgeouts (1.0.8802.1.1.2.1.2.5) | read-only | No | As per MIB |
| lldpLocChassisIdSubtype (1.0.8802.1.1.2.1.3.1) | read-only | Current | As per MIB |
| lldpLocChassisId (1.0.8802.1.1.2.1.3.2) | read-only | Current | As per MIB |
| lldpLocSysName (1.0.8802.1.1.2.1.3.3) | read-only | Current | As per MIB |
| lldpLocSysDesc (1.0.8802.1.1.2.1.3.4) | read-only | Current | As per MIB |
| lldpLocSysCapSupported (1.0.8802.1.1.2.1.3.5) | read-only | Current | As per MIB |
| lldpLocSysCapEnabled (1.0.8802.1.1.2.1.3.6) | read-only | Current | As per MIB |

lldpPortConfigTable

OID of this table is: 1.0.8802.1.1.2.1.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpPortConfigPortNum (1.0.8802.1.1.2.1.1.6.1.1) | not-accessible | Current | As per MIB |
| lldpPortConfigAdminStatus (1.0.8802.1.1.2.1.1.6.1.2) | read-write | Current | As per MIB |
| lldpPortConfigNotificationEnable (1.0.8802.1.1.2.1.1.6.1.3) | read-write | Current | As per MIB |
| lldpPortConfigTLVsTxEnable (1.0.8802.1.1.2.1.1.6.1.4) | read-write | Current | Every bit will be set to ‘1’ by default, which means all LLDP basic operational TLVs will be transmitted on the given port. |

lldpConfigManAddrTable

OID of this table is: 1.0.8802.1.1.2.1.1.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpConfigManAddrPortsTxEnable (1.0.8802.1.1.2.1.1.7.1.1) | read-write | Current | Each port of the system is represented by a single bit within the value of this object. Every bit will be set to ‘1’ by default, which means all ports are specified for advertising indicated management address instance. |

lldpStatsTxPortTable

OID of this table is: 1.0.8802.1.1.2.1.2.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpStatsTxPortNum (1.0.8802.1.1.2.1.2.6.1.1) | not-accessible | Current | As per MIB |
| lldpStatsTxPortFramesTotal (1.0.8802.1.1.2.1.2.6.1.2) | read-only | No | As per MIB |

lldpStatsRxPortTable

OID of this table is: 1.0.8802.1.1.2.1.2.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpStatsRxPortNum (1.0.8802.1.1.2.1.2.7.1.1) | not-accessible | Current | As per MIB |
| lldpStatsRxPortFramesDiscardedTotal (1.0.8802.1.1.2.1.2.7.1.2) | read-only | No | As per MIB |
| lldpStatsRxPortFramesErrors (1.0.8802.1.1.2.1.2.7.1.3) | read-only | No | As per MIB |
| lldpStatsRxPortFramesTotal (1.0.8802.1.1.2.1.2.7.1.4) | read-only | No | As per MIB |
| lldpStatsRxPortTLVsDiscardedTotal (1.0.8802.1.1.2.1.2.7.1.5) | read-only | No | As per MIB |
| lldpStatsRxPortTLVsUnrecognizedTotal (1.0.8802.1.1.2.1.2.7.1.6) | read-only | No | As per MIB |
| lldpStatsRxPortAgeoutsTotal (1.0.8802.1.1.2.1.2.7.1.7) | read-only | No | As per MIB |

lldpLocPortTable

OID of this table is: 1.0.8802.1.1.2.1.3.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpLocPortNum (1.0.8802.1.1.2.1.3.7.1.1) | not-accessible | Current | As per MIB |
| lldpLocPortIdSubtype (1.0.8802.1.1.2.1.3.7.1.2) | read-only | Current | As per MIB |
| lldpLocPortId (1.0.8802.1.1.2.1.3.7.1.3) | read-only | Current | If the port has MED neighbor, the PortID will be MAC address; If it has no MED neighbor, the PortID will be interface name. |
| lldpLocPortDesc (1.0.8802.1.1.2.1.3.7.1.4) | read-only | Current | As per MIB |

lldpLocManAddrTable

OID of this table is: 1.0.8802.1.1.2.1.3.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpLocManAddrSubtype (1.0.8802.1.1.2.1.3.8.1.1) | not-accessible | Current | As per MIB |
| lldpLocManAddr (1.0.8802.1.1.2.1.3.8.1.2) | not-accessible | Current | As per MIB |
| lldpLocManAddrLen (1.0.8802.1.1.2.1.3.8.1.3) | read-only | Current | As per MIB |
| lldpLocManAddrIfSubtype (1.0.8802.1.1.2.1.3.8.1.4) | read-only | Current | The subtype is IfIndex. |
| lldpLocManAddrIfId (1.0.8802.1.1.2.1.3.8.1.5) | read-only | Current | IfIndex of the interface associated with the management address. When the interface associated with the management address does not exist, this object will be zero. |
| lldpLocManAddrOID (1.0.8802.1.1.2.1.3.8.1.6) | read-only | Current | Not supported |

lldpRemTable

OID of this table is: 1.0.8802.1.1.2.1.4.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpRemTimeMark (1.0.8802.1.1.2.1.4.1.1.1) | not-accessible | No | This object identifies the time when the remote system information was created or updated. Network management can look up the history of remote system information with this object. |
| lldpRemLocalPortNum (1.0.8802.1.1.2.1.4.1.1.2) | not-accessible | No | As per MIB |
| lldpRemIndex (1.0.8802.1.1.2.1.4.1.1.3) | not-accessible | No | As per MIB |
| lldpRemChassisIdSubtype (1.0.8802.1.1.2.1.4.1.1.4) | read-only | No | As per MIB |
| lldpRemChassisId (1.0.8802.1.1.2.1.4.1.1.5) | read-only | No | As per MIB |
| lldpRemPortIdSubtype (1.0.8802.1.1.2.1.4.1.1.6) | read-only | No | As per MIB |
| lldpRemPortId (1.0.8802.1.1.2.1.4.1.1.7) | read-only | No | As per MIB |
| lldpRemPortDesc (1.0.8802.1.1.2.1.4.1.1.8) | read-only | No | As per MIB |
| lldpRemSysName (1.0.8802.1.1.2.1.4.1.1.9) | read-only | No | As per MIB |
| lldpRemSysDesc (1.0.8802.1.1.2.1.4.1.1.10) | read-only | No | As per MIB |
| lldpRemSysCapSupported (1.0.8802.1.1.2.1.4.1.1.11) | read-only | No | As per MIB |
| lldpRemSysCapEnabled (1.0.8802.1.1.2.1.4.1.1.12) | read-only | No | As per MIB |

lldpRemManAddrTable

OID of this table is: 1.0.8802.1.1.2.1.4.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpRemManAddrSubtype (1.0.8802.1.1.2.1.4.2.1.1) | not-accessible | No | As per MIB |
| lldpRemManAddr (1.0.8802.1.1.2.1.4.2.1.2) | not-accessible | No | As per MIB |
| lldpRemManAddrIfSubtype (1.0.8802.1.1.2.1.4.2.1.3) | read-only | No | As per MIB |
| lldpRemManAddrIfId (1.0.8802.1.1.2.1.4.2.1.4) | read-only | No | As per MIB |
| lldpRemManAddrOID (1.0.8802.1.1.2.1.4.2.1.5) | read-only | No | Not supported |

lldpRemUnknownTLVTable

OID of this table is: 1.0.8802.1.1.2.1.4.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpRemUnknownTLVType (1.0.8802.1.1.2.1.4.3.1.1) | not-accessible | No | As per MIB |
| lldpRemUnknownTLVInfo (1.0.8802.1.1.2.1.4.3.1.2) | read-only | No | As per MIB |

lldpRemOrgDefInfoTable

OID of this table is: 1.0.8802.1.1.2.1.4.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| lldpRemOrgDefInfoOUI (1.0.8802.1.1.2.1.4.4.1.1) | not-accessible | No | As per MIB |
| lldpRemOrgDefInfoSubtype (1.0.8802.1.1.2.1.4.4.1.2) | not-accessible | No | As per MIB |
| lldpRemOrgDefInfoIndex (1.0.8802.1.1.2.1.4.4.1.3) | not-accessible | No | As per MIB |
| lldpRemOrgDefInfo (1.0.8802.1.1.2.1.4.4.1.4) | read-only | No | As per MIB |