# Contents

MGMD-STD-MIB	2

mgmdRouterInterfaceTable	2

mgmdRouterCacheTable	4

mgmdInverseRouterCacheTable	4

mgmdRouterSrcListTable	5

MGMD-STD-MIB

This Multicast Group Membership Discovery (MGMD) MIB module contains eight tables:

(1)  the MGMD Host Interface Table, which contains one row for each interface on which IGMP or MLD is enabled on a host.

(2)  the MGMD Router Interface Table, which contains one row for each interface on which MGMD is enabled on a router.

(3)  the MGMD Host Cache Table, which contains one row for each IP multicast group for which there are members on a particular interface on a host.

(4)  the MGMD Router Cache Table, which contains one row for each IP multicast group for which there are members on a particular interface on a router.

(5)  the reverse MGMD Host Table, which contains one row for each interface for which there are active multicast groups on a host.

(6)  the reverse MGMD Router Table, which contains one row for each interface for which there are active multicast groups on a router.

(7)  the MGMD HostSrcList Table, which contains one row for each entry in the source filter record for an interface and multicast group pair on a host.

(8)  the MGMD RouterSrcList Table, which contains one row for each entry in the source filter record for an interface and multicast group pair on a router.

All tables are intended for EITHER router OR host functionality as indicated by the name and corresponding description, although it is anticipated that there will be scenarios where both terms might apply to a device, e.g., a router that joins a multicast group also as a host for measurement purposes.  The source list tables provide an extension to the cache tables to indicate the source-specific includes or excludes associated with each IP multicast group on each specific interface.  This functionality is only supported in IGMPv3 and MLDv2-capable nodes.

Incorporated within the MGMD MIB tables are objects for the management of IGMP and MLD proxy devices as described in RFC 4605 [RFC4605].  Proxy devices can be used in simple topologies where it is not necessary to run a full multicast routing protocol.  A proxy device can make forwarding decisions based on IGMP or MLD group membership activity.

The MIB references InterfaceIndex and InterfaceIndexOrZero objects as defined in RFC 2863 [RFC2863], the MIB that describes generic objects for network interface sub-layers.

Extensive references to the InetAddress and InetAddressType objects are made as defined in RFC 4001 [RFC4001].

mgmdRouterInterfaceTable

OID of this table is: 1.3.6.1.2.1.185.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mgmdRouterInterfaceIfIndex (1.3.6.1.2.1.185.1.2.1.1) | not-accessible | Current | As per MIB |
| mgmdRouterInterfaceQuerierType (1.3.6.1.2.1.185.1.2.1.2) | not-accessible | Current | As per MIB |
| mgmdRouterInterfaceQuerier (1.3.6.1.2.1.185.1.2.1.3) | read-only | No | As per MIB |
| mgmdRouterInterfaceQueryInterval (1.3.6.1.2.1.185.1.2.1.4) | read-create | Current | As per MIB |
| mgmdRouterInterfaceStatus (1.3.6.1.2.1.185.1.2.1.5) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |
| mgmdRouterInterfaceVersion (1.3.6.1.2.1.185.1.2.1.6) | read-create | Current | Default value is 2 |
| mgmdRouterInterfaceQueryMaxResponseTime (1.3.6.1.2.1.185.1.2.1.7) | read-create | Current | Range from 10 to 31740 |
| mgmdRouterInterfaceQuerierUpTime (1.3.6.1.2.1.185.1.2.1.8) | read-only | No | As per MIB |
| mgmdRouterInterfaceQuerierExpiryTime (1.3.6.1.2.1.185.1.2.1.9) | read-only | No | As per MIB |
| mgmdRouterInterfaceWrongVersionQueries (1.3.6.1.2.1.185.1.2.1.10) | read-only | No | Not supported |
| mgmdRouterInterfaceJoins (1.3.6.1.2.1.185.1.2.1.11) | read-only | No | As per MIB |
| mgmdRouterInterfaceProxyIfIndex (1.3.6.1.2.1.185.1.2.1.12) | read-create | Current | Only support read operation |
| mgmdRouterInterfaceGroups (1.3.6.1.2.1.185.1.2.1.13) | read-only | No | As per MIB |
| mgmdRouterInterfaceRobustness (1.3.6.1.2.1.185.1.2.1.14) | read-create | Current | As per MIB |
| mgmdRouterInterfaceLastMemberQueryInterval (1.3.6.1.2.1.185.1.2.1.15) | read-create | Current | Range from 10 to 250 |
| mgmdRouterInterfaceLastMemberQueryCount (1.3.6.1.2.1.185.1.2.1.16) | read-only | Current | As per MIB |
| mgmdRouterInterfaceStartupQueryCount (1.3.6.1.2.1.185.1.2.1.17) | read-only | Current | As per MIB |
| mgmdRouterInterfaceStartupQueryInterval (1.3.6.1.2.1.185.1.2.1.18) | read-only | Current | Range from 1 to 31744 |

mgmdRouterCacheTable

OID of this table is: 1.3.6.1.2.1.185.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mgmdRouterCacheAddressType (1.3.6.1.2.1.185.1.4.1.1) | not-accessible | No | As per MIB |
| mgmdRouterCacheAddress (1.3.6.1.2.1.185.1.4.1.2) | not-accessible | No | As per MIB |
| mgmdRouterCacheIfIndex (1.3.6.1.2.1.185.1.4.1.3) | not-accessible | No | As per MIB |
| mgmdRouterCacheLastReporter (1.3.6.1.2.1.185.1.4.1.4) | read-only | No | As per MIB |
| mgmdRouterCacheUpTime (1.3.6.1.2.1.185.1.4.1.5) | read-only | No | As per MIB |
| mgmdRouterCacheExpiryTime (1.3.6.1.2.1.185.1.4.1.6) | read-only | No | As per MIB |
| mgmdRouterCacheExcludeModeExpiryTimer (1.3.6.1.2.1.185.1.4.1.7) | read-only | No | As per MIB |
| mgmdRouterCacheVersion1HostTimer (1.3.6.1.2.1.185.1.4.1.8) | read-only | No | As per MIB |
| mgmdRouterCacheVersion2HostTimer (1.3.6.1.2.1.185.1.4.1.9) | read-only | No | As per MIB |
| mgmdRouterCacheSourceFilterMode (1.3.6.1.2.1.185.1.4.1.10) | read-only | No | As per MIB |

mgmdInverseRouterCacheTable

OID of this table is: 1.3.6.1.2.1.185.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mgmdInverseRouterCacheIfIndex (1.3.6.1.2.1.185.1.6.1.1) | not-accessible | No | As per MIB |
| mgmdInverseRouterCacheAddressType (1.3.6.1.2.1.185.1.6.1.2) | not-accessible | No | As per MIB |
| mgmdInverseRouterCacheAddress (1.3.6.1.2.1.185.1.6.1.3) | read-only | No | As per MIB |

mgmdRouterSrcListTable

OID of this table is: 1.3.6.1.2.1.185.1.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mgmdRouterSrcListAddressType (1.3.6.1.2.1.185.1.8.1.1) | not-accessible | No | As per MIB |
| mgmdRouterSrcListAddress (1.3.6.1.2.1.185.1.8.1.2) | not-accessible | No | As per MIB |
| mgmdRouterSrcListIfIndex (1.3.6.1.2.1.185.1.8.1.3) | not-accessible | No | As per MIB |
| mgmdRouterSrcListHostAddress (1.3.6.1.2.1.185.1.8.1.4) | not-accessible | No | As per MIB |
| mgmdRouterSrcListExpire (1.3.6.1.2.1.185.1.8.1.5) | read-only | No | As per MIB |