# Contents

MPLS-FRR-GENERAL-STD-MIB	2

scalar objects of mplsFrrGeneralObjects	2

mplsFrrGeneralConstraintsTable	2

mplsFrrGeneralTunnelARHopTable	3

MPLS-FRR-GENERAL-STD-MIB

scalar objects of mplsFrrGeneralObjects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsFrrGeneralProtectionMethod (1.3.6.1.2.1.202.1.1) | read-write | Current | Only support read operation, Only support facilityBackup(3) |
| mplsFrrGeneralIngressTunnelInstances (1.3.6.1.2.1.204.1.2) | read-only | Current | Only support the number of bypass tunnels |

mplsFrrGeneralConstraintsTable

OID of this table is: 1.3.6.1.2.1.202.1.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsFrrGeneralConstraintsIfIndexOrZero (1.3.6.1.2.1.202.1.3.1.1) | not-accessible | No | As per MIB |
| mplsFrrGeneralConstraintsTunnelIndex (1.3.6.1.2.1.202.1.3.1.2) | not-accessible | No | As per MIB |
| mplsFrrGeneralConstraintsTunnelInstance (1.3.6.1.2.1.202.1.3.1.3) | not-accessible | No | Only support 0 when operation is set. |
| mplsFrrGeneralConstraintsProtectionType (1.3.6.1.2.1.202.1.3.1.4) | read-create | Current | Only support read operation, Only support nodeProtection(2) |
| mplsFrrGeneralConstraintsSetupPrio (1.3.6.1.2.1.202.1.3.1.5) | read-create | Current | Only support read operation |
| mplsFrrGeneralConstraintsHoldingPrio (1.3.6.1.2.1.202.1.3.1.6) | read-create | Current | Only support read operation |
| mplsFrrGeneralConstraintsInclAnyAffinity (1.3.6.1.2.1.202.1.3.1.7) | read-create | Current | Only support read operation |
| mplsFrrGeneralConstraintsInclAllAffinity (1.3.6.1.2.1.202.1.3.1.8) | read-create | Current | Only support read operation |
| mplsFrrGeneralConstraintsExclAnyAffinity (1.3.6.1.2.1.202.1.3.1.9) | read-create | Current | Only support read operation |
| mplsFrrGeneralConstraintsHopLimit (1.3.6.1.2.1.202.1.3.1.10) | read-create | Current | Not supported. The value is always 255. |
| mplsFrrGeneralConstraintsBandwidth (1.3.6.1.2.1.202.1.3.1.11) | read-create | Current | Only support CT0 |
| mplsFrrGeneralConstraintsStorageType (1.3.6.1.2.1.202.1.3.1.12) | read-create | Current | Not supported. The value is always nonvolatile(3). |
| mplsFrrGeneralConstraintsRowStatus (1.3.6.1.2.1.202.1.3.1.13) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |

mplsFrrGeneralTunnelARHopTable

OID of this table is: 1.3.6.1.2.1.202.1.4.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsFrrGeneralTunnelARHopSessionAttributeFlags (1.3.6.1.2.1.202.1.4.1.1) | read-only | Current | As per MIB |
| mplsFrrGeneralTunnelARHopRROSubObjectFlags (1.3.6.1.2.1.202.1.4.1.2) | read-only | Current | As per MIB |