# Contents

MPLS-LDP-STD-MIB	2

Scalar objects of mplsLdpLsrObjects	2

Scalar objects of mplsLdpEntityObjects	2

mplsLdpEntityTable	2

mplsLdpEntityStatsTable	3

Scalar objects of mplsLdpSessionObjects	4

mplsLdpPeerTable	5

mplsLdpSessionTable	5

mplsLdpSessionStatsTable	5

mplsLdpHelloAdjacencyTable	6

Scalar objects of mplsFecObjects	6

mplsFecTable	6

mplsLdpSessionPeerAddrTable	7

MPLS-LDP-STD-MIB

Scalar objects of mplsLdpLsrObjects

OID of this table is: 1.3.6.1.2.1.10.166.4.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpLsrId (1.3.6.1.2.1.10.166.4.1.1.1) | read-only | Current | As per MIB |
| mplsLdpLsrLoopDetectionCapable (1.3.6.1.2.1.10.166.4.1.1.2) | read-only | Current | The value is always hopCountAndPathVector(5) |

Scalar objects of mplsLdpEntityObjects

OID of this table is: 1.3.6.1.2.1.10.166.4.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpEntityLastChange (1.3.6.1.2.1.10.166.4.1.2.1) | read-only | No | As per MIB |
| mplsLdpEntityIndexNext (1.3.6.1.2.1.10.166.4.1.2.2) | read-only | Yes | The value is always 0 |

mplsLdpEntityTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.2.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpEntityLdpId (1.3.6.1.2.1.10.166.4.1.2.3.1.1) | not-accessible | Current | As per MIB |
| mplsLdpEntityIndex (1.3.6.1.2.1.10.166.4.1.2.3.1.2) | not-accessible | No | As per MIB |
| mplsLdpEntityProtocolVersion (1.3.6.1.2.1.10.166.4.1.2.3.1.3) | read-create | Current | Only support read operation. The value is always 1 |
| mplsLdpEntityAdminStatus (1.3.6.1.2.1.10.166.4.1.2.3.1.4) | read-create | Current | Only support read operation. The value is always enable(1) |
| mplsLdpEntityOperStatus (1.3.6.1.2.1.10.166.4.1.2.3.1.5) | read-only | Current | Only support enabled(2) and disabled(3) |
| mplsLdpEntityTcpPort (1.3.6.1.2.1.10.166.4.1.2.3.1.6) | read-create | Current | Only support read operation. The value is always 646 |
| mplsLdpEntityUdpDscPort (1.3.6.1.2.1.10.166.4.1.2.3.1.7) | read-create | Current | Only support read operation. The value is always 646 |
| mplsLdpEntityMaxPduLength (1.3.6.1.2.1.10.166.4.1.2.3.1.8) | read-create | Current | Only support read operation. The value is always 4096 |
| mplsLdpEntityKeepAliveHoldTimer (1.3.6.1.2.1.10.166.4.1.2.3.1.9) | read-create | Current | Only support read operation. The default value is 45. Range from 15 to 65535 |
| mplsLdpEntityHelloHoldTimer (1.3.6.1.2.1.10.166.4.1.2.3.1.10) | read-create | Current | Only support read operation. |
| mplsLdpEntityInitSessionThreshold (1.3.6.1.2.1.10.166.4.1.2.3.1.11) | read-create | Current | Only support read operation. The value is always 8 |
| mplsLdpEntityLabelDistMethod (1.3.6.1.2.1.10.166.4.1.2.3.1.12) | read-create | Current | Only support read operation The value is always downstreamUnsolicited(2) |
| mplsLdpEntityLabelRetentionMode (1.3.6.1.2.1.10.166.4.1.2.3.1.13) | read-create | Current | Only support read operation The value is always liberal(2) |
| mplsLdpEntityPathVectorLimit (1.3.6.1.2.1.10.166.4.1.2.3.1.14) | read-create | Current | Only support read operation. The default value is 0. Range from 0 to 32 |
| mplsLdpEntityHopCountLimit (1.3.6.1.2.1.10.166.4.1.2.3.1.15) | read-create | Current | Only support read operation. The default value is 0 Range from 0 to 32 |
| mplsLdpEntityTransportAddrKind (1.3.6.1.2.1.10.166.4.1.2.3.1.16) | read-create | Current | Only support read operation. The default value is loopback(2) |
| mplsLdpEntityTargetPeer (1.3.6.1.2.1.10.166.4.1.2.3.1.17) | read-create | Current | Only support read operation. The default value is false |
| mplsLdpEntityTargetPeerAddrType (1.3.6.1.2.1.10.166.4.1.2.3.1.18) | read-create | Current | Only support read operation. The value is always IPV4 |
| mplsLdpEntityTargetPeerAddr (1.3.6.1.2.1.10.166.4.1.2.3.1.19) | read-create | Current | Only support read operation |
| mplsLdpEntityLabelType (1.3.6.1.2.1.10.166.4.1.2.3.1.20) | read-create | Current | Only support read operation The value is always generic(1) |
| mplsLdpEntityDiscontinuityTime (1.3.6.1.2.1.10.166.4.1.2.3.1.21) | read-only | No | As per MIB |
| mplsLdpEntityStorageType (1.3.6.1.2.1.10.166.4.1.2.3.1.22) | read-create | Current | Only support read operation. The value is always nonVolatile(3) |
| mplsLdpEntityRowStatus (1.3.6.1.2.1.10.166.4.1.2.3.1.23) | read-create | Current | Only support read operation Only support active(1) |

mplsLdpEntityStatsTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.2.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpEntityStatsSessionAttempts (1.3.6.1.2.1.10.166.4.1.2.4.1.1) | read-only | No | As per MIB |
| mplsLdpEntityStatsSessionRejectedNoHelloErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.2) | read-only | No | As per MIB |
| mplsLdpEntityStatsSessionRejectedAdErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.3) | read-only | No | As per MIB |
| mplsLdpEntityStatsSessionRejectedMaxPduErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.4) | read-only | No | As per MIB |
| mplsLdpEntityStatsSessionRejectedLRErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.5) | read-only | No | As per MIB |
| mplsLdpEntityStatsBadLdpIdentifierErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.6) | read-only | No | As per MIB |
| mplsLdpEntityStatsBadPduLengthErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.7) | read-only | No | As per MIB |
| mplsLdpEntityStatsBadMessageLengthErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.8) | read-only | No | As per MIB |
| mplsLdpEntityStatsBadTlvLengthErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.9) | read-only | No | As per MIB |
| mplsLdpEntityStatsMalformedTlvValueErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.10) | read-only | No | As per MIB |
| mplsLdpEntityStatsKeepAliveTimerExpErrors (1.3.6.1.2.1.10.166.4.1.2.4.1.11) | read-only | No | As per MIB |
| mplsLdpEntityStatsShutdownReceivedNotifications (1.3.6.1.2.1.10.166.4.1.2.4.1.12) | read-only | No | As per MIB |
| mplsLdpEntityStatsShutdownSentNotifications (1.3.6.1.2.1.10.166.4.1.2.4.1.13) | read-only | No | As per MIB |

Scalar objects of mplsLdpSessionObjects

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpPeerLastChange (1.3.6.1.2.1.10.166.4.1.3.1) | read-only | No | As per MIB |

mplsLdpPeerTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpPeerLdpId (1.3.6.1.2.1.10.166.4.1.3.2.1.1) | not-accessible | Current | As per MIB |
| mplsLdpPeerLabelDistMethod (1.3.6.1.2.1.10.166.4.1.3.2.1.2) | read-only | Current | As per MIB |
| mplsLdpPeerPathVectorLimit (1.3.6.1.2.1.10.166.4.1.3.2.1.3) | read-only | Current | As per MIB |
| mplsLdpPeerTransportAddrType (1.3.6.1.2.1.10.166.4.1.3.2.1.4) | read-only | Current | The value is always IPV4 |
| mplsLdpPeerTransportAddr (1.3.6.1.2.1.10.166.4.1.3.2.1.5) | read-only | Current | As per MIB |

mplsLdpSessionTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpSessionStateLastChange (1.3.6.1.2.1.10.166.4.1.3.3.1.1) | read-only | No | As per MIB |
| mplsLdpSessionState (1.3.6.1.2.1.10.166.4.1.3.3.1.2) | read-only | Current | As per MIB |
| mplsLdpSessionRole (1.3.6.1.2.1.10.166.4.1.3.3.1.3) | read-only | Current | As per MIB |
| mplsLdpSessionProtocolVersion (1.3.6.1.2.1.10.166.4.1.3.3.1.4) | read-only | Current | As per MIB |
| mplsLdpSessionKeepAliveHoldTimeRem (1.3.6.1.2.1.10.166.4.1.3.3.1.5) | read-only | No | As per MIB |
| mplsLdpSessionKeepAliveTime (1.3.6.1.2.1.10.166.4.1.3.3.1.6) | read-only | Current | As per MIB |
| mplsLdpSessionMaxPduLength (1.3.6.1.2.1.10.166.4.1.3.3.1.7) | read-only | Current | As per MIB |
| mplsLdpSessionDiscontinuityTime (1.3.6.1.2.1.10.166.4.1.3.3.1.8) | read-only | No | As per MIB |

mplsLdpSessionStatsTable

OID of this table is:  1.3.6.1.2.1.10.166.4.1.3.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpSessionStatsUnknownMesTypeErrors (1.3.6.1.2.1.10.166.4.1.3.4.1.1) | read-only | No | As per MIB |
| mplsLdpSessionStatsUnknownTlvErrors (1.3.6.1.2.1.10.166.4.1.3.4.1.2) | read-only | No | As per MIB |

mplsLdpHelloAdjacencyTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3.5.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpHelloAdjacencyIndex (1.3.6.1.2.1.10.166.4.1.3.5.1.1.1) | not-accessible | No | As per MIB |
| mplsLdpHelloAdjacencyHoldTimeRem (1.3.6.1.2.1.10.166.4.1.3.5.1.1.2) | read-only | No | As per MIB |
| mplsLdpHelloAdjacencyHoldTime (1.3.6.1.2.1.10.166.4.1.3.5.1.1.3) | read-only | Current | As per MIB |
| mplsLdpHelloAdjacencyType (1.3.6.1.2.1.10.166.4.1.3.5.1.1.4) | read-only | Current | As per MIB |

Scalar objects of mplsFecObjects

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsFecLastChange (1.3.6.1.2.1.10.166.4.1.3.8.1) | read-only | Current | Not supported |
| mplsFecIndexNext (1.3.6.1.2.1.10.166.4.1.3.8.2) | read-only | Yes | The value is always 0 |

mplsFecTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3.8.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsFecIndex (1.3.6.1.2.1.10.166.4.1.3.8.3.1.1) | not-accessible | No | As per MIB |
| MplsFecType (1.3.6.1.2.1.10.166.4.1.3.8.3.1.2) | read-create | No | Only support read operation. |
| mplsFecAddrPrefixLength (1.3.6.1.2.1.10.166.4.1.3.8.3.1.3) | read-create | No | Only support read operation. |
| mplsFecAddrType (1.3.6.1.2.1.10.166.4.1.3.8.3.1.4) | read-create | No | Only support read operation. The value is always IPV4 |
| mplsFecAddr (1.3.6.1.2.1.10.166.4.1.3.8.3.1.5) | read-create | No | Only support read operation. |
| mplsFecStorageType (1.3.6.1.2.1.10.166.4.1.3.8.3.1.6) | read-create | No | Only support read operation. The value is always volatile(2) |
| mplsFecRowStatus (1.3.6.1.2.1.10.166.4.1.3.8.3.1.7) | read-create | No | Only support read operation. Only support active(1) |

mplsLdpSessionPeerAddrTable

OID of this table is: 1.3.6.1.2.1.10.166.4.1.3.11

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLdpSessionPeerAddrIndex (1.3.6.1.2.1.10.166.4.1.3.11.1.1) | not-accessible | No | As per MIB |
| mplsLdpSessionPeerNextHopAddrType (1.3.6.1.2.1.10.166.4.1.3.11.1.2) | read-only | Current | The value is always IPV4 |
| mplsLdpSessionPeerNextHopAddr (1.3.6.1.2.1.10.166.4.1.3.11.1.3) | read-only | Current | As per MIB |