# Contents

MPLS-LSR-STD-MIB	2

scalar objects of mplsLsrObjects	2

mplsInterfaceTable	2

mplsInterfacePerfTable	3

mplsInSegmentTable	3

mplsInSegmentPerfTable	4

mplsOutSegmentTable	4

mplsOutSegmentPerfTable	5

mplsXCTable	5

mplsLabelStackTable	7

mplsInSegmentMapTable	8

MPLS-LSR-STD-MIB

MPLS-LSR-STD-MIB contains managed object definitions for	the Multiprotocol Label Switching (MPLS) Router as defined in RFC 3031.

scalar objects of mplsLsrObjects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsInSegmentIndexNext (1.3.6.1.2.1.10.166.2.1.3) | read-only | No | As per MIB |
| mplsOutSegmentIndexNext (1.3.6.1.2.1.10.166.2.1.6) | read-only | No | As per MIB |
| mplsXCIndexNext (1.3.6.1.2.1.10.166.2.1.9) | read-only | No | As per MIB |
| mplsMaxLabelStackDepth (1.3.6.1.2.1.10.166.2.1.11) | read-only | Yes | As per MIB |
| mplsLabelStackIndexNext (1.3.6.1.2.1.10.166.2.1.12) | read-only | Yes | Not supported |
| mplsXCNotificationsEnable (1.3.6.1.2.1.10.166.2.1.15) | read-write | Current | As per MIB |

mplsInterfaceTable

OID of this table is: 1.3.6.1.2.1.10.166.2.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsInterfaceIndex (1.3.6.1.2.1.10.166.2.1.1.1.1) | not-accessible | No | As per MIB |
| mplsInterfaceLabelMinIn (1.3.6.1.2.1.10.166.2.1.1.1.2) | read-only | Current | As per MIB |
| mplsInterfaceLabelMaxIn (1.3.6.1.2.1.10.166.2.1.1.1.3) | read-only | Current | As per MIB |
| mplsInterfaceLabelMinOut (1.3.6.1.2.1.10.166.2.1.1.1.4) | read-only | Current | As per MIB |
| mplsInterfaceLabelMaxOut (1.3.6.1.2.1.10.166.2.1.1.1.5) | read-only | Current | As per MIB |
| mplsInterfaceTotalBandwidth (1.3.6.1.2.1.10.166.2.1.1.1.6) | read-only | Current | As per MIB Only suitable for TE |
| mplsInterfaceAvailableBandwidth (1.3.6.1.2.1.10.166.2.1.1.1.7) | read-only | Current | As per MIB Only suitable for TE |
| mplsInterfaceLabelParticipationType (1.3.6.1.2.1.10.166.2.1.1.1.8) | read-only | Current | The value is always 0, which means perPlatform. |

mplsInterfacePerfTable

OID of this table is: 1.3.6.1.2.1.10.166.2.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsInterfacePerfInLabelsInUse (1.3.6.1.2.1.10.166.2.1.2.1.1) | read-only | No | As per MIB |
| mplsInterfacePerfInLabelLookupFailures (1.3.6.1.2.1.10.166.2.1.2.1.2) | read-only | No | As per MIB |
| mplsInterfacePerfOutLabelsInUse (1.3.6.1.2.1.10.166.2.1.2.1.3) | read-only | No | As per MIB |
| mplsInterfacePerfOutFragmentedPkts (1.3.6.1.2.1.10.166.2.1.2.1.4) | read-only | No | As per MIB |

mplsInSegmentTable

Note: When creating a new instance of this table, the following objects SHOULD be set simultaneously:

mplsInSegmentLabel, mplsInSegmentRowStatus.

OID of this table is: 1.3.6.1.2.1.10.166.2.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsInSegmentIndex (1.3.6.1.2.1.10.166.2.1.4.1.1) | not-accessible | No | A 5-byte octet string. The first byte is type, the other 4 bytes containing the in-label for type 1 or the position ID for type 0 |
| mplsInSegmentInterface (1.3.6.1.2.1.10.166.2.1.4.1.2) | read-create | No | Only support read operation |
| mplsInSegmentLabel (1.3.6.1.2.1.10.166.2.1.4.1.3) | read-create | No | For set operation, the value of this object is from 16 to maximum static label supported by the product |
| mplsInSegmentLabelPtr (1.3.6.1.2.1.10.166.2.1.4.1.4) | read-create | No | Only support read operation |
| mplsInSegmentNPop (1.3.6.1.2.1.10.166.2.1.4.1.5) | read-create | No | Only support read operation |
| mplsInSegmentAddrFamily (1.3.6.1.2.1.10.166.2.1.4.1.6) | read-create | No | Only support read operation |
| mplsInSegmentXCIndex (1.3.6.1.2.1.10.166.2.1.4.1.7) | read-only | No | As per MIB |
| mplsInSegmentOwner (1.3.6.1.2.1.10.166.2.1.4.1.8) | read-only | No | As per MIB |
| mplsInSegmentTrafficParamPtr (1.3.6.1.2.1.10.166.2.1.4.1.9) | read-create | No | Only suitable for TE |
| mplsInSegmentRowStatus (1.3.6.1.2.1.10.166.2.1.4.1.10) | read-create | No | Only support active(1), createAndGo(4) and destroy(6) |
| mplsInSegmentStorageType (1.3.6.1.2.1.10.166.2.1.4.1.11) | read-create | No | Only support read operation |

mplsInSegmentPerfTable

OID of this table is: 1.3.6.1.2.1.10.166.2.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsInSegmentPerfOctets (1.3.6.1.2.1.10.166.2.1.5.1.1) | read-only | No | As per MIB |
| mplsInSegmentPerfPackets (1.3.6.1.2.1.10.166.2.1.5.1.2) | read-only | No | As per MIB |
| mplsInSegmentPerfErrors (1.3.6.1.2.1.10.166.2.1.5.1.3) | read-only | No | As per MIB |
| mplsInSegmentPerfDiscards (1.3.6.1.2.1.10.166.2.1.5.1.4) | read-only | No | As per MIB |
| mplsInSegmentPerfHCOctets (1.3.6.1.2.1.10.166.2.1.5.1.5) | read-only | No | As per MIB |
| mplsInSegmentPerfDiscontinuityTime (1.3.6.1.2.1.10.166.2.1.5.1.6) | read-only | No | As per MIB |

mplsOutSegmentTable

Note: When creating a new instance of this table, the following objects SHOULD be set simultaneously:

mplsOutSegmentInterface, mplsOutSegmentLabel, mplsOutSegmentNextHopAddr, mplsOutSegmentRowStatus.

OID of this table is: 1.3.6.1.2.1.10.166.2.1.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsOutSegmentIndex (1.3.6.1.2.1.10.166.2.1.7.1.1) | not-accessible | No | The description of this index is identical to that of the mplsXCIndex. |
| mplsOutSegmentInterface (1.3.6.1.2.1.10.166.2.1.7.1.2) | read-create | No | As per MIB |
| mplsOutSegmentPushTopLabel (1.3.6.1.2.1.10.166.2.1.7.1.3) | read-create | No | Only support read operation |
| mplsOutSegmentTopLabel (1.3.6.1.2.1.10.166.2.1.7.1.4) | read-create | No | For set operation, the value of this object is 0,3,and range from 16 to maximum static label confirmed by products |
| mplsOutSegmentTopLabelPtr (1.3.6.1.2.1.10.166.2.1.7.1.5) | read-create | No | Only support read operation |
| mplsOutSegmentNextHopAddrType (1.3.6.1.2.1.10.166.2.1.7.1.6) | read-create | No | Only support read operation |
| mplsOutSegmentNextHopAddr (1.3.6.1.2.1.10.166.2.1.7.1.7) | read-create | No | As per MIB |
| mplsOutSegmentXCIndex (1.3.6.1.2.1.10.166.2.1.7.1.8) | read-only | No | As per MIB |
| mplsOutSegmentOwner (1.3.6.1.2.1.10.166.2.1.7.1.9) | read-only | No | As per MIB |
| mplsOutSegmentTrafficParamPtr (1.3.6.1.2.1.10.166.2.1.7.1.10) | read-create | No | Only suitable for TE |
| mplsOutSegmentRowStatus (1.3.6.1.2.1.10.166.2.1.7.1.11) | read-create | No | Only support active(1), createAndGo(4) and destroy(6) |
| mplsOutSegmentStorageType (1.3.6.1.2.1.10.166.2.1.7.1.12) | read-create | No | Only support read operation |

mplsOutSegmentPerfTable

OID of this table is: 1.3.6.1.2.1.10.166.2.1.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsOutSegmentPerfOctets (1.3.6.1.2.1.10.166.2.1.8.1.1) | read-only | No | As per MIB |
| mplsOutSegmentPerfPackets (1.3.6.1.2.1.10.166.2.1.8.1.2) | read-only | No | As per MIB |
| mplsOutSegmentPerfErrors (1.3.6.1.2.1.10.166.2.1.8.1.3) | read-only | No | As per MIB |
| mplsOutSegmentPerfDiscards (1.3.6.1.2.1.10.166.2.1.8.1.4) | read-only | No | As per MIB |
| mplsOutSegmentPerfHCOctets (1.3.6.1.2.1.10.166.2.1.8.1.5) | read-only | No | As per MIB |
| mplsOutSegmentPerfDiscontinuityTime (1.3.6.1.2.1.10.166.2.1.8.1.6) | read-only | No | As per MIB |

mplsXCTable

OID of this table is: 1.3.6.1.2.1.10.166.2.1.10

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsXCIndex (1.3.6.1.2.1.10.166.2.1.10.1.1) | not-accessible | No | A variable-length octet string, the size of which depends on the first byte. The first byte is type. See Notes following this table |
| mplsXCInSegmentIndex (1.3.6.1.2.1.10.166.2.1.10.1.2) | not-accessible | No | As per MIB |
| mplsXCOutSegmentIndex (1.3.6.1.2.1.10.166.2.1.10.1.3) | not-accessible | No | Be identical to mplsXCIndex |
| mplsXCLspId (1.3.6.1.2.1.10.166.2.1.10.1.4) | read-create | No | Only support read operation |
| mplsXCLabelStackIndex (1.3.6.1.2.1.10.166.2.1.10.1.5) | read-create | No | Not supported |
| mplsXCOwner (1.3.6.1.2.1.10.166.2.1.10.1.6) | read-only | No | As per MIB |
| mplsXCRowStatus (1.3.6.1.2.1.10.166.2.1.10.1.7) | read-create | No | Only support active(1), createAndGo(4) and destroy(6). |
| mplsXCStorageType (1.3.6.1.2.1.10.166.2.1.10.1.8) | read-create | No | Only support read operation |
| mplsXCAdminStatus (1.3.6.1.2.1.10.166.2.1.10.1.9) | read-create | No | Only support read operation |
| mplsXCOperStatus (1.3.6.1.2.1.10.166.2.1.10.1.10) | read-only | No | As per MIB |

Notes: mplsXCIndex structure:

.

Detailed information is shown below:

mplsLabelStackTable

The table is not supported.

OID of this table is: 1.3.6.1.2.1.10.166.2.1.13

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsLabelStackIndex (1.3.6.1.2.1.10.166.2.1.13.1.1) | not-accessible | No | As per MIB |
| mplsLabelStackLabelIndex (1.3.6.1.2.1.10.166.2.1.13.1.2) | not-accessible | No | As per MIB |
| mplsLabelStackLabel (1.3.6.1.2.1.10.166.2.1.13.1.3) | read-create | No | The value is always 0. |
| mplsLabelStackLabelPtr (1.3.6.1.2.1.10.166.2.1.13.1.4) | read-create | No | The value is always 0.0 |
| mplsLabelStackRowStatus (1.3.6.1.2.1.10.166.2.1.13.1.5) | read-create | No | The value is always 0. |
| mplsLabelStackStorageType (1.3.6.1.2.1.10.166.2.1.13.1.6) | read-create | No | The value is always 0. |

mplsInSegmentMapTable

OID of this table is: 1.3.6.1.2.1.10.166.2.1.14

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mplsInSegmentMapInterface (1.3.6.1.2.1.10.166.2.1.14.1.1) | not-accessible | No | As per MIB |
| mplsInSegmentMapLabel (1.3.6.1.2.1.10.166.2.1.14.1.2) | not-accessible | No | As per MIB |
| mplsInSegmentMapLabelPtrIndex (1.3.6.1.2.1.10.166.2.1.14.1.3) | not-accessible | No | As per MIB |
| mplsInSegmentMapIndex (1.3.6.1.2.1.10.166.2.1.14.1.4) | read-only | No | As per MIB |