# Contents

OSPFV3-MIB	2

ospfv3GeneralGroup	2

ospfv3AreaTable	4

ospfv3AsLsdbTable	5

ospfv3AreaLsdbTable	6

ospfv3LinkLsdbTable	7

ospfv3HostTable	8

ospfv3IfTable	8

ospfv3VirtIfTable	11

ospfv3NbrTable	12

ospfv3CfgNbrTable	13

ospfv3VirtNbrTable	14

ospfv3AreaAggregateTable	15

ospfv3VirtLinkLsdbTable	16

OSPFV3-MIB

ospfv3GeneralGroup

OID of this table is: 1.3.6.1.2.1.191.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3RouterId (1.3.6.1.2.1.191.1.1.1) | read-write | Current | Only support read operation |
| ospfv3AdminStatus (1.3.6.1.2.1.191.1.1.2) | read-write | No | Only support read operation |
| ospfv3VersionNumber (1.3.6.1.2.1.191.1.1.3) | read-only | Yes | As per MIB |
| ospfv3AreaBdrRtrStatus (1.3.6.1.2.1.191.1.1.4) | read-only | No | As per MIB |
| ospfv3ASBdrRtrStatus (1.3.6.1.2.1.191.1.1.5) | read-write | No | Only support read operation |
| ospfv3AsScopeLsaCount (1.3.6.1.2.1.191.1.1.6) | read-only | No | As per MIB |
| ospfv3AsScopeLsaCksumSum (1.3.6.1.2.1.191.1.1.7) | read-only | No | As per MIB |
| ospfv3OriginateNewLsas (1.3.6.1.2.1.191.1.1.8) | read-only | No | As per MIB |
| ospfv3RxNewLsas (1.3.6.1.2.1.191.1.1.9) | read-only | No | As per MIB |
| ospfv3ExtLsaCount (1.3.6.1.2.1.191.1.1.10) | read-only | No | As per MIB |
| ospfv3ExtAreaLsdbLimit (1.3.6.1.2.1.191.1.1.11) | read-write | Yes | Only support read operation |
| ospfv3ExitOverflowInterval (1.3.6.1.2.1.191.1.1.12) | read-write | Yes | Only support read operation |
| ospfv3DemandExtensions (1.3.6.1.2.1.191.1.1.13) | read-write | Yes | Only support read operation |
| ospfv3ReferenceBandwidth (1.3.6.1.2.1.191.1.1.14) | read-write | Current | The portion less than 1000 of the whole value will be ignored. |
| ospfv3RestartSupport (1.3.6.1.2.1.191.1.1.15) | read-write | Current | If value changes from 1 to 2 or 3, IETF GR is supported; if value changes between 2 and 3, the original GR type is not changed. |
| ospfv3RestartInterval (1.3.6.1.2.1.191.1.1.16) | read-write | Current | The value range is from 40 to 1800 |
| ospfv3RestartStrictLsaChecking (1.3.6.1.2.1.191.1.1.17) | read-write | Current | The default value is false(2). |
| ospfv3RestartStatus (1.3.6.1.2.1.191.1.1.18) | read-only | No | As per MIB |
| ospfv3RestartAge (1.3.6.1.2.1.191.1.1.19) | read-only | No | As per MIB |
| ospfv3RestartExitReason (1.3.6.1.2.1.191.1.1.20) | read-only | No | As per MIB |
| ospfv3NotificationEnable (1.3.6.1.2.1.191.1.1.21) | read-write | Current | If no OSPFv3 notification can be generated, the value of this object is false (1); Otherwise true (1). If this object is set to true (1), then it enables the generation of all OSPFv3 notifications. |
| ospfv3StubRouterSupport (1.3.6.1.2.1.191.1.1.22) | read-only | Yes | As per MIB |
| ospfv3StubRouterAdvertisement (1.3.6.1.2.1.191.1.1.23) | read-write | Current | As per MIB |
| ospfv3DiscontinuityTime (1.3.6.1.2.1.191.1.1.24) | read-only | No | As per MIB |
| ospfv3RestartTime (1.3.6.1.2.1.191.1.1.25) | read-only | No | As per MIB |

ospfv3AreaTable

OID of this table is: 1.3.6.1.2.1.191.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3AreaId (1.3.6.1.2.1.191.1.2.1.1) | not-accessible | No | As per MIB |
| ospfv3AreaImportAsExtern (1.3.6.1.2.1.191.1.2.1.2) | read-create | No | Only support read operation |
| ospfv3AreaSpfRuns (1.3.6.1.2.1.191.1.2.1.3) | read-only | No | As per MIB |
| ospfv3AreaBdrRtrCount (1.3.6.1.2.1.191.1.2.1.4) | read-only | No | As per MIB |
| ospfv3AreaAsBdrRtrCount (1.3.6.1.2.1.191.1.2.1.5) | read-only | No | As per MIB |
| ospfv3AreaScopeLsaCount (1.3.6.1.2.1.191.1.2.1.6) | read-only | No | As per MIB |
| ospfv3AreaScopeLsaCksumSum (1.3.6.1.2.1.191.1.2.1.7) | read-only | No | As per MIB |
| ospfv3AreaSummary (1.3.6.1.2.1.191.1.2.1.8) | read-create | Current | Only support read and write operations |
| ospfv3AreaRowStatus (1.3.6.1.2.1.191.1.2.1.9) | read-create | No | Only support read operation |
| ospfv3AreaStubMetric (1.3.6.1.2.1.191.1.2.1.10) | read-create | Current | Only support read and write operations. The default value is 1. The value range is from 0 to 16777214 |
| ospfv3AreaNssaTranslatorRole (1.3.6.1.2.1.191.1.2.1.11) | read-create | Current | Only support read and write operations |
| ospfv3AreaNssaTranslatorState (1.3.6.1.2.1.191.1.2.1.12) | read-only | No | As per MIB |
| ospfv3AreaNssaTranslatorStabInterval (1.3.6.1.2.1.191.1.2.1.13) | read-create | Current | Only support read and write operations. The default value is 0. The value range is from 0 to 900 |
| ospfv3AreaNssaTranslatorEvents (1.3.6.1.2.1.191.1.2.1.14) | read-only | No | As per MIB |
| ospfv3AreaStubMetricType (1.3.6.1.2.1.191.1.2.1.15) | read-create | Yes | Only support read operation |
| ospfv3AreaTEEnabled (1.3.6.1.2.1.191.1.2.1.16) | read-create | Yes | Only support read operation |

ospfv3AsLsdbTable

OID of this table is: 1.3.6.1.2.1.191.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3AsLsdbType (1.3.6.1.2.1.191.1.3.1.1) | not-accessible | No | As per MIB |
| ospfv3AsLsdbRouterId (1.3.6.1.2.1.191.1.3.1.2) | not-accessible | No | As per MIB |
| ospfv3AsLsdbLsid (1.3.6.1.2.1.191.1.3.1.3) | not-accessible | No | As per MIB |
| ospfv3AsLsdbSequence (1.3.6.1.2.1.191.1.3.1.4) | read-only | No | As per MIB |
| ospfv3AsLsdbAge (1.3.6.1.2.1.191.1.3.1.5) | read-only | No | As per MIB |
| ospfv3AsLsdbChecksum (1.3.6.1.2.1.191.1.3.1.6) | read-only | No | As per MIB |
| ospfv3AsLsdbAdvertisement (1.3.6.1.2.1.191.1.3.1.7) | read-only | No | As per MIB |
| ospfv3AsLsdbTypeKnown (1.3.6.1.2.1.191.1.3.1.8) | read-only | No | As per MIB |

ospfv3AreaLsdbTable

OID of this table is: 1.3.6.1.2.1.191.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3AreaLsdbAreaId (1.3.6.1.2.1.191.1.4.1.1) | not-accessible | No | As per MIB |
| ospfv3AreaLsdbType (1.3.6.1.2.1.191.1.4.1.2) | not-accessible | No | As per MIB |
| ospfv3AreaLsdbRouterId (1.3.6.1.2.1.191.1.4.1.3) | not-accessible | No | As per MIB |
| ospfv3AreaLsdbLsid (1.3.6.1.2.1.191.1.4.1.4) | not-accessible | No | As per MIB |
| ospfv3AreaLsdbSequence (1.3.6.1.2.1.191.1.4.1.5) | read-only | No | As per MIB |
| ospfv3AreaLsdbAge (1.3.6.1.2.1.191.1.4.1.6) | read-only | No | As per MIB |
| ospfv3AreaLsdbChecksum (1.3.6.1.2.1.191.1.4.1.7) | read-only | No | As per MIB |
| ospfv3AreaLsdbAdvertisement (1.3.6.1.2.1.191.1.4.1.8) | read-only | No | As per MIB |
| ospfv3AreaLsdbTypeKnown (1.3.6.1.2.1.191.1.4.1.9) | read-only | No | As per MIB |

ospfv3LinkLsdbTable

OID of this table is: 1.3.6.1.2.1.191.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3LinkLsdbIfIndex (1.3.6.1.2.1.191.1.5.1.1) | not-accessible | No | As per MIB |
| ospfv3LinkLsdbIfInstId (1.3.6.1.2.1.191.1.5.1.2) | not-accessible | No | As per MIB |
| ospfv3LinkLsdbType (1.3.6.1.2.1.191.1.5.1.3) | not-accessible | No | As per MIB |
| ospfv3LinkLsdbRouterId (1.3.6.1.2.1.191.1.5.1.4) | not-accessible | No | As per MIB |
| ospfv3LinkLsdbLsid (1.3.6.1.2.1.191.1.5.1.5) | not-accessible | No | As per MIB |
| ospfv3LinkLsdbSequence (1.3.6.1.2.1.191.1.5.1.6) | read-only | No | As per MIB |
| ospfv3LinkLsdbAge (1.3.6.1.2.1.191.1.5.1.7) | read-only | No | As per MIB |
| ospfv3LinkLsdbChecksum (1.3.6.1.2.1.191.1.5.1.8) | read-only | No | As per MIB |
| ospfv3LinkLsdbAdvertisement (1.3.6.1.2.1.191.1.5.1.9) | read-only | No | As per MIB |
| ospfv3LinkLsdbTypeKnown (1.3.6.1.2.1.191.1.5.1.10) | read-only | No | As per MIB |

ospfv3HostTable

OID of this table is: 1.3.6.1.2.1.191.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3HostAddressType (1.3.6.1.2.1.191.1.6.1.) | not-accessible | No | Not supported |
| ospfv3HostAddress (1.3.6.1.2.1.191.1.6.2.) | not-accessible | No | Not supported |
| ospfv3HostMetric (1.3.6.1.2.1.191.1.6.3.) | read-create | No | Not supported |
| ospfv3HostRowStatus( 1.3.6.1.2.1.191.1.6.4.) | read-create | No | Not supported |
| ospfv3HostAreaID (1.3.6.1.2.1.191.1.6.5.) | read-create | No | Not supported |

ospfv3IfTable

OID of this table is: 1.3.6.1.2.1.191.1.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3IfIndex (1.3.6.1.2.1.191.1.7.1.1) | not-accessible | No | As per MIB |
| ospfv3IfInstId (1.3.6.1.2.1.191.1.7.1.2) | not-accessible | No | As per MIB |
| ospfv3IfAreaId (1.3.6.1.2.1.191.1.7.1.3) | read-create | No | Only support read operation |
| ospfv3IfType (1.3.6.1.2.1.191.1.7.1.4) | read-create | Current | Only support read and write operations, and write operation is not supported by LoopBack interface. |
| ospfv3IfAdminStatus (1.3.6.1.2.1.191.1.7.1.5) | read-create | Yes | Only support read operation |
| ospfv3IfRtrPriority (1.3.6.1.2.1.191.1.7.1.6) | read-create | Current | Only support read and write operations, and write operation is not supported by LoopBack interface. |
| ospfv3IfTransitDelay (1.3.6.1.2.1.191.1.7.1.7) | read-create | Current | Only support read and write operations, and write operation is not supported by LoopBack interface. The value range is from 1 to 1800. |
| ospfv3IfRetransInterval (1.3.6.1.2.1.191.1.7.1.8) | read-create | Current | Only support read and write operations, and write operation is not supported by LoopBack interface. The value range is from 1 to 1800. |
| ospfv3IfHelloInterval (1.3.6.1.2.1.191.1.7.1.9) | read-create | Current | Only support read and write operations, and write operation is not supported by LoopBack interface. |
| ospfv3IfRtrDeadInterval (1.3.6.1.2.1.191.1.7.1.10) | read-create | Current | Only support read and write operations, and write operation is not supported by LoopBack interface. When the value of ospfv3IfRtrDeadInterval is bigger than 65535, 65535 is returned. |
| ospfv3IfPollInterval (1.3.6.1.2.1.191.1.7.1.11) | read-create | Current | Only support read and write operations. The value range is from 1 to 65535, and write operation is not supported by LoopBack interface. |
| ospfv3IfState (1.3.6.1.2.1.191.1.7.1.12) | read-only | No | As per MIB |
| ospfv3IfDesignatedRouter (1.3.6.1.2.1.191.1.7.1.13) | read-only | No | As per MIB |
| ospfv3IfBackupDesignatedRouter (1.3.6.1.2.1.191.1.7.1.14) | read-only | No | As per MIB |
| ospfv3IfEvents(1.3.6.1.2.1.191.1.7.1.15) | read-only | No | As per MIB |
| ospfv3IfRowStatus (1.3.6.1.2.1.191.1.7.1.16) | read-create | No | Only support read operation |
| ospfv3IfDemand (1.3.6.1.2.1.191.1.7.1.17) | read-create | Yes | Only support read operation |
| ospfv3IfMetricValue (1.3.6.1.2.1.191.1.7.1.18) | read-create | Current | Only support read and write operations . The value range is from 0 to 65535 for LoopBack interface, and from 1 to 65535 for other interfaces. |
| ospfv3IfLinkScopeLsaCount (1.3.6.1.2.1.191.1.7.1.19) | read-only | No | As per MIB |
| ospfv3IfLinkLsaCksumSum (1.3.6.1.2.1.191.1.7.1.20) | read-only | No | As per MIB |
| ospfv3IfDemandNbrProbe (1.3.6.1.2.1.191.1.7.1.21) | read-create | Yes | Only support read operation |
| ospfv3IfDemandNbrProbeRetransLimit (1.3.6.1.2.1.191.1.7.1.22) | read-create | Yes | Only support read operation |
| ospfv3IfDemandNbrProbeInterval (1.3.6.1.2.1.191.1.7.1.23) | read-create | Yes | Only support read operation |
| ospfv3IfTEDisabled (1.3.6.1.2.1.191.1.7.1.24) | read-create | Yes | Only support read operation |
| ospfv3IfLinkLSASuppression (1.3.6.1.2.1.191.1.7.1.25) | read-create | Yes | Only support read operation |

ospfv3VirtIfTable

OID of this table is: 1.3.6.1.2.1.191.1.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3VirtIfAreaId (1.3.6.1.2.1.191.1.8.1.1) | not-accessible | No | As per MIB |
| ospfv3VirtIfNeighbor (1.3.6.1.2.1.191.1.8.1.2) | not-accessible | No | As per MIB |
| ospfv3VirtIfIndex (1.3.6.1.2.1.191.1.8.1.3) | read-only | No | As per MIB |
| ospfv3VirtIfInstId (1.3.6.1.2.1.191.1.8.1.4) | read-only | No | As per MIB |
| ospfv3VirtIfTransitDelay (1.3.6.1.2.1.191.1.8.1.5) | read-create | Current | Only support read and write operations. The value range is from 1 to 1800 |
| ospfv3VirtIfRetransInterval (1.3.6.1.2.1.191.1.8.1.6) | read-create | Current | Only support read and write operations. The value range is from 1 to 1800 |
| ospfv3VirtIfHelloInterval (1.3.6.1.2.1.191.1.8.1.7) | read-create | Current | Only support read and write operations. The value range is from 1 to 8192 |
| ospfv3VirtIfRtrDeadInterval (1.3.6.1.2.1.191.1.8.1.8) | read-create | Current | Only support read and write operations. The default value is 40. The range is from 1 to 32768 |
| ospfv3VirtIfState (1.3.6.1.2.1.191.1.8.1.9) | read-only | No | As per MIB |
| ospfv3VirtIfEvents (1.3.6.1.2.1.191.1.8.1.10) | read-only | No | As per MIB |
| ospfv3VirtIfRowStatus (1.3.6.1.2.1.191.1.8.1.11) | read-create | No | Only support read operation |
| ospfv3VirtIfLinkScopeLsaCount (1.3.6.1.2.1.191.1.8.1.12) | read-only | No | As per MIB |
| ospfv3VirtIfLinkLsaCksumSum (1.3.6.1.2.1.191.1.8.1.13) | read-only | No | As per MIB |

ospfv3NbrTable

OID of this table is: 1.3.6.1.2.1.191.1.9

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3NbrIfIndex (1.3.6.1.2.1.191.1.9.1.1) | not-accessible | No | As per MIB |
| ospfv3NbrIfInstId (1.3.6.1.2.1.191.1.9.1.2) | not-accessible | No | As per MIB |
| ospfv3NbrRtrId (1.3.6.1.2.1.191.1.9.1.3) | not-accessible | No | As per MIB |
| ospfv3NbrAddressType (1.3.6.1.2.1.191.1.9.1.4) | read-only | No | As per MIB |
| ospfv3NbrAddress (1.3.6.1.2.1.191.1.9.1.5) | read-only | No | As per MIB |
| ospfv3NbrOptions (1.3.6.1.2.1.191.1.9.1.6) | read-only | No | As per MIB |
| ospfv3NbrPriority (1.3.6.1.2.1.191.1.9.1.7) | read-only | No | As per MIB |
| ospfv3NbrState (1.3.6.1.2.1.191.1.9.1.8) | read-only | No | As per MIB |
| ospfv3NbrEvents (1.3.6.1.2.1.191.1.9.1.9) | read-only | No | As per MIB |
| ospfv3NbrLsRetransQLen (1.3.6.1.2.1.191.1.9.1.10) | read-only | No | As per MIB |
| ospfv3NbrHelloSuppressed (1.3.6.1.2.1.191.1.9.1.11) | read-only | No | As per MIB |
| ospfv3NbrIfId (1.3.6.1.2.1.191.1.9.1.12) | read-only | No | As per MIB |
| ospfv3NbrRestartHelperStatus (1.3.6.1.2.1.191.1.9.1.13) | read-only | No | As per MIB |
| ospfv3NbrRestartHelperAge (1.3.6.1.2.1.191.1.9.1.14) | read-only | No | As per MIB |
| ospfv3NbrRestartHelperExitReason (1.3.6.1.2.1.191.1.9.1.15) | read-only | No | As per MIB |

ospfv3CfgNbrTable

OID of this table is: 1.3.6.1.2.1.191.1.10

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3CfgNbrIfIndex (1.3.6.1.2.1.191.1.10.1.1) | not-accessible | No | As per MIB |
| ospfv3CfgNbrIfInstId (1.3.6.1.2.1.191.1.10.1.2) | not-accessible | No | As per MIB |
| ospfv3CfgNbrAddressType (1.3.6.1.2.1.191.1.10.1.3) | not-accessible | No | As per MIB |
| ospfv3CfgNbrAddress (1.3.6.1.2.1.191.1.10.1.4) | not-accessible | No | As per MIB |
| ospfv3CfgNbrPriority (1.3.6.1.2.1.191.1.10.1.5) | read-create | Current | Only support read and write operations, and write operation is only supported by NBMA neighbor |
| ospfv3CfgNbrRowStatus (1.3.6.1.2.1.191.1.10.1.6) | read-create | No | Only support read operation |

ospfv3VirtNbrTable

OID of this table is: 1.3.6.1.2.1.191.1.11

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3VirtNbrArea (1.3.6.1.2.1.191.1.11.1.1) | not-accessible | No | As per MIB |
| ospfv3VirtNbrRtrId (1.3.6.1.2.1.191.1.11.1.2) | not-accessible | No | As per MIB |
| ospfv3VirtNbrIfIndex (1.3.6.1.2.1.191.1.11.1.3) | read-only | No | As per MIB |
| ospfv3VirtNbrIfInstId (1.3.6.1.2.1.191.1.11.1.4) | read-only | No | As per MIB |
| ospfv3VirtNbrAddressType (1.3.6.1.2.1.191.1.11.1.5) | read-only | No | As per MIB |
| ospfv3VirtNbrAddress (1.3.6.1.2.1.191.1.11.1.6) | read-only | No | As per MIB |
| ospfv3VirtNbrOptions (1.3.6.1.2.1.191.1.11.1.7) | read-only | No | As per MIB |
| ospfv3VirtNbrState (1.3.6.1.2.1.191.1.11.1.8) | read-only | No | As per MIB |
| ospfv3VirtNbrEvents (1.3.6.1.2.1.191.1.11.1.9) | read-only | No | As per MIB |
| ospfv3VirtNbrLsRetransQLen(1.3.6.1.2.1.191.1.11.1.10) | read-only | No | As per MIB |
| ospfv3VirtNbrHelloSuppressed (1.3.6.1.2.1.191.1.11.1.11) | read-only | No | As per MIB |
| ospfv3VirtNbrIfId (1.3.6.1.2.1.191.1.11.1.12) | read-only | No | As per MIB |
| ospfv3VirtNbrRestartHelperStatus (1.3.6.1.2.1.191.1.11.1.13) | read-only | No | As per MIB |
| ospfv3VirtNbrRestartHelperAge (1.3.6.1.2.1.191.1.11.1.14) | read-only | No | As per MIB |
| ospfv3VirtNbrRestartHelperExitReason (1.3.6.1.2.1.191.1.11.1.15) | read-only | No | As per MIB |

ospfv3AreaAggregateTable

OID of this table is: 1.3.6.1.2.1.191.1.12

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3AreaAggregateAreaID (1.3.6.1.2.1.191.1.12.1.1) | not-accessible | No | As per MIB |
| ospfv3AreaAggregateAreaLsdbType (1.3.6.1.2.1.191.1.12.1.2) | not-accessible | No | As per MIB |
| ospfv3AreaAggregatePrefixType (1.3.6.1.2.1.191.1.12.1.3) | not-accessible | No | As per MIB |
| ospfv3AreaAggregatePrefix (1.3.6.1.2.1.191.1.12.1.4) | not-accessible | No | As per MIB |
| ospfv3AreaAggregatePrefixLength (1.3.6.1.2.1.191.1.12.1.5) | not-accessible | No | As per MIB |
| ospfv3AreaAggregateRowStatus (1.3.6.1.2.1.191.1.12.1.6) | read-create | No | Only support read operation |
| ospfv3AreaAggregateEffect (1.3.6.1.2.1.191.1.12.1.7) | read-create | Current | Only support read and write operations |
| ospfv3AreaAggregateRouteTag (1.3.6.1.2.1.191.1.12.1.8) | read-create | Current | Only support read and write operations |

ospfv3VirtLinkLsdbTable

OID of this table is: 1.3.6.1.2.1.191.1.13

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ospfv3VirtLinkLsdbIfAreaId (1.3.6.1.2.1.191.1.13.1.1) | not-accessible | No | As per MIB |
| ospfv3VirtLinkLsdbIfNeighbor(1.3.6.1.2.1.191.1.13.1.2) | not-accessible | No | As per MIB |
| ospfv3VirtLinkLsdbType (1.3.6.1.2.1.191.1.13.1.3) | not-accessible | No | As per MIB |
| ospfv3VirtLinkLsdbRouterId (1.3.6.1.2.1.191.1.13.1.4) | not-accessible | No | As per MIB |
| ospfv3VirtLinkLsdbLsid (1.3.6.1.2.1.191.1.13.1.5) | not-accessible | No | As per MIB |
| ospfv3VirtLinkLsdbSequence(1.3.6.1.2.1.191.1.13.1.6) | read-only | No | As per MIB |
| ospfv3VirtLinkLsdbAge (1.3.6.1.2.1.191.1.13.1.7) | read-only | No | As per MIB |
| ospfv3VirtLinkLsdbChecksum(1.3.6.1.2.1.191.1.13.1.8) | read-only | No | As per MIB |
| ospfv3VirtLinkLsdbAdvertisement (1.3.6.1.2.1.191.1.13.1.9) | read-only | No | As per MIB |
| ospfv3VirtLinkLsdbTypeKnown (1.3.6.1.2.1.191.1.13.1.10) | read-only | No | As per MIB |