# Contents

PIM-BSR-MIB	2

pimBsrCandidateRPTable	2

pimBsrElectedBSRRPSetTable	2

pimBsrCandidateBSRTable	3

pimBsrElectedBSRTable	4

PIM-BSR-MIB

The PIM-BSR-MIB module for management of the Bootstrap Router (BSR) mechanism for PIM routers.

pimBsrCandidateRPTable

OID of this table is: 1.3.6.1.2.1.172.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pimBsrCandidateRPAddressType (1.3.6.1.2.1.172.1.1.1.1) | not-accessible | Current | Only support: ipv4(1) ipv6(2) |
| pimBsrCandidateRPAddress (1.3.6.1.2.1.172.1.1.1.2) | not-accessible | Current | As per MIB |
| pimBsrCandidateRPGroupAddress (1.3.6.1.2.1.172.1.1.1.3) | not-accessible | Current | Only support all multicast group range |
| pimBsrCandidateRPGroupPrefixLength (1.3.6.1.2.1.172.1.1.1.4) | not-accessible | Current | Only support all multicast group range |
| pimBsrCandidateRPBidir (1.3.6.1.2.1.172.1.1.1.5) | read-create | Current | read-only |
| pimBsrCandidateRPAdvTimer (1.3.6.1.2.1.172.1.1.1.6) | read-only | Current | The object is zero if the advertisement timer is not running. |
| pimBsrCandidateRPPriority (1.3.6.1.2.1.172.1.1.1.7) | read-create | Current | read-only |
| pimBsrCandidateRPAdvInterval (1.3.6.1.2.1.172.1.1.1.8) | read-create | Current | read-only |
| pimBsrCandidateRPHoldtime (1.3.6.1.2.1.172.1.1.1.9) | read-create | Current | read-only Size list: 1: 1..65535 |
| pimBsrCandidateRPStatus (1.3.6.1.2.1.172.1.1.1.10) | read-create | Current | read-only Only support : active(1) createAndGo(4) destroy(6) |
| pimBsrCandidateRPStorageType (1.3.6.1.2.1.172.1.1.1.11) | read-create | Current | read-only Only support: other(1) |

pimBsrElectedBSRRPSetTable

OID of this table is: 1.3.6.1.2.1.172.1.2

This table only supports public net.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pimBsrElectedBSRGrpMappingAddrType (1.3.6.1.2.1.172.1.2.1.2) | not-accessible | Current | Only support: ipv4(1) ipv6(2) |
| pimBsrElectedBSRGrpMappingGrpAddr (1.3.6.1.2.1.172.1.2.1.3) | not-accessible | Current | As per MIB |
| pimBsrElectedBSRGrpMappingGrpPrefixLen (1.3.6.1.2.1.172.1.2.1.4) | not-accessible | Current | As per MIB |
| pimBsrElectedBSRGrpMappingRPAddr (1.3.6.1.2.1.172.1.2.1.5) | not-accessible | Current | As per MIB |
| pimBsrElectedBSRRPSetPriority (1.3.6.1.2.1.172.1.2.1.6) | read-only | Current | As per MIB |
| pimBsrElectedBSRRPSetHoldtime (1.3.6.1.2.1.172.1.2.1.7) | read-only | Current | As per MIB |
| pimBsrElectedBSRRPSetExpiryTime (1.3.6.1.2.1.172.1.2.1.8) | read-only | Current | The value zero indicates that this entry will be aged out at once. |
| pimBsrElectedBSRRPSetGrpBidir (1.3.6.1.2.1.172.1.2.1.9) | read-only | Current | As per MIB |

pimBsrCandidateBSRTable

OID of this table is: 1.3.6.1.2.1.172.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pimBsrCandidateBSRZoneIndex (1.3.6.1.2.1.172.1.3.1.1) | not-accessible | Current | Only support 0xFFFFFFFF 0X7FFFFFFF |
| pimBsrCandidateBSRAddressType (1.3.6.1.2.1.172.1.3.1.2) | read-create | Current | Only support: ipv4(1) ipv6(2) |
| pimBsrCandidateBSRAddress (1.3.6.1.2.1.172.1.3.1.3) | read-create | Current | As per MIB |
| pimBsrCandidateBSRPriority (1.3.6.1.2.1.172.1.3.1.4) | read-create | Current | Default values: 1: 64 (int) Size list: 1: 0..255 |
| pimBsrCandidateBSRHashMaskLength (1.3.6.1.2.1.172.1.3.1.5) | read-create | Current | Size list: 1: 0..32 |
| pimBsrCandidateBSRElectedBSR (1.3.6.1.2.1.172.1.3.1.6) | read-only | Current | As per MIB |
| pimBsrCandidateBSRBootstrapTimer (1.3.6.1.2.1.172.1.3.1.7) | read-only | Current | As per MIB |
| pimBsrCandidateBSRStatus (1.3.6.1.2.1.172.1.3.1.8) | read-create | Current | Only support : active(1) createAndGo(4) destroy(6) |
| pimBsrCandidateBSRStorageType (1.3.6.1.2.1.172.1.3.1.9) | read-create | Current | read-only Only support: other(1) |

pimBsrElectedBSRTable

OID of this table is: 1.3.6.1.2.1.172.1.4

This table only supports public net.

This table doesn’t support administrative scoping.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pimBsrElectedBSRZoneIndex (1.3.6.1.2.1.172.1.4.1.1) | not-accessible | Current | Only support 0xFFFFFFFF 0X7FFFFFFF |
| pimBsrElectedBSRAddressType (1.3.6.1.2.1.172.1.4.1.2) | read-only | Current | Only support: ipv4(1) ipv6(2) |
| pimBsrElectedBSRAddress (1.3.6.1.2.1.172.1.4.1.3) | read-only | Current | As per MIB |
| pimBsrElectedBSRPriority (1.3.6.1.2.1.172.1.4.1.4) | read-only | Current | As per MIB |
| pimBsrElectedBSRHashMaskLength (1.3.6.1.2.1.172.1.4.1.5) | read-only | Current | Size list: 1: 0..128 |
| pimBsrElectedBSRExpiryTime (1.3.6.1.2.1.172.1.4.1.6) | read-only | Current | As per MIB |