# Contents

PW-ENET-STD-MIB	2

pwEnetTable	2

PW-ENET-STD-MIB

This MIB is used to describe the Ethernet PW information. Only support for VPWS.

pwEnetTable

OID of this table is: 1.3.6.1.2.1.180.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pwEnetPwInstance (1.3.6.1.2.1.180.1.1.1.1) | not-accessible | Current | As per MIB |
| pwEnetPwVlan (1.3.6.1.2.1.180.1.1.1.2) | read-create | Current | As per MIB |
| pwEnetVlanMode (1.3.6.1.2.1.180.1.1.1.3) | read-create | Current | As per MIB |
| pwEnetPortVlan (1.3.6.1.2.1.180.1.1.1.4) | read-create | Current | As per MIB |
| pwEnetPortIfIndex (1.3.6.1.2.1.180.1.1.1.5) | read-create | Current | As per MIB |
| pwEnetPwIfIndex (1.3.6.1.2.1.180.1.1.1.6) | read-create | Current | Not supported |
| pwEnetRowStatus (1.3.6.1.2.1.180.1.1.1.7) | read-create | Current | Only support active(1)，createAndGo(4)，destroy(6) |
| pwEnetStorageType (1.3.6.1.2.1.180.1.1.1.8) | read-create | Current | Only support read operation.The value is always nonVolatile(3). |

Notes: See the following matrix for value combinations of pwEnetPortVlan, pwType (in PW-STD-MIB pwTable), pwEnetVlanMode, and pwEnetPwVlan.

| Ethernet service instance |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| encapsulation | AC access mode | pwType | pwEnetPortVlan | pwEnetVlanMode | pwEnetPwVlan |  |
| default ¦ tagged | ethernet | ethernet | 4095 | portBased | 4095 | For set operation, only support default encapsulation |
| default ¦ tagged | ethernet | ethernetTagged | 4095 | addVlan | 0 or requested VLAN | For set operation, only support default encapsulation |
| default ¦ tagged | vlan | ethernet | 4095 | removeVlan | 0 | For set operation, only support default encapsulation |
| default ¦ tagged | vlan | ethernetTagged | 4095 | noChange or changeVlan | 4095 or requested VLAN | For set operation, only support default encapsulation |
| untagged | ethernet | ethernet | 0 | nochange | 0 |  |
| untagged | ethernet | ethernetTagged | 0 | addVlan | 0 or requested VLAN |  |
| untagged | vlan | ethernet | 0 | nochange | 0 | Only support read operation |
| untagged | vlan | ethernetTagged | 0 | addVlan | 0 or requested VLAN | Only support read operation |
| s-vid [only-tagged] | ethernet | ethernet | s-vid | noChange | s-vid | For set operation, only support s-vid encapsulation |
| s-vid [only-tagged] | ethernet | ethernetTagged | s-vid | addVlan | 0 or requested VLAN | For set operation, only support s-vid encapsulation |
| s-vid [only-tagged] | vlan | ethernet | s-vid | removeVlan | 0 | For set operation, only support s-vid encapsulation |
| s-vid [only-tagged] | vlan | ethernetTagged | s-vid | noChange or changeVlan | s-vid or requested VLAN | For set operation, only support s-vid encapsulation |

| L3 interface |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| interface type | AC access mode | pwType | pwEnetPortVlan | pwEnetVlanMode | pwEnetPwVlan |
| main | ethernet | ethernet | 4095 | portBased | 4095 |
| main | ethernet | ethernetTagged | 4095 | addVlan | 0 or requested VLAN |
| sub | vlan | ethernet | vlan dot1q | removeVlan | 4095 |
| sub | vlan | ethernetTagged | vlan dot1q | changeVlan | requested VLAN |
| sub | vlan | ethernetTagged | vlan dot1q | noChange | vlan dot1q |
| vlan | vlan | ethernet | vlan ID | removeVlan | 4095 |
| vlan | vlan | ethernetTagged | vlan ID | changeVlan | requested VLAN |
| vlan | vlan | ethernetTagged | vlan ID | noChange | vlan ID |