# Contents

Q-BRIDGE-MIB	2

dot1qBase group	2

dot1qFdbTable	2

dot1qTpFdbTable	2

dot1qTpGroupTable	3

dot1qForwardAllTable	3

dot1qForwardUnregisteredTable	3

dot1qStaticUnicastTable	4

dot1qStaticMulticastTable	4

Scalar objects of dot1qVlan group	5

dot1qVlanCurrentTable	5

dot1qVlanStaticTable	6

dot1qPortVlanTable	7

dot1qPortVlanStatisticsTable	7

dot1qPortVlanHCStatisticsTable	8

dot1qLearningConstraintsTable	8

dot1vProtocolGroupTable	8

dot1vProtocolPortTable	9

Q-BRIDGE-MIB

The corresponding function should be implemented before this MIB is supported.

dot1qBase group

OID of this table is: 1.3.6.1.2.1.17.7.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qVlanVersionNumber (1.3.6.1.2.1.17.7.1.1.1) | read-only | Current | As per MIB |
| dot1qMaxVlanId (1.3.6.1.2.1.17.7.1.1.2) | read-only | Current | As per MIB |
| dot1qMaxSupportedVlans (1.3.6.1.2.1.17.7.1.1.3) | read-only | Current | As per MIB |
| dot1qNumVlans (1.3.6.1.2.1.17.7.1.1.4) | read-only | No | As per MIB |
| dot1qGvrpStatus (1.3.6.1.2.1.17.7.1.1.5) | read-write | No | Not supported |

dot1qFdbTable

Information about a specific Filtering Database.

OID of this table is: 1.3.6.1.2.1.17.7.1.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qFdbId (1.3.6.1.2.1.17.7.1.2.1.1.1) | not-accessible | No | As per MIB |
| dot1qFdbDynamicCount (1.3.6.1.2.1.17.7.1.2.1.1.2) | read-only | No | As per MIB |

dot1qTpFdbTable

OID of this table is: 1.3.6.1.2.1.17.7.1.2.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qTpFdbAddress (1.3.6.1.2.1.17.7.1.2.2.1.1) | not-accessible | Current | As per MIB |
| dot1qTpFdbPort (1.3.6.1.2.1.17.7.1.2.2.1.2) | read-only | Current | As per MIB |
| dot1qTpFdbStatus (1.3.6.1.2.1.17.7.1.2.2.1.3) | read-only | Current | The MAC addresses configured as dynamic are integrated into the MAC addresses learned. |

dot1qTpGroupTable

OID of this table is: 1.3.6.1.2.1.17.7.1.2.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qTpGroupAddress (1.3.6.1.2.1.17.7.1.2.3.1.1) | not-accessible | Current | As per MIB |
| dot1qTpGroupEgressPorts (1.3.6.1.2.1.17.7.1.2.3.1.2) | read-only | Current | As per MIB |
| dot1qTpGroupLearnt (1.3.6.1.2.1.17.7.1.2.3.1.3) | read-only | Current | As per MIB |

dot1qForwardAllTable

A table containing forwarding information for each

VLAN, specifying the set of ports to which forwarding of

all multicasts applies, configured statically by

management or dynamically by GMRP.  An entry appears in

this table for all VLANs that are currently

instantiated.

OID of this table is: 1.3.6.1.2.1.17.7.1.2.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qForwardAllPorts (1.3.6.1.2.1.17.7.1.2.4.1.1) | read-only | No | Not supported |
| dot1qForwardAllStaticPorts (1.3.6.1.2.1.17.7.1.2.4.1.2) | read-write | Current | Not supported |
| dot1qForwardAllForbiddenPorts (1.3.6.1.2.1.17.7.1.2.4.1.3) | read-write | Current | Not supported |

dot1qForwardUnregisteredTable

A table containing forwarding information for each

VLAN, specifying the set of ports to which forwarding of

multicast group-addressed frames for which no

more specific forwarding information applies.  This is

configured statically by management and determined

dynamically by GMRP.  An entry appears in this table for

all VLANs that are currently instantiated.

OID of this table is: 1.3.6.1.2.1.17.7.1.2.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qForwardUnregisteredPorts (1.3.6.1.2.1.17.7.1.2.5.1.1) | read-only | No | Not supported |
| dot1qForwardUnregisteredStaticPorts (1.3.6.1.2.1.17.7.1.2.5.1.2) | read-write | Current | Not supported |
| dot1qForwardUnregisteredForbiddenPorts (1.3.6.1.2.1.17.7.1.2.5.1.3) | read-write | Current | Not supported |

dot1qStaticUnicastTable

A table containing filtering information for Unicast

MAC addresses for each Filtering Database, configured

into the device by (local or network) management

specifying the set of ports to which frames received

from specific ports and containing specific unicast

destination addresses are allowed to be forwarded.  A

value of zero in this table (as the port number from

which frames with a specific destination address are

received) is used to specify all ports for which there

is no specific entry in this table for that particular

destination address.  Entries are valid for unicast

addresses only.

OID of this table is: 1.3.6.1.2.1.17.7.1.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qStaticUnicastAddress (1.3.6.1.2.1.17.7.1.3.1.1.1) | not-accessible | No | Not supported |
| dot1qStaticUnicastReceivePort (1.3.6.1.2.1.17.7.1.3.1.1.2) | not-accessible | No | Not supported |
| dot1qStaticUnicastAllowedToGoTo (1.3.6.1.2.1.17.7.1.3.1.1.3) | read-write | Current | Not supported |
| dot1qStaticUnicastStatus (1.3.6.1.2.1.17.7.1.3.1.1.4) | read-write | Current | Not supported |

dot1qStaticMulticastTable

A table containing filtering information for Multicast

and Broadcast MAC addresses for each VLAN, configured

into the device by (local or network) management

specifying the set of ports to which frames received

from specific ports and containing specific Multicast

and Broadcast destination addresses are allowed to be

forwarded.  A value of zero in this table (as the port

number from which frames with a specific destination

address are received) is used to specify all ports for

which there is no specific entry in this table for that

particular destination address.  Entries are valid for

Multicast and Broadcast addresses only.

OID of this table is: 1.3.6.1.2.1.17.7.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qStaticMulticastAddress (1.3.6.1.2.1.17.7.1.3.2.1.1) | not-accessible | No | Not supported |
| dot1qStaticMulticastReceivePort (1.3.6.1.2.1.17.7.1.3.2.1.2) | not-accessible | No | Not supported |
| dot1qStaticMulticastStaticEgressPorts (1.3.6.1.2.1.17.7.1.3.2.1.3) | read-write | Current | Not supported |
| dot1qStaticMulticastForbiddenEgressPorts (1.3.6.1.2.1.17.7.1.3.2.1.4) | read-write | Current | Not supported |
| dot1qStaticMulticastStatus (1.3.6.1.2.1.17.7.1.3.2.1.5) | read-write | Current | Not supported |

Scalar objects of dot1qVlan group

OID of this table is: 1.3.6.1.2.1.17.7.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qVlanNumDeletes (1.3.6.1.2.1.17.7.1.4.1) | read-only | No | Not supported |
| dot1qNextFreeLocalVlanIndex (1.3.6.1.2.1.17.7.1.4.4) | read-only | No | Not supported |
| dot1qConstraintSetDefault (1.3.6.1.2.1.17.7.1.4.9) | read-write | Current | Not supported |
| dot1qConstraintTypeDefault (1.3.6.1.2.1.17.7.1.4.10) | read-write | Current | Only supported read operation. |

dot1qVlanCurrentTable

OID of this table is: 1.3.6.1.2.1.17.7.1.4.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qVlanTimeMark (1.3.6.1.2.1.17.7.1.4.2.1.1) | not-accessible | No | As per MIB |
| dot1qVlanIndex (1.3.6.1.2.1.17.7.1.4.2.1.2) | not-accessible | No | As per MIB |
| dot1qVlanFdbId (1.3.6.1.2.1.17.7.1.4.2.1.3) | read-only | No | As per MIB |
| dot1qVlanCurrentEgressPorts (1.3.6.1.2.1.17.7.1.4.2.1.4) | read-only | No | As per MIB |
| dot1qVlanCurrentUntaggedPorts (1.3.6.1.2.1.17.7.1.4.2.1.5) | read-only | No | As per MIB |
| dot1qVlanStatus (1.3.6.1.2.1.17.7.1.4.2.1.6) | read-only | No | As per MIB |
| dot1qVlanCreationTime (1.3.6.1.2.1.17.7.1.4.2.1.7) | read-only | No | As per MIB |

dot1qVlanStaticTable

OID of this table is: 1.3.6.1.2.1.17.7.1.4.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qVlanStaticName (1.3.6.1.2.1.17.7.1.4.3.1.1) | read-create | Current | A null string or zero length string inputted as dot1qVlanStaticName will be replaced with the default name of the vlan. Default value for each vlan is “vlan xxxx”, and xxxx is serial number of the vlan. |
| dot1qVlanStaticEgressPorts (1.3.6.1.2.1.17.7.1.4.3.1.2) | read-create | Current | As per MIB |
| dot1qVlanForbiddenEgressPorts (1.3.6.1.2.1.17.7.1.4.3.1.3) | read-create | Current | Not supported |
| dot1qVlanStaticUntaggedPorts (1.3.6.1.2.1.17.7.1.4.3.1.4) | read-create | Current | As per MIB |
| dot1qVlanStaticRowStatus (1.3.6.1.2.1.17.7.1.4.3.1.5) | read-create | Current | Only support active(1), createAndgo(4) and destroy(6). The default vlan can not be destroyed. |

dot1qPortVlanTable

OID of this table is: 1.3.6.1.2.1.17.7.1.4.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qPvid (1.3.6.1.2.1.17.7.1.4.5.1.1) | read-write | Current | This node does not support set operation when the port type is access |
| dot1qPortAcceptableFrameTypes (1.3.6.1.2.1.17.7.1.4.5.1.2) | read-write | No | Only supported read operation. |
| dot1qPortIngressFiltering (1.3.6.1.2.1.17.7.1.4.5.1.3) | read-write | No | Only supported read operation. |
| dot1qPortGvrpStatus (1.3.6.1.2.1.17.7.1.4.5.1.4) | read-write | Current | Not supported |
| dot1qPortGvrpFailedRegistrations (1.3.6.1.2.1.17.7.1.4.5.1.5) | read-only | No | Not supported |
| dot1qPortGvrpLastPduOrigin (1.3.6.1.2.1.17.7.1.4.5.1.6) | read-only | No | Not supported |

dot1qPortVlanStatisticsTable

A table containing per-port, per-VLAN statistics for

traffic received.  Separate objects are provided for both the

most-significant and least-significant bits of statistics

counters for ports that are associated with this transparent

bridge.  The most-significant bit objects are only required on

high-capacity interfaces, as defined in the conformance clauses

for these objects.  This mechanism is provided as a way to read

64-bit counters for agents that support only SNMPv1.

OID of this table is: 1.3.6.1.2.1.17.7.1.4.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qTpVlanPortInFrames (1.3.6.1.2.1.17.7.1.4.6.1.1) | read-only | No | Not supported |
| dot1qTpVlanPortOutFrames (1.3.6.1.2.1.17.7.1.4.6.1.2) | read-only | No | Not supported |
| dot1qTpVlanPortInDiscards (1.3.6.1.2.1.17.7.1.4.6.1.3) | read-only | No | Not supported |
| dot1qTpVlanPortInOverflowFrames (1.3.6.1.2.1.17.7.1.4.6.1.4) | read-only | No | Not supported |
| dot1qTpVlanPortOutOverflowFrames (1.3.6.1.2.1.17.7.1.4.6.1.5) | read-only | No | Not supported |
| dot1qTpVlanPortInOverflowDiscards (1.3.6.1.2.1.17.7.1.4.6.1.6) | read-only | No | Not supported |

dot1qPortVlanHCStatisticsTable

A table containing per-port, per-VLAN statistics for

traffic on high-capacity interfaces.

OID of this table is: 1.3.6.1.2.1.17.7.1.4.7

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qTpVlanPortHCInFrames (1.3.6.1.2.1.17.7.1.4.7.1.1) | read-only | No | Not supported |
| dot1qTpVlanPortHCOutFrames (1.3.6.1.2.1.17.7.1.4.7.1.2) | read-only | No | Not supported |
| dot1qTpVlanPortHCInDiscards (1.3.6.1.2.1.17.7.1.4.7.1.3) | read-only | No | Not supported |

dot1qLearningConstraintsTable

The identity of the constraint set to which

dot1qConstraintVlan belongs.  These values may be chosen

by the management station.

OID of this table is: 1.3.6.1.2.1.17.7.1.4.8

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1qConstraintVlan (1.3.6.1.2.1.17.7.1.4.8.1.1) | not-accessible | No | Not supported |
| dot1qConstraintSet (1.3.6.1.2.1.17.7.1.4.8.1.2) | not-accessible | No | Not supported |
| dot1qConstraintType (1.3.6.1.2.1.17.7.1.4.8.1.3) | read-create | Current | Not supported |
| dot1qConstraintStatus (1.3.6.1.2.1.17.7.1.4.8.1.4) | read-create | Current | Not supported |

dot1vProtocolGroupTable

A table that contains mappings from Protocol

Templates to Protocol Group Identifiers used for

Port-and-Protocol-based VLAN Classification.

OID of this table is: 1.3.6.1.2.1.17.7.1.5.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1vProtocolTemplateFrameType (1.3.6.1.2.1.17.7.1.5.1.1.1) | not-accessible | No | Not supported |
| dot1vProtocolTemplateProtocolValue (1.3.6.1.2.1.17.7.1.5.1.1.2) | not-accessible | No | Not supported |
| dot1vProtocolGroupId (1.3.6.1.2.1.17.7.1.5.1.1.3) | read-create | Current | Not supported |
| dot1vProtocolGroupRowStatus (1.3.6.1.2.1.17.7.1.5.1.1.4) | read-create | Current | Not supported |

dot1vProtocolPortTable

A table that contains VID sets used for

Port-and-Protocol-based VLAN Classification.

OID of this table is: 1.3.6.1.2.1.17.7.1.5.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| dot1vProtocolPortGroupId (1.3.6.1.2.1.17.7.1.5.2.1.1) | not-accessible | No | Not supported |
| dot1vProtocolPortGroupVid (1.3.6.1.2.1.17.7.1.5.2.1.2) | read-create | current | Not supported |
| dot1vProtocolPortRowStatus (1.3.6.1.2.1.17.7.1.5.2.1.3) | read-create | Current | Not supported |