# Contents

RMON-MIB	2

etherStatsTable	2

historyControlTable	3

etherHistoryTable	4

alarmTable	5

eventTable	5

logTable	6

RMON-MIB

The RMON-MIB is used with network management protocols in TCP/IP-based internets. In particular, it defines objects for managing remote network monitoring devices.

etherStatsTable

Note: When creating a new instance of this table, the following objects SHOULD be set simultaneously: etherStatsDataSource, etherStatsStatus.

OID of this table is: 1.3.6.1.2.1.16.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| etherStatsIndex (1.3.6.1.2.1.16.1.1.1.1) | read-only | Current | As per MIB |
| etherStatsDataSource (1.3.6.1.2.1.16.1.1.1.2) | read-create | Current | As per MIB |
| etherStatsDropEvents (1.3.6.1.2.1.16.1.1.1.3) | read-only | No | As per MIB |
| etherStatsOctets (1.3.6.1.2.1.16.1.1.1.4) | read-only | No | As per MIB |
| etherStatsPkts (1.3.6.1.2.1.16.1.1.1.5) | read-only | No | As per MIB |
| etherStatsBroadcastPkts (1.3.6.1.2.1.16.1.1.1.6) | read-only | No | As per MIB |
| etherStatsMulticastPkts (1.3.6.1.2.1.16.1.1.1.7) | read-only | No | As per MIB |
| etherStatsCRCAlignErrors (1.3.6.1.2.1.16.1.1.1.8) | read-only | No | The packet length can be specified by command: jumboframe enable. |
| etherStatsUndersizePkts (1.3.6.1.2.1.16.1.1.1.9) | read-only | No | As per MIB |
| etherStatsOversizePkts (1.3.6.1.2.1.16.1.1.1.10) | read-only | No | The packet length can be specified by command: jumboframe enable. |
| etherStatsFragments (1.3.6.1.2.1.16.1.1.1.11) | read-only | No | As per MIB |
| etherStatsJabbers (1.3.6.1.2.1.16.1.1.1.12) | read-only | No | Not supported |
| etherStatsCollisions (1.3.6.1.2.1.16.1.1.1.13) | read-only | No | As per MIB |
| etherStatsPkts64Octets (1.3.6.1.2.1.16.1.1.1.14) | read-only | No | As per MIB |
| etherStatsPkts65to127Octets (1.3.6.1.2.1.16.1.1.1.15) | read-only | No | As per MIB |
| etherStatsPkts128to255Octets (1.3.6.1.2.1.16.1.1.1.16) | read-only | No | As per MIB |
| etherStatsPkts256to511Octets (1.3.6.1.2.1.16.1.1.1.17) | read-only | No | As per MIB |
| etherStatsPkts512to1023Octets (1.3.6.1.2.1.16.1.1.1.18) | read-only | No | As per MIB |
| etherStatsPkts1024to1518Octets (1.3.6.1.2.1.16.1.1.1.19) | read-only | No | As per MIB |
| etherStatsOwner (1.3.6.1.2.1.16.1.1.1.20) | read-create | Current | The length of value is from 0 to 127. Not support non-printable characters and ‘?’. Default value is zero-length string. |
| etherStatsStatus (1.3.6.1.2.1.16.1.1.1.21) | read-create | Current | As per MIB |

historyControlTable

Note: When creating a new instance of this table, the following objects SHOULD be set simultaneously: historyControlDataSource, historyControlStatus.

OID of this table is: 1.3.6.1.2.1.16.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| historyControlIndex (1.3.6.1.2.1.16.2.1.1.1) | read-only | Current | As per MIB |
| historyControlDataSource (1.3.6.1.2.1.16.2.1.1.2) | read-create | Current | As per MIB |
| historyControlBucketsRequested (1.3.6.1.2.1.16.2.1.1.3) | read-create | Current | historyControlBucketsRequested only supports 50 buckets at maximum currently, If a value more than 50 is set, the system will return success, but the historyControlBucketGranted will be assigned to 50 as for implementation. |
| historyControlBucketsGranted (1.3.6.1.2.1.16.2.1.1.4) | read-only | No | As per MIB |
| historyControlInterval (1.3.6.1.2.1.16.2.1.1.5) | read-create | Current | Range from 5 to 3600. Default value is 1800. |
| historyControlOwner (1.3.6.1.2.1.16.2.1.1.6) | read-create | Current | The length of value is from 0 to 127. Not support non-printable characters and ‘?’. Default value is zero-length string. |
| historyControlStatus (1.3.6.1.2.1.16.2.1.1.7) | read-create | Current | As per MIB |

etherHistoryTable

OID of this table is: 1.3.6.1.2.1.16.2.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| etherHistoryIndex (1.3.6.1.2.1.16.2.2.1.1) | read-only | No | As per MIB |
| etherHistorySampleIndex (1.3.6.1.2.1.16.2.2.1.2) | read-only | No | As per MIB |
| etherHistoryIntervalStart (1.3.6.1.2.1.16.2.2.1.3) | read-only | No | As per MIB |
| etherHistoryDropEvents (1.3.6.1.2.1.16.2.2.1.4) | read-only | No | As per MIB |
| etherHistoryOctets (1.3.6.1.2.1.16.2.2.1.5) | read-only | No | As per MIB |
| etherHistoryPkts (1.3.6.1.2.1.16.2.2.1.6) | read-only | No | As per MIB |
| etherHistoryBroadcastPkts (1.3.6.1.2.1.16.2.2.1.7) | read-only | No | As per MIB |
| etherHistoryMulticastPkts (1.3.6.1.2.1.16.2.2.1.8) | read-only | No | As per MIB |
| etherHistoryCRCAlignErrors (1.3.6.1.2.1.16.2.2.1.9) | read-only | No | The packet length can be specified by command: jumboframe enable. |
| etherHistoryUndersizePkts (1.3.6.1.2.1.16.2.2.1.10) | read-only | No | As per MIB |
| etherHistoryOversizePkts (1.3.6.1.2.1.16.2.2.1.11) | read-only | No | The packet length can be specified by command: jumboframe enable. |
| etherHistoryFragments (1.3.6.1.2.1.16.2.2.1.12) | read-only | No | As per MIB |
| etherHistoryJabbers (1.3.6.1.2.1.16.2.2.1.13) | read-only | No | Not supported |
| etherHistoryCollisions (1.3.6.1.2.1.16.2.2.1.14) | read-only | No | As per MIB |
| etherHistoryUtilization (1.3.6.1.2.1.16.2.2.1.15) | read-only | No | As per MIB |

alarmTable

Note:

1) When creating a new instance of this table, the following objects SHOULD be set simultaneously: alarmVariable, alarmStatus.

2) Configuring will be failed, if the length of command string build from current configuration is greater than max length of CLI.

OID of this table is: 1.3.6.1.2.1.16.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| alarmIndex (1.3.6.1.2.1.16.3.1.1.1) | read-only | Current | As per MIB |
| alarmInterval (1.3.6.1.2.1.16.3.1.1.2) | read-create | Current | Range from 5 to 65535. Default value is 1800. |
| alarmVariable (1.3.6.1.2.1.16.3.1.1.3) | read-create | Current | As per MIB |
| alarmSampleType (1.3.6.1.2.1.16.3.1.1.4) | read-create | Current | Default value is absoluteValue(1). |
| alarmValue (1.3.6.1.2.1.16.3.1.1.5) | read-only | No | If out of the positive range of alarmValue, the alarmValue may be a negative. |
| alarmStartupAlarm (1.3.6.1.2.1.16.3.1.1.6) | read-create | Current | Default value is risingOrFallingAlarm(3). |
| alarmRisingThreshold (1.3.6.1.2.1.16.3.1.1.7) | read-create | Current | Default value is 1. |
| alarmFallingThreshold (1.3.6.1.2.1.16.3.1.1.8) | read-create | Current | Default value is 0. |
| alarmRisingEventIndex (1.3.6.1.2.1.16.3.1.1.9) | read-create | Current | Default value is 0. |
| alarmFallingEventIndex (1.3.6.1.2.1.16.3.1.1.10) | read-create | Current | Default value is 0. |
| alarmOwner (1.3.6.1.2.1.16.3.1.1.11) | read-create | Current | The length of value is from 0 to 127. Not support non-printable characters and ‘?’. Default value is zero-length string. |
| alarmStatus (1.3.6.1.2.1.16.3.1.1.12) | read-create | Current | As per MIB |

eventTable

Note:

1) When creating a new instance of this table, the following objects SHOULD be set simultaneously: eventStatus.

2) Configuring will be failed, if the length of command string build from current configuration is greater than max length of CLI.

OID of this table is: 1.3.6.1.2.1.16.9.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| eventIndex (1.3.6.1.2.1.16.9.1.1.1) | read-only | Current | As per MIB |
| eventDescription (1.3.6.1.2.1.16.9.1.1.2) | read-create | Current | Default value is zero-length string. |
| eventType (1.3.6.1.2.1.16.9.1.1.3) | read-create | Current | Default value is none(1). |
| eventCommunity (1.3.6.1.2.1.16.9.1.1.4) | read-create | Current | Not supported. The value of eventCommunity will be saved but take no effect. |
| eventLastTimeSent (1.3.6.1.2.1.16.9.1.1.5) | read-only | No | As per MIB |
| eventOwner (1.3.6.1.2.1.16.9.1.1.6) | read-create | Current | The length of value is from 0 to 127. Not support non-printable characters and ‘?’. Default value is zero-length string. |
| eventStatus (1.3.6.1.2.1.16.9.1.1.7) | read-create | Current | As per MIB |

logTable

OID of this table is: 1.3.6.1.2.1.16.9.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| logEventIndex (1.3.6.1.2.1.16.9.2.1.1) | read-only | No | As per MIB |
| logIndex (1.3.6.1.2.1.16.9.2.1.2) | read-only | No | As per MIB |
| logTime (1.3.6.1.2.1.16.9.2.1.3) | read-only | No | As per MIB |
| logDescription (1.3.6.1.2.1.16.9.2.1.4) | read-only | No | As per MIB |