# Contents

RMON2-MIB	2

usrHistoryControlTable	2

usrHistoryObjectTable	2

usrHistoryTable	3

probeConfigTable	3

RMON2-MIB

The RMON2-MIB is used with network management protocols in TCP/IP-based internets. In particular, it defines objects for managing remote network monitoring devices.

usrHistoryControlTable

Note: When creating a new instance of this table, the following objects SHOULD be set simultaneously: usrHistoryControlStatus.OID of this table is: 1.3.6.1.2.1.16.18.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| usrHistoryControlIndex (1.3.6.1.2.1.16.18.1.1.1) | not-accessible | Current | As per MIB |
| usrHistoryControlObjects (1.3.6.1.2.1.16.18.1.1.2) | read-create | Current | Range from 1 to 30. Default value is 1. |
| usrHistoryControlBucketsRequested (1.3.6.1.2.1.16.18.1.1.3) | read-create | Current | usrHistoryControlBucketsRequested only supports 255 buckets at maximum currently, If a value more than 255 is set, the system will return success, but the historyControlBucketGranted will be assigned to 255 as for implementation. |
| usrHistoryControlBucketsGranted (1.3.6.1.2.1.16.18.1.1.4) | read-only | No | As per MIB |
| usrHistoryControlInterval (1.3.6.1.2.1.16.18.1.1.5) | read-create | Current | Range from 1 to 4294967. |
| usrHistoryControlOwner (1.3.6.1.2.1.16.18.1.1.6) | read-create | Current | The length of value is from 0 to 127. Not support non-printable characters and ‘?’. Default value is zero-length string. |
| usrHistoryControlStatus (1.3.6.1.2.1.16.18.1.1.7) | read-create | Current | When new row created, only createAndWait(5) can be set. |

usrHistoryObjectTable

OID of this table is: 1.3.6.1.2.1.16.18.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| usrHistoryObjectIndex (1.3.6.1.2.1.16.18.2.1.1) | not-accessible | Current | As per MIB |
| usrHistoryObjectVariable (1.3.6.1.2.1.16.18.2.1.2) | Read-create | Current | As per MIB |
| usrHistoryObjectSampleType (1.3.6.1.2.1.16.18.2.1.3) | Read-create | Current | As per MIB |

usrHistoryTable

OID of this table is: 1.3.6.1.2.1.16.18.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| usrHistorySampleIndex (1.3.6.1.2.1.16.18.3.1.1) | not-accessible | No | As per MIB |
| usrHistoryIntervalStart (1.3.6.1.2.1.16.18.3.1.2) | read-only | No | As per MIB |
| usrHistoryIntervalEnd (1.3.6.1.2.1.16.18.3.1.3) | read-only | No | As per MIB |
| usrHistoryAbsValue (1.3.6.1.2.1.16.18.3.1.4) | read-only | No | As per MIB |
| usrHistoryValStatus (1.3.6.1.2.1.16.18.3.1.5) | read-only | No | As per MIB |

probeConfigTable

Only objects listed below are supported, others not mentioned are not supported.

OID of this table is: 1.3.6.1.2.1.16.19

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| probeCapabilities (1.3.6.1.2.1.16.19.1) | read-only | No | As per MIB |
| probeSoftwareRev (1.3.6.1.2.1.16.19.2) | read-only | No | As per MIB |
| probeHardwareRev (1.3.6.1.2.1.16.19.3) | read-only | No | Not supported. The value is always zero-length string. |
| probeDateTime (1.3.6.1.2.1.16.19.4) | read-only | No | As per MIB |
| probeResetControl (1.3.6.1.2.1.16.19.5) | read-write | No | Only support read operation. |