# Contents

SFLOW-MIB	2

Scalar objects	2

sFlowRcvrTable	2

sFlowFsTable	2

sFlowCpTable	3

SFLOW-MIB

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| sFlowVersion (1.3.6.1.4.1.14706.1.1.1) | read-only | No | As per MIB |
| sFlowAgentAddressType (1.3.6.1.4.1.14706.1.1.2) | read-only | No | As per MIB |
| sFlowAgentAddress (1.3.6.1.4.1.14706.1.1.3) | read-only | No | As per MIB |

sFlowRcvrTable

OID of this table is: 1.3.6.1.4.1.14706.1.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| sFlowRcvrIndex (1.3.6.1.4.1.14706.1.1.4.1.1) | not-accessible | Current | Range from 1 to 10. |
| sFlowRcvrOwner (1.3.6.1.4.1.14706.1.1.4.1.2) | read-write | Current | When claim an sFlowRcvrTable entry, the entity taking control of the sampler must set both the owner and a value for sFlowRcvrTimeout in the same SNMP set request. |
| sFlowRcvrTimeout (1.3.6.1.4.1.14706.1.1.4.1.3) | read-write | Current | When reading or writing a permanent sFlowRcvrTable entry, the value will be -1. |
| sFlowRcvrMaximumDatagramSize (1.3.6.1.4.1.14706.1.1.4.1.4) | read-write | Current | Range from 200 to 3000. |
| sFlowRcvrAddressType (1.3.6.1.4.1.14706.1.1.4.1.5) | read-write | Current | As per MIB |
| sFlowRcvrAddress (1.3.6.1.4.1.14706.1.1.4.1.6) | read-write | Current | As per MIB |
| sFlowRcvrPort (1.3.6.1.4.1.14706.1.1.4.1.7) | read-write | Current | As per MIB |
| sFlowRcvrDatagramVersion (1.3.6.1.4.1.14706.1.1.4.1.8) | read-write | Current | Only support V5. |

sFlowFsTable

OID of this table is: 1.3.6.1.4.1.14706.1.1.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| sFlowFsDataSource (1.3.6.1.4.1.14706.1.1.5.1.1) | not-accessible | Current | As per MIB |
| sFlowFsInstance (1.3.6.1.4.1.14706.1.1.5.1.2) | not-accessible | Current | Only four instance is supported for one datasource on device. |
| sFlowFsReceiver (1.3.6.1.4.1.14706.1.1.5.1.3) | read- write | Current | The range depends on sFlowRcvrTable. When the receiver expires, the value is not changed. |
| sFlowFsPacketSamplingRate (1.3.6.1.4.1.14706.1.1.5.1.4) | read- write | Current | The range is product specific. A sampling rate of 0 disables sampling. |
| sFlowFsMaximumHeaderSize (1.3.6.1.4.1.14706.1.1.5.1.5) | read- write | Current | Range from 18 to 512. |

sFlowCpTable

OID of this table is: 1.3.6.1.4.1.14706.1.1.6

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| sFlowCpDataSource (1.3.6.1.4.1.14706.1.1.6.1.1) | not-accessible | Current | As per MIB |
| sFlowCpInstance (1.3.6.1.4.1.14706.1.1.6.1.2) | not-accessible | Current | When setting the sFlowCpTable, this value must be 1. Only one instance is supported for one datasource on device. |
| sFlowCpReceiver (1.3.6.1.4.1.14706.1.1.6.1.3) | read- write | Current | The range depends on sFlowRcvrTable. When the receiver expires, the value is not changed. |
| sFlowCpInterval (1.3.6.1.4.1.14706.1.1.6.1.4) | read- write | Current | Range from 2 to 86400. A sampling interval of 0 disables counter sampling. |