# Contents

TE-MIB	2

scalar objects of teInfo	2

teAdminGroupTable	2

teTunnelTable	2

tePathTable	4

tePathHopTable	5

TE-MIB

scalar objects of teInfo

OID of this table is: 1.3.6.1.2.1.122.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| teDisProtocol (1.3.6.1.2.1.122.1.1.1) | read-only | Current | As per MIB |
| teSignalingProto (1.3.6.1.2.1.122.1.1.2) | read-only | Current | As per MIB |
| teNotificationEnable (1.3.6.1.2.1.122.1.1.3) | read-write | Current | As per MIB |
| teNextTunnelIndex (1.3.6.1.2.1.122.1.1.4) | read-only | Current | As per MIB |
| teNextPathHopIndex (1.3.6.1.2.1.122.1.1.5) | read-only | Current | As per MIB |
| teConfiguredTunnels (1.3.6.1.2.1.122.1.1.6) | read-only | Current | As per MIB |
| teActiveTunnels (1.3.6.1.2.1.122.1.1.7) | read-only | Current | As per MIB |
| tePrimaryTunnels (1.3.6.1.2.1.122.1.1.8) | read-only | Current | As per MIB |

teAdminGroupTable

OID of this table is: 1.3.6.1.2.1.122.1.1.9

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| teAdminGroupNumber (1.3.6.1.2.1.122.1.1.9.1.1) | not-accessible | No | Not supported |
| teAdminGroupName (1.3.6.1.2.1.122.1.1.9.1.2) | read-create | Current | Not supported |
| teAdminGroupRowStatus (1.3.6.1.2.1.122.1.1.9.1.3) | read-create | Current | Not supported |

teTunnelTable

OID of this table is: 1.3.6.1.2.1.122.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| teTunnelIndex (1.3.6.1.2.1.122.1.2.1.1) | not-accessible | No | As per MIB |
| teTunnelName (1.3.6.1.2.1.122.1.2.1.2) | read-create | Current | As per MIB |
| teTunnelNextPathIndex (1.3.6.1.2.1.122.1.2.1.3) | read-only | Current | As per MIB |
| teTunnelRowStatus (1.3.6.1.2.1.122.1.2.1.4) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |
| teTunnelStorageType (1.3.6.1.2.1.122.1.2.1.5) | read-create | Current | Not supported. The value is always nonvolatile(3) |
| teTunnelSourceAddressType (1.3.6.1.2.1.122.1.2.1.6) | read-create | Current | Only support ipv4(1) |
| teTunnelSourceAddress (1.3.6.1.2.1.122.1.2.1.7) | read-create | Current | Only support read operation |
| teTunnelDestinationAddressType (1.3.6.1.2.1.122.1.2.1.8) | read-create | Current | Only support ipv4(1) |
| teTunnelDestinationAddress (1.3.6.1.2.1.122.1.2.1.9) | read-create | Current | As per MIB |
| teTunnelState (1.3.6.1.2.1.122.1.2.1.10) | read-only | Current | Only support up(2), down(3) |
| teTunnelDiscontinuityTimer (1.3.6.1.2.1.122.1.2.1.11) | read-only | No | Not supported. The value is always 0 |
| teTunnelOctets (1.3.6.1.2.1.122.1.2.1.12) | read-only | No | Not supported. The value is always 0 |
| teTunnelPackets (1.3.6.1.2.1.122.1.2.1.13) | read-only | No | Not supported. The value is always 0 |
| teTunnelLPOctets (1.3.6.1.2.1.122.1.2.1.14) | read-only | No | Not supported. The value is always 0 |
| teTunnelLPPackets (1.3.6.1.2.1.122.1.2.1.15) | read-only | No | Not supported. The value is always 0 |
| teTunnelAge (1.3.6.1.2.1.122.1.2.1.16) | read-only | No | As per MIB |
| teTunnelTimeUp (1.3.6.1.2.1.122.1.2.1.17) | read-only | No | As per MIB |
| teTunnelPrimaryTimeUp (1.3.6.1.2.1.122.1.2.1.18) | read-only | No | As per MIB |
| teTunnelTransitions (1.3.6.1.2.1.122.1.2.1.19) | read-only | No | As per MIB |
| teTunnelLastTransition (1.3.6.1.2.1.122.1.2.1.20) | read-only | No | As per MIB |
| teTunnelPathChanges (1.3.6.1.2.1.122.1.2.1.21) | read-only | No | As per MIB |
| teTunnelLastPathChange (1.3.6.1.2.1.122.1.2.1.22) | read-only | No | As per MIB |
| teTunnelConfiguredPaths (1.3.6.1.2.1.122.1.2.1.23) | read-only | Current | As per MIB |
| teTunnelStandbyPaths (1.3.6.1.2.1.122.1.2.1.24) | read-only | Current | As per MIB |
| teTunnelOperationalPaths (1.3.6.1.2.1.122.1.2.1.25) | read-only | Current | As per MIB |

tePathTable

OID of this table is: 1.3.6.1.2.1.122.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| tePathIndex (1.3.6.1.2.1.122.1.3.1.1) | not-accessible | NO | As per MIB |
| tePathName (1.3.6.1.2.1.122.1.3.1.2) | read-create | Current | Only support read operation |
| tePathRowStatus (1.3.6.1.2.1.122.1.3.1.3) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |
| tePathStorageType (1.3.6.1.2.1.122.1.3.1.4) | read-create | Current | Not supported. The value is always nonvolatile(3) |
| tePathType (1.3.6.1.2.1.122.1.3.1.5) | read-create | Current | Only support primary(2), standby(3) |
| tePathConfiguredRoute (1.3.6.1.2.1.122.1.3.1.6) | read-create | Current | As per MIB |
| tePathBandwidth (1.3.6.1.2.1.122.1.3.1.7) | read-create | Current | Only support read operation |
| tePathIncludeAny (1.3.6.1.2.1.122.1.3.1.8) | read-create | Current | Not supported |
| tePathIncludeAll (1.3.6.1.2.1.122.1.3.1.9) | read-create | Current | Not supported |
| tePathExclude (1.3.6.1.2.1.122.1.3.1.10) | read-create | Current | Not supported |
| tePathSetupPriority (1.3.6.1.2.1.122.1.3.1.11) | read-create | Current | Not supported |
| tePathHoldPriority (1.3.6.1.2.1.122.1.3.1.12) | read-create | Current | Not supported |
| tePathProperties (1.3.6.1.2.1.122.1.3.1.13) | read-create | Current | Not supported |
| tePathOperStatus (1.3.6.1.2.1.122.1.3.1.14) | read-only | Current | Only support down(1) , dormant(3), operational(5) |
| tePathAdminStatus (1.3.6.1.2.1.122.1.3.1.15) | read-create | Current | Not supported. The value is always normal(1). |
| tePathComputedRoute (1.3.6.1.2.1.122.1.3.1.16) | read-only | No | As per MIB |
| tePathRecordedRoute (1.3.6.1.2.1.122.1.3.1.17) | read-only | No | As per MIB |

tePathHopTable

OID of this table is: 1.3.6.1.2.1.122.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| teHopListIndex (1.3.6.1.2.1.122.1.4.1.1) | not-accessible | No | As per MIB |
| tePathHopIndex (1.3.6.1.2.1.122.1.4.1.2) | not-accessible | No | As per MIB |
| tePathHopRowStatus (1.3.6.1.2.1.122.1.4.1.3) | read-create | Current | Only support active(1), createAndGo(4) and destroy(6) |
| tePathHopStorageType (1.3.6.1.2.1.122.1.4.1.4) | read-create | Current | Not supported. The value is always nonvolatile(3) |
| tePathHopAddrType (1.3.6.1.2.1.122.1.4.1.5) | read-create | Current | Only support ipv4(1) |
| tePathHopAddress (1.3.6.1.2.1.122.1.4.1.6) | read-create | Current | As per MIB |
| tePathHopType (1.3.6.1.2.1.122.1.4.1.7) | read-only | Current | As per MIB |